Highway Development Control Consultation Response

Planning Application Ref:  PL/2025/05762

Site Address:  Armishaw Removals, Netherhampton Road, Netherhampton, Salisbury, SP2 8RA

Proposal:  Mixed-use commercial development including a convenience/food retail unit, a drive-through coffee unit, two B8 units with ancillary office use, and the retention of an existing B8 warehouse, along with associated parking, access, and servicing arrangements.

Date of Consultation Response: 13/10/2025

Dear Adam,

I refer to the above mentioned planning application to which I make the following initial Highway observations.

Layout

The footway should extend into the site on both sides of the junction radius with internal crossing points added, including dropped kerbs and demarcated with a coloured strip, to guide pedestrians across the internal access road.  Another crossing point with coloured surfacing is required on the access road adjacent to the convenience store, as indicated on the Layout drawing.

The proposed convenience store is shown positioned too close to the road edge on the junction radius, it is essential that it is setback off this kerbline by at least 2 metres.  The addition of a footway will help to achieve this setback and will assist with visibility for vehicles emerging from the Armishaws access.  The design of the convenience store should be such that vehicles are discouraged from parking on the access road adjacent to the store.

The intervisibility between pedestrians/cyclists on the shared use path (SUP) where they cross the site access junction heading east, and vehicles exiting the site, is currently restricted by the fence and overgrown vegetation on the junction.  It is essential that this visibility is significantly improved. The addition of a footway as mentioned above will assist with this requirement.  To further enhance the crossing of the junction, a coloured strip should be provided to indicate to drivers that cyclists may be crossing.  “Beware Cyclists” signage should be located on the exit from the site.

Following the inclusion of the above requirements, the junction will need to be re-tracked for the largest vehicle type travelling through it (possibly artics from In-Excess).

Parking

I agree with the car parking assessment provided in the TA and satisfied it meets the requirements of the Car Parking Strategy (as contained within the LTP3), the parking is shown well distributed across the site.

The amount of cycle parking is acceptable as detailed in the TA, with details of the stands to be approved.

Overnight parking is to be provided for 3 pantechnicons associated with Armishaws although the aerial photography appears to show significantly more than 3 vehicles parked within the site.  Will 3 spaces be sufficient and where will additional vehicles park?

Traffic Assessment

The figures provided in the traffic assessment are considered to be accurate and are agreed.  The increase in vehicle trips on the A3094 Netherhampton Road and through the site access can be accommodated and will not negatively impact on traffic flows or create excessive queuing.  It is determined that no changes are needed to the site junction in terms of capacity improvements.

Contributions

In terms of active travel infrastructure the site can be currently accessed using a shared use path (SUP) adjacent to the Netherhampton Road. However, the SUP is relatively narrow in places, at approx. 2m. Given the amount of new housing being built along the Netherhampton Road it is likely that there will likely be an increase in active travel to this proposed development.  To encourage an increased uptake in active travel, work is required to improve the width of the SUP.  This is identified in the Salisbury LCWIP and the Active Travel Delivery Plan/ map for Salisbury.

A contribution of £35k should be requested towards these active travel improvements on Netherhampton Road.