APPLICATION FOR A CERTIFICATE OF LAWFULNESS FOR PROPOSED USE OR DEVELOPMENT

| APPLICATION REFERENCE: | PL/2025/09334 |
| --- | --- |
| APPLICATION TYPE: | Lawful development: Proposed use |
| SITE ADDRESS: | 72 Saxon Leas, Middle Winterslow, Salisbury, SP5 1RW |
| DEVELOPMENT DESCRIPTION: | Loft conversion and new porch |
| RECOMMENDATION: | Permitted Development |

PROPOSAL

This application seeks a Certificate of Lawfulness (CLP) under Section 192 of the Town and Country Planning Act 1990 (as amended) for a proposed loft conversion and new porch

The planning merits of the proposal are not under consideration. The only issue is whether the proposed works constitute “permitted development” (PD) under the provisions of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended) [hereafter referred to as "the GPDO"].

RELEVANT LEGISLATION AND POLICY

The statutory framework governing CLP’s is set out in Section 192 of the Town and Country Planning Act 1990 (as amended). This section establishes that a proposed development or use is "lawful" if it would not constitute a breach of planning control. A CLP can be granted if the Local Planning Authority is satisfied, on the balance of probabilities, that the proposed development or use would not require planning permission.

- The application has been assessed against the relevant criteria set out in Schedule 2, Part 1, Classes A–H of the GPDO 2015 (as amended).

REPRESENTATIONS

There is no legal obligation to consult third parties, including Town/Parish Councils or neighbouring properties, for this type of application.

RELEVANT SITE HISTORY

The site has the following relevant planning history:

Proposal / Summary

S/1983/0240 - Full planning permission Decision - Approved Date: 14/4/1983 – CONSERVATORY

S/1998/1003 - Full planning permission Decision - Approve with Conditions Date: 4/8/1998 – EXTENSION TO PROVIDE CONSERVATORY AND TOILET

S/77/0787 - Historic apps 1974 to 1983 Decision - Approve Date: 14/9/1977 – Residential development + construction of new vehicular access

ASSESSMENT AND CONCLUSION

The proposal seeks to utilise permitted development rights under Part 1 of Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended).

Having completed the detailed assessment checklist at Appendix 1 and 2 and based on the submitted plans, supporting information, and a review of relevant constraints (including planning history, Article 4 Directions, and any planning conditions), it is concluded that:

Permitted Development (Lawful)

The proposed development does fall within the scope of permitted development, as defined under Schedule 2, Part 1, Classes A & D of the GPDO. It meets all relevant limitations and conditions, and no restrictions (such as Article 4 Directions or planning conditions) are in force that would remove the permitted development rights relied upon.

The proposed development is considered lawful under Section 192 of the Town and Country Planning Act 1990 (as amended).

RECOMMENDATION

It is recommended that a Certificate of Lawfulness for Proposed Development be GRANTED for the following use/development:

Reason for approval: The proposed development is deemed lawful under Schedule 2, Part 1 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended). Therefore, the proposal is in accordance with relevant statutory provisions and does not require planning permission.

APPENDIX 1 –

Checklist for Householder permitted development under Parts 1, 2 or 14 of Schedule 2 of the Town and Country Planning (General Permitted Development) Order 2015 (as amended)

If yes is answered to any of the below questions, then planning permission will be required for the proposal. If not, then proceed to complete the relevant checklist(s) at Appendix 2, 3 and 4.

| Constraint | Yes/No/N/A | Notes |
| --- | --- | --- |
| Is there an Article 4 Direction removing PD rights for a class within which the proposal falls? | NO |  |
| Is there a condition removing PD rights for a class within which the proposal falls? | NO |  |
| For the purposes of Part 1, is the property a flat or maisonette? | NO | Flats do not benefit from Part 1 PD rights |

APPENDIX 2 –

Checklist for Householder permitted development under Part 1 of Schedule 2 of the Town and Country Planning (General Permitted Development) Order 2015 (as amended)

| ### Class B – The enlargement of a dwellinghouse consisting of an addition or alteration to its roof |  | Yes/No |
| --- | --- | --- |
| If yes answer questions below, if no skip to Class C |  |  |
| Criteria (B.1) – Development is not permitted if: |  |  |
| (a) | Was the permission to use the dwellinghouse granted Class G, M, MA, N, P, PA or Q of Part 3 of this Schedule? | NO |
| (b) | Would the roof height be increased beyond the highest part of the existing roof? | NO |
| (c) | Would the enlargement extend beyond the plane of the roof slope forming the principal elevation and facing a highway? | NO |
| (d) | Would the cubic content of the resulting roof space exceed the cubic content of the original roof space by more than 40m³ (terraced) or 50m³ (other)? | NO |
| (e) | Would it include the (i) construction or provision of a veranda, balcony or raised platform, (ii) the installation, alteration or, the installation, alteration or replacement of a chimney, flue or soil and vent pipe? | NO |
| (f) | Is the dwellinghouse is on article 2(3) land? | NO |
| (g) | Was the dwellinghouse built under Part 20 of this Schedule? | NO |
| (h) | Was the dwellinghouse enlarged by virtue of permission granted by Class AA (enlargement of a dwellinghouse by construction of additional storeys0? | NO |

| Criteria (B.2) – Development is permitted by Class B subject to the following conditions: |  | Yes/No |
| --- | --- | --- |
| (a) | Are the materials used in any exterior work of a similar appearance? | YES |
| (b)(i) | Is the proposal a hip-to-gable enlargement or a junction with a rear/side extension? (If yes, B.2(b)(i) does not apply.) | NO |
| (b)(i)(aa) | Where applicable, are the eaves of the original roof maintained or reinstated? | YES |
| (b)(i)(bb) | Where applicable, is the edge of the enlargement at least 0.2m from the eaves, measured along the roof slope from the outside edge of the eaves? | YES |
| (b)(ii) | Where applicable, does no part of the enlargement extend beyond the outside face of any external wall of the original dwellinghouse? | YES |
| (c) | Would any upper-floor windows in side elevation be non-obscure/non-fixed shut below 1.7m? | N/A |

| any part of the structure would be within 2 metres of any boundary of the curtilage of the dwellinghouse with a highway |  |  |
| --- | --- | --- |
| ### Class D – The erection or construction of a porch outside any external door of a dwellinghouse |  | Yes/No |
| If yes answer questions below, if no skip to Class E |  |  |
| Criteria (D.1) – Development is not permitted if: |  |  |
| (a) | Was the permission to use the dwellinghouse granted Class G, M, MA, N, P, PA or Q of Part 3 of this Schedule? | NO |
| (b) | Would the ground area (measured externally) of the structure exceed 3 square metres? | NO |
| (c) | Would any part of the structure be more than 3 metres above ground level? | NO |
| (d) | Would any part of the structure be within 2 metres of any boundary of the curtilage of the dwellinghouse with a highway? | NO |
| (e) | Was the dwellinghouse built under Part 20 of this Schedule? | NO |