DOC 2025/08854 & 2025/09025

Site: Land at Pry Farm, B4553, Swindon, SN5 4JP

Proposal: Discharge of condition 3 (Joinery) Conversion of a former agricultural barn to provide one residential dwelling (Use Class C3) with associated parking and garden.

Conservation Officer: Alex Marsh

________________________________________________________________________

Scope of comments: the following comments relate to the built historic environment.

Site Description:

Pry Farm, Purton is situated to the west side of the B4553 at the north west edge of Swindon and 2 km east of Purton. Pry Farmhouse is late 17th century 2 storey and attic, built in rubble limestone with a concrete slate roof. The plan main range with symmetrical frontage orientated to the east has a central door and a front wing of storey and a half.

Pry Farmhouse has a courtyard of barns to the north subject of the application including a two storey barn to the north edge with gable onto the road and a linear single storey mono pitch, former piggery running parallel with the road that are part of the earlier farmstead buildings.

The barn is in rubble limestone with some brick work internally and slate roof. The piggery has a rubble limestone back wall about 1.2 m onto the road, which has been raised about 0.5m on blockwork the rest of the building is 20th century constructed in block work with corrugated about cement sheet roof. These barns are considered to be curtilage listed.

Heritage Assets:

Pry Farmhouse, Purton, The Pry, grade II

Relevant Planning History:

2024/10051/ LBC & 2024/09839 Planning  Conversion of a former agricultural barn to provide one residential dwelling with associated parking and garden.

The Proposal

Discharge of condition No. 3 2024/10051/ LBC & 2024/09839

Discharge of condition 3. Prior to installation, details of all new external window and door joinery shall be submitted to and approved in writing by the Local Planning Authority. The submitted details shall include depth of reveal, details of heads, sills and lintels, elevations at a scale of not less than 1:10 and horizontal/vertical frame sections (including sections through glazing bars) at not less than 1:2. The works shall be carried out in accordance with the approved details.

REASON: In the interests of preserving the character and appearance of the listed building and its setting.

Assessment:

The information provided is drawing with Barn elevations @scale 1:20 and window section details horizontal and vertical at scale 1:5

The details provided are acceptable subject to a minor alteration to include a single horizontal glazing bar fitted to the Casement Windows No’s W1, W3, W4 and W6.

Summary: The NPPF confirms that when considering the impact of a proposed development on the significance of a designated heritage asset, great weight should be given to the asset’s conservation (and the more important the asset, the greater the weight should be). It makes clear that any harm to a designated heritage asset requires clear and convincing justification.

The details provided for both DOC applications are acceptable subject to a minor alteration to include a single horizontal glazing bar fitted to the Casement Windows No’s W1, W3, W4 and W6.