APPLICATION FOR LISTED BUILDING CONSENT

OFFICER DELGATED REPORT

| APPLICATION REFERENCE: | PL/2025/08786 |
| --- | --- |
| APPLICATION TYPE: | Listed building consent (Alt/Ext) |
| SITE ADDRESS: | 32 West Street, Malmesbury, SN16 0AR |
| DESCRIPTION OF WORKS: | Internal alterations and alterations to windows and doors on the rear elevation |
| RECOMMENDATION: | Approve |

### SITE DESCRIPTION/CONTEXT

No’s 32 is end of terrace and left hand of a pair of terrace house built in the early mid-19th century.

No. 32 is two storey built in coursed limestone rubble with stone slate roof and brick gable stacks. The windows are late 20th century casements and gabled dormer.

The original house has a double depth plan. The property has been extended in the 1980’s with a two storey side and rear extension with a lean to rear addition.

It has a two-window range with irregular quoins with dressed cambered heads with linked imposts to openings, inner doorways, with No.32 doorway infilled with a fixed light window and boarding below when the 1980’s extension shifted the front door of the property to the side two storey extension under a new lean-to porch on timber post.

The terrace front onto West Street. No. 32 has its south end gable, side and rear additions onto Glovers Court. The terrace is in Malmsbury Conservation Area.

### PROPOSAL

Internal alterations and alterations to windows and doors on the rear elevation including French doors with side lights to rear patio and rooflights to the rear lean to extension.

### RELEVANT PLANNING HISTORY

Proposal / Summary

N/88/01559/LBC - Listed building consent Decision - Approved Date: 8/7/1988 – ALTERATIONS TO LISTED BUILDING(ERECTION OF REAR PORCH)

N/88/01591/SEC - Unknown Decision - Permitted Development Date: 8/7/1988 – SECTION 53 DETERMINATION ERECTION OF REAR PORCH

N/10/01928/TCA - Work to Trees in a Cons Area Decision - Approved Date: 16/7/2010 – Fell Norway Spruce Tree

ENQ/2022/00765 - Pre-Application enquiry Decision - Pre-App Response Date: 18/5/2022 – Grade II listed property. Proposed change bathrooms, kitchen, internal layout by removing and re-instating stud partitions. Proposed replacement all windows which are currently painted soft wood, advice sought about timber or powder coated aluminium. N

ENQ/2024/00584 - Pre-Application enquiry Decision - Pre-App Response Date: 16/5/2024 – Removal and Replacement of All Windows with Wooden Double Glazed Window.

PL/2024/07494 - Listed building consent (Alt/Ext) Decision - Approve with Conditions Date: 2/12/2024 – Inspect and replace stone, slate roof tiles to gable roof and repoint chimney stack where required. Replace all windows with double glazed, timber casements. Replace front elevation, ground and first floor casement windows with double glazed, sash window

N/05/02339/COU - Full planning permission Decision - Approve with Conditions Date: 28/10/2005 – Change of Use of Office to Buggy Store and Creation of Office and Guest Suite

N/81/01105/F - Historic apps 1974 to 1983 Decision - Approve with Conditions Date: 30/7/1981 – Extension to dwelling

N/81/01405/LB - Historic apps 1974 to 1983 Decision - Approve Date: 25/9/1981 – Extension to Grade ll listed building

N/83/01518/LB - Historic apps 1974 to 1983 Decision - Approve with Conditions Date: 6/10/1983 – Alterations to listed building - change of roof materials

PL/2021/04530 - Householder planning permission Decision - Approve with Conditions Date: 16/7/2021 – Proposed wooden garden room with felt shingle roof

PL/2025/08786 - Listed building consent (Alt/Ext) – proposed internal and external alterations

RELEVANT LEGISLATION, POLICIES AND GUIDANCE

Legislation

From the point of view of the historic environment the main statutory tests are set out within the Planning (Listed Building and Conservation Areas) Act 1990.  Section 16 requires that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

Section 72(1) of the Planning (Listed Building and Conservation Areas) Act 1990 also requires the Council to pay special attention to the desirability of preserving or enhancing the character or appearance of designated Conservation Areas.

National Policy and Guidance

The National Planning Policy Framework (NPPF) sets out government policy, including in respect of the historic environment.

Section 16 of the NPPF ('Conserving and enhancing the historic environment') sets out the Government's high-level policies concerning heritage and sustainable development.

Paragraphs 213-215 of the NPPF require a balanced approach with any harm which would be caused being weighed against the potential public benefits which might be achieved. Paragraph 210 requires local planning authorities to take account of the desirability of sustaining and enhancing the significance of heritage assets and of new development making a positive contribution to local character and distinctiveness.

Section 14 of the NPPF addresses the challenge of climate change, flooding and coastal change.

Paragraph 167 advises that “Local planning authorities should also give significant weight to the need to support energy efficiency and low carbon heating improvements to existing buildings, both domestic and non-domestic (including through installation of heat pumps and solar panels where these do not already benefit from permitted development rights).

National Planning Practice Guidance provides guidance on interpreting the NPPF.

Historic England Advice Note 2: Making changes to Heritage Assets, illustrates the application of policies set out in the NPPF in determining applications for LBC (and planning permission).

Historic England’s publication ‘Traditional Windows – Their Care, Repair and Upgrading (2017)’ encourages the retention of windows that contribute to the significance of listed buildings but sets also out their approach to the alteration/replacement of windows where this is acceptable in principle:

- Where historic windows, whether original or later insertions, make a positive contribution to the significance of a listed building they should be retained and repaired where possible. If beyond repair they should be replaced with accurate copies.

- Where historic windows have already been replaced with windows whose design follows historic patterns, these usually make a positive contribution to the significance of listed buildings. When they do, they should therefore be retained and repaired where possible. If beyond repair they should be replaced with accurate copies.

- Where historic windows or replacement windows of historic pattern survive without historic glass it may be possible to introduce slim-profile double-glazing without harming the significance of the listed building. There are compatibility issues to consider as the introduction of double-glazing can require the renewal of the window frame to accommodate thicker glazing, thereby harming significance. These issues are covered in Section 4 of this guidance.

- Where historic windows have been replaced with ones whose design does not follow historic patterns, these are unlikely to contribute to the significance of listed buildings. Replacing such windows with new windows of a sympathetic historic pattern, whether single-glazed or incorporating slim-profile double-glazing, may cause no additional harm. It also provides an opportunity to enhance the significance of the building, which is the desired outcome under national policy.

- Where a new window or re-glazing is agreed, the reflective properties of secondary and double-glazing as compared to modern, polished single-glazing, do not usually harm the significance of the building. But when new multi-paned windows are proposed, the desirability of reproducing broken reflections by individually glazing each pane should be considered. Where the aesthetic value of the building is high, then the impact on the whole of the relevant elevation should be considered, including the desirability of accurately matching other windows.

Historic England Advice Note 18: Adapting Historic Buildings for Energy and Carbon Efficiency, provides advice in support of consistent decision making on approaches to improve the energy efficiency and support carbon reduction of historic buildings, whilst conserving their significance.

Wiltshire Core Strategy:

Whilst not directly relevant to listed building consent applications, the below policies give an indication of the Council’s approach to development affecting heritage assets:

- Core  Policy 57 ‘Ensuring high quality design and place shaping’: A high standard of design is required in all new developments, including extensions, alterations, and changes of use of existing buildings. Development is expected to create a strong sense of place through drawing on the local context and being complementary to the locality.

- Core Policy 58 ‘Ensuring the conservation of the historic environment’ requires that “designated heritage assets and their settings will be conserved, and where appropriate enhanced, in a manner appropriate to their significance.”

ISSUES

Impact upon the preservation of the listed building or its setting or any features of special architectural and/or historic interest.

CONSULTATION RESPONSES

At the Malmesbury Town Council Planning and Environment committee meeting on 25th November 2025 it was resolved to submit No Objection.

REPRESENTATIONS

In accordance with the requirements of Section 5(1) of the Planning (Listed Buildings and Conservation Areas) Regulations 1990 (as amended) the application for listed building consent was publicised by way of:

- A site notice displayed near the site;

- An advertisement placed in the local press.

These methods were used to inform interested parties and to allow a statutory consultation period of 21 days for representations to be submitted.

No representations have been received during the consultation period.

ASSESSMENT

In accordance with Section 16(2) of the Planning (Listed Buildings and Conservation Areas) Act 1990, special regard has been given to the significance of the listed building, including its architectural and historic interest, and the desirability of preserving both the building and its setting. Relevant policies within the National Planning Policy Framework (NPPF), particularly Section 16: Conserving and Enhancing the Historic Environment, have also been taken into account. These policies require that any harm to the significance of a designated heritage asset must be clearly justified and weighed against the public benefits of the proposal.

In order to be acceptable, the application needs to be assessed against the following criteria:

Information considered/submitted

Paragraph 207 requires that applicants describe the significance of any heritage assets affected, including any contribution made by their setting. This should include sufficient information to provide a clear understanding of the significance of the heritage asset and its setting and the potential impact of any proposals on that significance.

The application information includes Design and Access Statement, Window and Door sections, Conservation Rooflight Details, and Existing and Proposed Elevation and Floor Plan.

Assessment of significance:

According to the National Planning Policy Framework (NPPF), significance is defined as:

“The value of a heritage asset to this and future generations because of its heritage interest. The interest may be archaeological, architectural, artistic or historic. Significance derives not only from a heritage asset’s physical presence, but also from its setting.”

In this case significance comes from its contribution to a 17th and early 19th century urban terrace, its form, layout and architectural expression/detailing and use of local design and materials have significant historic and architectural interest. Its location in a terrace of similar period terrace houses add a positive contribution to the character and appearance of the Conservation Area.

Impact of proposals on the significance and setting of the building/structure:

Understanding the impact of proposed works on a listed building requires a careful and proportionate assessment of how those changes affect the asset’s heritage significance and its setting. This includes both the physical alterations to the building itself and any changes to its spatial or visual relationship with surrounding structures or spaces.

The significance of a heritage asset is not static; it is shaped by its architectural, historic, and communal values, and by how these are experienced in context. Therefore, any intervention must be considered in light of the asset’s identified significance, with attention given to both potential harm and benefits. Benefits may include repairs that support the long-term conservation of the building, improved energy performance, or enhanced public access and understanding.

This assessment is grounded in the statutory duty under Section 16 of the Planning (Listed Buildings and Conservation Areas) Act 1990, which requires special regard to be given to the desirability of preserving the building and its features of special interest. It is also informed by national policy, particularly paragraphs 199–208 of the National Planning Policy Framework (NPPF), and relevant local heritage policies and guidance.

The proposal relates to internal alterations and external alterations to windows and doors to the  1980’s rear two storey side and rear extension and lean to that includes the kitchen and a snug living room.

The internal alterations relate to the relocation of the kitchen to the adjoining snug room including slightly widening the existing door opening between the two with a new bi fold door.

A stud partition including door to the existing utility would also be reduced and the rear wall to the lean-to internal opening would be widened, both minor alterations assist in slightly enlarging the room plan to be used as a dining /living room.

This new enlarged room would be further enhanced through the removal of the rear former kitchen door and window with a new French door with side lights with top hung opening lights and two rooflights to the lean to roof to add light to the rear (west) elevation onto the rear garden /patio.

The other external alteration is to attach an EV charge point to the rear wall of the side extension.

The internal and external alterations proposed are all to the modern building fabric of the 1980’s extension and while they will be viewed in the setting of the main 19th century part of the property and its attached terrace neighbours and rear walled gardens they are considered to have a neutral impact on the significance property.

The new external works to the lean-to elevation are discrete and not visible in public views and would be considered to have a neutral impact on the character and appearance of the Conservation Area.

CONCLUSION

The NPPF confirms that when considering the impact of a proposed development on the significance of a designated heritage asset, great weight should be given to the asset’s conservation (and the more important the asset, the greater the weight should be). It makes clear that any harm to a designated heritage asset requires clear and convincing justification.

The overall impact of the proposals on heritage assets will be neutral as both the internal and external alterations proposed are to the modern building fabric of the 1980’s extension and have no direct impact on significant fabric or setting of the listed building or on the character and appearance of the Conservation Area. On this basis no specific justification is required however it is considered the works will assist in securing the future use of a heritage asset in support of its long-term conservation.

The requirements of current conservation legislation, policy or guidance are considered to be met.

RECOMMENDATION:

That Listed Building Consent be GRANTED, subject to the standard conditions for 3 year Listed Building Consent and the Approved Plans List.