PL/2025/08717

Site: 38 North Street, Calne, SN11 0HJ

Proposal: Replace existing wall with cream painted breeze block wall at height of 1.3m. Relocation of existing timber fence to be positioned immediately behind new garden wall with a total height of 3.13m (From street ground level to top of fence)

________________________________________________________________________

Recommendations:

|  | No comment |
| --- | --- |
|  | Support in principle subject to amendments to the scheme (as set out below) |
|  | Support subject to conditions (as set out below) |
| X | Object (for reasons set out below) |
|  | No objections |

Scope of comments: the following comments relate to the built historic environment.

Site Description:

No.38 North Street is a detached Victorian Villa property built in rusticated coursed local stone with ashlar quoins and window surrounds. The orientation is side onto the North Street with the double fronted principal elevation facing the garden to the south.

The central door sits deep in reveal with round arched head. The south elevation window includes ashlar stone bay windows with parapet with circular hole pattern, originally with sliding sash windows with side margins to the ground floor either side.

To the first floor the former timber sliding sash windows had side margins to each side and the central window above the door has a matching round arched head and was a round headed sliding sash. All windows are not replaced with uPVC versions but reflect the side margin window pattern.

The gable elevation onto the road has a single window at attic level and ground floor with ashlar surrounds and to the north is a 20th century pair of gable rear ranges with coursed rubble stone and ashlar quoins to the road and render to the rear slightly set in

The roof has a main roof overhang at the eaves and clay Roman tiles and the rear wings have Roman tile roofs.

The proposal relates to the front garden to the south of the property which has a low rubble stone wall (500 mm) abutting the pavement with an established hedgerow above. There is a single timber boarded round head ed gate entrance centrally to the wall.

The property has architectural interest, and I considered a non-designated heritage asset and makes a positive contribution Calne Conservation Area.

Heritage Assets:

No. 38 North Street is a Non-Designated Heritage Asset

Calne Conservation Area

Relevant Planning History:

N/A

The Proposal: Replace existing wall with cream painted breeze block wall at height of 1.3m. Relocation of existing timber fence to be positioned immediately behind new garden wall with a total height of 3.13m (From street ground level to top of fence)

Policy:

Section 72(1) of the Planning (Listed Building and Conservation Areas) Act 1990 requires the Council to pay special attention to the desirability of preserving or enhancing the character or appearance of designated Conservation Areas.

The NPPF outlines government policy, including its policy in respect of the historic environment. Section 16 of the NPPF 'Conserving and enhancing the historic environment' sets out the Government's high-level policies concerning heritage and sustainable development. Paragraphs 213 - 215 of the NPPF require a balanced approach with any harm which would be caused being weighed against the potential public benefits which might be achieved.

Paragraph 210 requires local planning authorities should take account of the desirability of sustaining and enhancing the significance of heritage assets and of new development making a positive contribution to local character and distinctiveness.

Paragraph 216 The effect of an application on the significance of a non-designated heritage asset should be taken into account in determining the application. In weighing application that directly or indirectly affect non designated heritage assets a balanced judgement will be required having regard to the scale of any harm or loss and the significance of the heritage asset

219. Local planning authorities should look for opportunities for new development within Conservation Areas and World Heritage Sites, and within the setting of heritage assets, to enhance or better reveal their significance. Proposals that preserve those elements of the setting that make a positive contribution to the asset (or which better reveal its significance) should be treated favourably.

National Planning Practice Guidance provides guidance on interpreting the NPPF.

The Council’s Core Strategy Policy CP58 ‘Ensuring the conservation of the historic environment’ requires that “designated heritage assets and their settings will be conserved, and where appropriate enhanced, in a manner appropriate to their significance.”

Wiltshire Council’s Core Strategy Policy CP57 ‘Ensuring high quality design and place shaping’: A high standard of design is required in all new developments, including extensions, alterations, and changes of use of existing buildings. Development is expected to create a strong sense of place through drawing on the local context and being complementary to the locality.

Historic England Advice Note 2 – Making changes to Heritage Assets illustrates the application of policies set out in the NPPF in determining applications for PP and LBC.

Issues: Impact on the Non Designated Heritage Asset, and on Conservation Area character and appearance, design and materials.

Significance of assets and contribution of setting: The NPPF defines significance as the “value of a heritage asset to this and future generations because of its heritage interest.

The interest may be architectural, or historic as set out in the Town and Country Planning (Listed Building and Conservation Areas) Act 1990 S 1.

The NPPF defines significance as the “value of a heritage asset to this and future generations because of its heritage interest. The interest may be, architectural, or historic. Significance derives not only from a heritage asset’s physical presence, but also from its setting.” Historic England assesses significance in a similar manner, referring to evidential, historic, aesthetic, and communal values of a place.

The significance of 38 North Street comes from its architectural interest and it positive contribution to the character and appearance of the Conservation Area

Assessment:

The proposal is to remove the existing stone low wall to the front garden to the south of the entrance gate to the front garden and house and replace this with a painted breeze block wall at height of 1.3m. In addition, the existing timber fence would be located behind the new garden wall with a total height of 3.13m (From street ground level to top of fence).

The impact would be the loss of about half the length of a continues stretch of garden boundary onto the highway of about 50 m. It would create two distinctly different sections of boundary wall about 25m in length in appearance and materials. The resulting appearance of the new section of wall and its use of materials would appear incongruous and discordant in appearance and harmful to the character and appearance of the Conservation Area.

The application has not indicated the justification for the proposal.

I consider the proposal is harmful to the significance of this non designated heritage asset and its contribution to the character and appearance of the Conservation Area.

Summary: The NPPF confirms that when considering the impact of a proposed development on the significance of a non designated heritage asset. The effect of an application on the significance of the asset should be considered in determining the application. In weighing application that directly or indirectly affect non designated heritage assets a balanced judgement will be required having regard to the scale of any harm or loss and the significance of the heritage asset.

In addition, it advises Local planning authorities should look for opportunities for new development within Conservation Areas and World Heritage Sites, and within the setting of heritage assets, to enhance or better reveal their significance.

In this case the loss of half the length of the low stone boundary wall and hedgerow behind is harmful and its replacement with distinctly different material and finishes is harmful to the significance of this non designated heritage asset and its contribution to the character and appearance of the Conservation Area.