Ref. 2026/00193

Site: Praters Farm, Easton Town, Sherston, Malmesbury, SN16 0PS

Proposal: Discharge of Conditions 6 & 8 (Joinery and new window details) of PL/2025/00178

Conservation Officer: Alex Marsh

________________________________________________________________________

Scope of comments: the following comments relate to the built historic environment.

Site Description:

Praters is a former farmhouse now country house dating from the early 17th century with later 17th century west wing addition. It is a two storey property with L shaped plan after later 17th century addition built in local limestone and stone tile roof.

Praters Farmhouse is in Easton Town which is at the eastern edge of Sherston and situated along a rural lane 50m south of the B4040 east-west road that forms the spine of Sherston. The farmhouse is positioned outside the development boundary and Sherston Conservation Area; however, it is located within the Cotswolds AONB.

Heritage Assets:

Praters Farmhouse Easton Town Sherston grade II

Barn to SW of Praters Farmhouse Easton Sherston grade II

Relevant Planning History:

Single-storey extension, internal alterations to ground floor layout and staircase, structural repairs and alterations to west elevation, opening up rear doorway, installation of an ASHP to replace oil tank, installation of underfloor heating to ground floor, and associated alterations

The Proposal

2025/00178 Discharge of conditions No. 6 and 8 on decision Ref.PL/2025/00178

Condition 6. No doors shall be installed into the west elevation until details of the door joinery and surround have been submitted to and approved in writing by the Local Planning Authority. The submitted details shall include depth of reveal, details of heads, jambs, lintels, and dripstones, elevations at a scale of not less than 1:10 and horizontal/vertical frame sections (including sections through glazing bars) at not less than 1:2. The works shall be carried out in accordance with the approved details.

REASON: The application contained insufficient information to enable this matter to be considered prior to granting listed building consent and the matter is required to be agreed with the Local Planning Authority before development commences in order that the development is undertaken in an acceptable manner, in the interests of preserving the character and appearance of the listed building and its setting.

Condition 8 No works for the unblocking and insertion of a window at first floor on the east elevation of the listed building hereby permitted shall commence on site until details of the window have been submitted to and approved in writing by the Local Planning Authority. The submitted details shall include depth of reveal, details of heads, sills and lintels, elevations at a scale of not less than 1:10 and horizontal/vertical frame sections (including sections through glazing bars/mullions) at not less than 1:2. The works shall be carried out in accordance with the approved details.

REASON: The application contained insufficient information to enable this matter to be considered prior to granting listed building consent and the matter is required to be agreed with the Local Planning Authority before development commences in order that the development is undertaken in an acceptable manner, in the interests of preserving the character and appearance of the listed building and its setting.

Assessment:

Condition No. 6 – With respect the French Doors. The proposed doors are not acceptable. The approved drawings under the Listed Building Consent 2025/00178 have an annotation Traditional timber framed doors to garden. The proposed doors are a composite material and not a traditionally constructed timber framed half glazed French style doors.

In addition, the condition requires details shall include depth of reveal, details of heads, jambs, lintels, and dripstones, elevations at a scale of not less than 1:10 and horizontal/vertical frame sections (including sections through glazing bars) at not less than 1:2.

The level of details is important to be able to control the form and construction of the door including details of the proposed new traditional timber door set in its opening, including heads, jambs, reveals, lintels etc and allow the works shall be carried out in accordance with the approved details.

I also consider that the door detail needs to come in with the details for Condition No. 5 related to the stone works and need for investigation of the west elevation wall structure around the ground floor windows /French door opening and buttress to be retained.

Condition 5 Following opening up investigative works of the west elevation wall structural defects around the ground floor window and existing buttress, no works to rebuild the stone walling shall take place until a sample panel not less than 1 metre square to show the proposed mortar composition and colour, and the method of pointing, has been prepared on site, inspected and approved in writing by the Local Planning Authority. The works shall thereafter be carried out to match the approved sample re-using the existing stones.

REASON: In the interests of preserving the character and appearance of the listed building and its setting.

Condition. 8 The proposed Steelyte slim leaded opener details are insufficient to discharge the condition. We need them to be shown in context in more detailed drawings to show details to include depth of reveal, details of heads, sills and lintels, and elevations at a scale of not less than 1:10 and horizontal/vertical frame sections (including sections through glazing bars/mullions) at not less than 1:2.

The proposed stone surrounds should form a part of the elevation and section details in context for the window and the French doors so that a detailed picture of the new openings /doors and windows are provided.

Summary: The NPPF confirms that when considering the impact of a proposed development on the significance of a designated heritage asset, great weight should be given to the asset’s conservation (and the more important the asset, the greater the weight should be). It makes clear that any harm to a designated heritage asset requires clear and convincing justification.

Condition 6 cannot be discharged as the level of details is insufficient. It is important to be able to control the form and construction of the new French doors including appropriate construction and material of the doors – in a traditional timber door but also detail of the new doors in context, set in its opening, including heads, jambs, reveals, lintels stone surrounds etc at an appropriate scale and allow the works shall be carried out in accordance with the approved details.

In addition, the detail for Condition 5 – works to stone work on West Elevation would benefit from being submitted at the same time so that the site visit can be undertaken.

Condition 8 The proposed Steelyte slim leaded opener window details are insufficient to discharge the condition. We need them to be shown in context in more detailed drawings to show details to include depth of reveal, details of heads, sills and lintels, stone surround and elevations at an appropriate scale.