PL/2026/00438

Site: Upper Studley Farm Swindon Road RWB SN4 8ET

Proposal: Regularisation of change of use of area of site from agricultural to class B8 with the replacement of barn and removal of open-air storage

Officer: Alex Marsh

________________________________________________________________________

Recommendations:

| X | Comment |
| --- | --- |
|  | Amendments to the scheme (as set out below) |
|  | Support subject to conditions (as set out below) |
|  | Object (for reasons set out below) |
|  | No objections |

Scope of comments: the following comments relate to the built historic environment.

Policy: From the point of view of the historic environment the main statutory tests are set out within the Planning (Listed Building and Conservation Areas) Act 1990. Sections 16 (LBC) and 66 (PP) require that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

The NPPF outlines government policy, including its policy in respect of the historic environment. Section 16 of the NPPF 'Conserving and enhancing the historic environment' sets out the Government's high-level policies concerning heritage and sustainable development.

Paragraphs 213 – 215 of the NPPF require a balanced approach with any harm which would be caused being weighed against the potential public benefits which might be achieved.

Paragraph 210 requires local planning authorities should take account of the desirability of sustaining and enhancing the significance of heritage assets and of new development making a positive contribution to local character and distinctiveness.

Paragraph 219 Local Planning Authorities should look for opportunities for new development …..within the setting of heritage assets to enhance or better reveal their significance.

The Council’s Core Strategy Policy CP58 ‘Ensuring the conservation of the historic environment’ requires that “designated heritage assets and their settings will be conserved, and where appropriate enhanced, in a manner appropriate to their significance.”

The Council’s Core Strategy CP 57: Ensuring high quality design requires a high quality of design in all new developments. Proposals are required to “demonstrate how the proposal will make a positive contribution to the character of Wiltshire through (amongst other things) “enhancing local distinctiveness by responding to the value of the natural and historic environment, relating positively to its landscape setting and the existing pattern of development” and “responding positively to the existing townscape and landscape features in terms of building layouts, built form, height, mass, scale, building line, plot size, elevational design, materials, streetscape and rooflines to effectively integrate the building into its setting.”

Historic England Advice Note 3 – The Setting of Heritage Assets set out advice for managing change within the setting of heritage assets.

Heritage Assets:

Upper Studley Farmhouse Lydiard Tregoze grade II

Site and assets considered:

Upper Studley Farmhouse dates from 1625 and us built in coursed limestone rubble with ashlar quoins and was a roman tiled roof when listed in 1985. It is 2 storey 3 bay on an L shaped Plan. Originally 2 bays with added third bay with attached agricultural space in 17th century.

The windows to the east principal elevation are 3 light ovolo moulded windows ground floor replaced with early hornless 19th century 9 over 9 pane sashes to left hand side. There is a moulded string course at first floor level. The north gables has a single light ovolo moulded windows.

The property has 20th century rear additions. Internally there are cross beam with barred run out stops and large blocked fireplaces. The staircase is in the re-entrant angle and the roof has a 5 bay double purlin structure.

The 1880 OS mapping has courtyard of farm buildings to the north of the farmhouse now only the Dairy Parlour remains which is in a poor condition and part collapsed.

The application site adjoins the existing commercial (former agricultural use) buildings an attached group of modern 20th century steel framed and Dutch Barn style buildings with recent updating including plinth clad in local stone..

Upper Studley Farmhouse is located 500 m south west of Junction 16 of the M4 and 2km east of Royal Wootton Bassett.

Proposal: Regularisation of change of use of area of site from agricultural to class B8 with the replacement of barn and removal of open-air storage

.

Relevant planning history:

N/12/03540/FUL The curtilage barn was proposed to be converted to holiday cottages which includes a structural report attached with some photos at that time of the farmhouse and barn  - this consent was not implemented

The Modern agricultural steel framed shed /Dutch barn were upgraded to commercial sheds under 19/04283/FUL

2024/09273 Change of Use to Gym

ENQ/2025/01184 Pre App relates to the Listed Buildings on the site and their proposed restoration and repair only (subject of enforcement ENF/2024/00978).

ENF/2024/00978 is extant and the issues related to the works to the farmhouse and old dairy are still being investigated.

2025/09748  Retrospective change of use of barns from agricultural to use class E(d) (indoor sport/recreation), E(g) (Site Office/Welfare) and B8 for both commercial and private purpose

Issues: The impact of alterations on setting of listed buildings.

Significance of assets and contribution of setting: The NPPF defines significance as the “value of a heritage asset to this and future generations because of its heritage interest.

The interest may be architectural, or historic as set out in the Town and Country Planning (Listed Building and Conservation Areas) Act 1990 S 1.

Significance derives not only from a heritage asset’s physical presence, but also from its setting.” Historic England assesses significance in a similar manner, referring to evidential, historic, aesthetic, and communal values of a place.

In this case the significance of Upper Studley Farmhouse is provided in the list description. It is a good example of an early 17th century vernacular farmhouse architectural style using good quality materials and craftmanship expressing local vernacular traditions. Its plan form has remained mostly intact and includes good quality internal and external features that illustrate the historic development of the building.

Assessment:

The proposals is for a retrospective change of use of area of this part of the site from agricultural to class B8 and for a new barn 20 m x 9.5 m and about 3.5 m to eaves and about 5 m ridge height, to be built in timber frame with brick plinth and black feather edge board horizontal cladding with a black corrugated sheet roof.

The site has an existing hardstand and has been used for unauthorised car storage related to a commercial sales business. This application looks to regularise this storage use and to erect a new barn.

The Barn will be located on the hardstanding at the east end adjacent to the existing commercial sheds. The hardstanding and a low plinth wall remnant suggest a former building on this location and ariel views and 1970 OS mapping in the design and access statement with the application confirm a former modern steel frame shed on the site of the hardstanding.

This proposal will introduce a smaller building than the previous structure and retain an area of hardstanding of 10 m wide to the west side. The barn will have 4 x 3m high roller shutter doors to the west elevation onto the hardstanding for access.

The rest of the hardstanding area will be removed and bounded to south and west edges with native hedgerow and an area of approximately 20 m x 10 m of wildflower meadow to soften the impact of the development on the wider setting include the listed farmhouse and dairy barn adjacent to the south of the hardstanding. The new building will be in black cladding and corrugated sheet roof to reduce the visual impact.

The impact on the setting of the listed Farmhouse and curtilage listed Dairy Parlour will be neutral from this new storage building and hardstanding storage as it is viewed in context of the existing modern farm buildings.

A condition for the landscaping works to be implemented prior to erecting the storage shed and thereafter maintained should be included.

There is an existing Enforcement case for works to Upper Studley Farmhouse and Dairy Parlour Barn.

Conservation Balancing

NPPF Paragraph 212 seeks Local Planning Authorities should look for opportunities for new development … within the setting of heritage assets to enhance or better reveal their significance.

The impact on the setting of the listed Farmhouse and curtilage listed Dairy Parlour will be neutral from this new storage building and hardstanding storage as it is viewed in context of the existing modern farm buildings.

A condition for the landscaping works to be implemented prior to erecting the storage shed and thereafter maintained should be included.