Replacement Windows and  Doors Details

at

Bennells, Oxenwood

March 2026

Windows and Doors

The majority of the existing windows and doors at Bennells are beyond repair, many have had repair works carried out previously and we would now wish to replace them with new windows and doors. The existing are single glazed and condensate internally.

Bennells is in a conservation area and we propose to replace the existing with windows and doors to closely match existing.

Planning Approval was granted on application ref PL/2034/05543

The existing Windows and Door frames are in Sapele hardwood and softwood. The proposal is to replace them using French Oak, deliberately cosmetically aged with dye and natural finish. Handpicked Grade A Oak from  specific Oak Supplier Hewins .

We propose to use 9 mm single acoustic glass toughened to Building Control requirements where necessary, 21 mm structural glazing bars cut in fine lambs tongue beaded in oak to glass.

Glass to have only a small furniture stamp of Pilkingtons’.

Doors 58 mm thick Oak with glazing bars same as windows, toughened glass to accord with building regs. All frames thresholds weather boards drip strips  all in cosmetically aged Oak .

East Elevation

Kitchen Ground Floor – Window W1 and Door D1

Master Bedroom First Floor – 2 windows - W2 and W3 are habitable rooms

and they must have an unobstructed opening of 0.33m², with a minimum clear opening width of at least 450mm

Kitchen No 2 Ground Floor                          Kitchen No 2 Ground Floor

Window W4                                                  Door D2

Bathroom No 2 Ground Floor     			Landing First Floor -

Window W5						Window W6

North Elevation

Kitchen Ground Floor                      			Hall Ground Floor

Window W7 						Window W8

Hall Ground Floor                                        		Landing First Floor –

Door D3							Window W9

Drawing Room Ground Floor                			Bath & Bed First Floor

Window W10							Windows W11 & W12

W11 and W12 is to the same habitable room and one window must have an unobstructed opening of 0.33m², with a minimum clear opening width of at least 450mm

West Elevation

Drawing Room Ground Floor - 3 no Windows W13, W14 & W15

Lounge 2 Ground Floor – 2no Windows W16 & W17

Kitchen No 2 Ground Floor – Window W18

Guest Bedrooms First Floor – 2no Windows W19 & W20

W19 and W20 are habitable rooms and they must have an unobstructed opening of 0.33m², with a minimum clear opening width of at least 450mm

South Elevation

Kitchen No 2 Ground Floor       			Kitchen Alley Ground Floor

Window W21						Window W22

En Suite Alley – First Floor Window W23

Spare Bedrooms First Floor – 2no Windows W24 & W25

W24 and W25 are habitable rooms and they must have an unobstructed opening of 0.33m², with a minimum clear opening width of at least 450mm