Highway Development Control Consultation Response

Planning Application Ref:  PL/2025/05762

Site Address:  Armishaw Removals, Netherhampton Road, Netherhampton, Salisbury, SP2 8RA

Proposal:  Mixed-use commercial development including a convenience/food retail unit, a drive-through coffee unit, two B8 units with ancillary office use, and the retention of an existing B8 warehouse, along with associated parking, access, and servicing arrangements.

Date of Consultation Response: 16/03/2026

Dear Adam,

I refer to the revised plans associated with the above planning application to which I make the following initial Highway observations.

Layout

The crossing point with coloured surfacing is required on the access road adjacent to the convenience store, as indicated on the Layout drawing; this shall be illustrated in colour for the drawing to be conditioned, if planning permission was to be granted.

The request for tracking has not been acknowledged, therefore I reiterate the requirement that the junction will need to be re-tracked for the largest vehicle type travelling through it (possibly artics from In-Excess).

As previously requested, to further enhance the crossing of the junction, a coloured strip should be provided to indicate to drivers that cyclists may be crossing.  “Beware Cyclists” signage should be located on the exit from the site.  The approved plan can then be conditioned if planning permission was to be granted.

Parking

Please acknowledge the previous question; overnight parking is to be provided for 3 pantechnicons associated with Armishaws although the aerial photography appears to show significantly more than 3 vehicles parked within the site.  Will 3 spaces be sufficient and where will additional vehicles park?

Traffic Assessment

Confirmation that no changes are needed to the site junction in terms of capacity improvements.

Contributions

In terms of active travel infrastructure the site can be currently accessed using a shared use path (SUP) adjacent to the Netherhampton Road. However, the SUP is relatively narrow in places, at approx. 2m. Given the amount of new housing being built along the Netherhampton Road it is likely that there will likely be an increase in active travel to this proposed development.  To encourage an increased uptake in active travel, work is required to improve the width of the SUP.  This is identified in the Salisbury LCWIP and the Active Travel Delivery Plan/ map for Salisbury.

A contribution of £35k should be requested towards these active travel improvements on Netherhampton Road.