APPLICATION FOR HOUSEHOLDER PLANNING PERMISSION

DELEGATED REPORT

| APPLICATION REFERENCE: | PL/2026/00961 |
| --- | --- |
| APPLICATION TYPE: | Householder planning permission |
| SITE ADDRESS: | 4 Saxon Close, Market Lavington, Devizes, SN10 4EF |
| DEVELOPMENT DESCRIPTION: | Removal of existing conservatory and construction of new single storey extension with internal alterations. |
| RECOMMENDATION: | That Planning Permission be GRANTED |

### PROPOSAL

### The proposal seeks planning permission for the removal of the existing rear conservatory and construction of a single-storey rear extension covering the width of the host dwelling. The extension would be flat roofed with two roof lanterns and additional window and door openings at the rear. The proposal also includes a parapet to be added to the garage roof to accommodate for the extension roof level.

### SITE DESCRIPTION/CONTEXT

The application relates to 4 Saxon Close, in Market Lavington. The site comprises a two-storey detached dwelling situated within a residential cul-de-sac in the built-up area of Market Lavington. The property is a corner plot and is accessed from the adjacent estate road, which has a single off-street parking space and an attached single garage. The dwelling has a modest rear garden, where the existing garage and a conservatory project beyond the original rear building line. The surrounding area is characterised by detached dwellings of similar form and appearance. The site lies in close proximity to Grade II, and I listed buildings located within the Market Lavington Conservation Area to the south, including Fives Court (List Entry No. 1364607), the Barn Immediately North of Old House (List Entry No. 1390687), and The Old House (Grade I) (List Entry No. 1035851). A Public Right of Way (MLAV21B) runs immediately to the south of the site boundary.

### The application site is subject to the following planning policy constraints:

- Located in proximity to listed buildings.

- Flood Zone 1.

- Located within the limits of development for Market Lavington.

- Salisbury Plain SPA (6400km buffer zone).

### PLANNING HISTORY

Proposal / Summary

13/01050/FUL - Full planning permission Decision - Approve with Conditions Date: 30/7/2013 – Formation of new entrance and parking bay

K/81/0019 - Full planning permission Decision - Approve with Conditions Date: 12/12/1981 – Residential development of 133 dwellings

K/15261/D - Reserved Matters Decision - Approve with Conditions Date: 8/3/1990 – ERECTION OF 31 DWELLINGS AND ASSOCIATED WORKS

K/80/0726 - Full planning permission – Estate layout for 121 residential dwellings

K/86/0216 - Outline Planning Decision - Approve with Conditions Date: 12/6/1987 – Residential development of 136 dwellings and garages

ENQ/2025/01590 - PD Enquiries Decision - PP Required Date: – Single storey rear extension following the demolition of existing conservatory.

RELEVANT POLICIES AND GUIDANCE

Wiltshire Core Strategy:

- Core Policy 41: Sustainable Construction and Low Carbon Energy

- Core Policy 57 Ensuring High Quality Design and Place Shaping

- Core Policy 51: Landscape

- Core Policy 58: Ensuring the Conservation of the Historic Environment

- Core Policy 61: Transport and New Development

- Core Policy 67: Flood Risk

- Core Policy 64: Demand Management

Market Lavington Made Neighbourhood Plan

Wiltshire Design Guide: Appendix D – Design Guidance for Householder extensions

The National Planning Policy Framework (NPPF)

The Planning Practice Guidance

National Design Guide

CONSULTATION RESPONSES

Market Lavington Parish Council: No comments received at the time of writing.

REPRESENTATIONS

No comments were received at the time of writing.

ASSESSMENT

Section 38(6) of the Planning and Compulsory Purchase Act 2004 states:

"If regard is to be had to the development plan for the purposes of any determination to be made under the planning Acts, the determination must be made in accordance with the plan unless material considerations indicate otherwise."

In accordance with this statutory requirement, this application has been assessed against the relevant policies of the Development Plan, including the adopted Wiltshire Core Strategy (2015) and, where applicable, any made Neighbourhood Plans.

Design and Impact upon the character and context of the area

Core Policy 57 - Ensuring High Quality Design and Place Shaping of the Wiltshire Core Strategy seeks to ensure that all new development is of high design quality, reflecting local context, character, and distinctiveness.

The proposed single-storey rear extension would be constructed in brick to match the host dwelling, with a single-ply membrane roof and concrete copings to the parapet. The windows and openings would use uPVC, timber, and powder-coated aluminium, maintaining a subservient appearance to the dwelling. While the proposal is not ideal, with the flat roof and its junction with the garage being somewhat awkward, it introduces no additional design harm compared with the existing conservatory and is therefore considered acceptable.

The extension would be positioned at the rear of the property, and although it may be partially visible from the public right-of-way to the south (MLAV21B) due to its lower elevation and boundary treatment, the visual impact is not considered materially harmful. It would replace an existing conservatory, maintaining a modest scale that remains subservient to the main dwellinghouse. As such, it would preserve the character and appearance of the area.

Having regard to the above, the proposal complies with the design principles set out in Core Policy 57 of the Wiltshire Core Strategy and Appendix D of the Wiltshire Design Guide.

Amenity

Point vii of Core Policy 57 - Ensuring High Quality Design and Place Shaping of the Wiltshire Core Strategy seeks to ensure that developments do not adversely affect neighbouring properties.

The National Planning Policy Framework (NPPF), particularly Paragraph 135(f), stresses that planning decisions should ensure developments provide a high standard of amenity for existing and future users.

The proposed development has been assessed in terms of its potential impact on the residential amenity of neighbouring properties, with particular regard to loss of light, overshadowing, loss of privacy, and overbearing effects.

The proposed extension would be single-storey and relatively modest in scale. It would be an infill development attached to the existing garage, meaning it would be positioned away from the shared neighbouring boundary to the west. As the property is a corner plot with no other shared boundaries, there are no concerns regarding overshadowing or overbearing impacts.

The only windows proposed above ground floor level are two roof lanterns. These would be high-level windows with no potential for overlooking. The remaining ground-floor openings face the rear of the property, offering no outlook toward neighbouring residential properties. As such, there are no concerns regarding overlooking.

Given the modest size of the garden, the proposal has also been assessed in terms of private amenity space. The Wiltshire Design Guide (Section 9.2) advises that minimum garden areas for houses should be equivalent to the footprint of the house. The proposed development would conflict with this guidance, reducing the amount of private amenity space. However, the increase in scale over the existing conservatory is modest, and the surrounding area is characterised by compact development with smaller gardens. While not ideal, the level of amenity space retained is therefore considered acceptable.

The proposal therefore accords with Core Policy 57 point vii in respect of safeguarding residential amenity and Appendix D of the Wiltshire Design Guide.

Highways Impact

Core Policy 57 (xiv) of the Wiltshire Core Strategy seeks to ensure that development must meet the requirements of Core Policy 61. This policy requires proposals to be capable of being served by safe access to the highway network. Core Policy 64 seeks to ensure compliance with parking standards set out in the Local Transport Plan.

The National Planning Policy Framework (NPPF), particularly Paragraph 115 and 116, requires that development proposals do not have an unacceptable impact on highway safety and that safe and suitable access to the site can be achieved for all users.

No alterations are proposed to the access, parking arrangements, or number of bedrooms on site. There are therefore no concerns regarding highways impacts.

The proposed development would not result in any adverse impact on highway safety. Adequate parking and turning provision is provided in accordance with Wiltshire’s adopted parking standards, and access arrangements are acceptable.

The proposal is therefore considered to comply with Core Policies 57 (xiv), 61 and 64 of the Wiltshire Core Strategy, the NPPF, and the Wiltshire Local Transport Plan.

Heritage Impact

From the point of view of the historic environment the main statutory tests are set out within the Planning (Listed Building and Conservation Areas) Act 1990.

Section 66 requires that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

The Council’s Core Strategy Policy CP58 ‘Ensuring the conservation of the historic environment’ requires that designated and non-designated heritage assets and their settings will be conserved, and where appropriate enhanced, in a manner appropriate to their significance and encourages proposals which improve the condition of heritage assets at risk.

The Council’s Core Strategy CP 57: Ensuring high quality design requires a high quality of design in all new developments. The policy requires applicants to “demonstrate how the proposal will make a positive contribution to the character of Wiltshire through (amongst other things) responding to the value of the natural and historic environment and being sympathetic to and conserving historic buildings and landscapes. Development is required to respond positively to the existing in terms of building layouts, built form, height, mass, scale, building line, plot size, elevational design, materials, streetscape and rooflines to effectively integrate the building into its setting.

The NPPF outlines government policy for planning. Chapter 16 of the NPPF 'Conserving and enhancing the historic environment' sets out the Government's high-level policies concerning heritage and sustainable development.

The site lies in proximity to two Grade II listed buildings: Fives Court and the Barn Immediately North of Old House, and a Grade I listed building, Old House, situated to the south of the site, which are within the Market Lavington Conservation Area. The proposed development is positioned at a lower elevation and surrounded by neighbouring properties south, meaning it is not visible to heritage assets. Together with the use of materials and a design approach that reflects the character of the host dwelling, the proposal is assessed as having a limited effect on the setting of these designated heritage assets. As such, by virtue of its scale, design, and use of materials, the proposals are considered to preserve the setting of listed buildings and would not result in harm to the significance of heritage assets.

The development is therefore considered to comply with Core Policy 58 of the Wiltshire Core Strategy and Sections 66 and 72 of the Planning (Listed Buildings and Conservation Areas) Act 1990, as well as the relevant provisions of the NPPF.

Biodiversity Net Gain

Householder planning applications are exempt from the requirement to deliver Biodiversity Net Gain under Schedule 7A, Section 4(2)(a) of the Town and Country Planning Act 1990, as inserted by the Environment Act 2021.

CONCLUSION

The proposed development is considered to be acceptable in terms of its design, scale, and relationship with the surrounding area. It would not result in any unacceptable harm to the character of the locality, the amenities of neighbouring occupiers, or the interests of highway safety, heritage, or ecology.

The proposal is therefore considered to comply with the relevant policies of the Wiltshire Core Strategy, the Wiltshire Local Plan, and the National Planning Policy Framework. As such, the application is recommended for approval, subject to the conditions set out below.

RECOMMENDATION:  That Planning Permission be GRANTED subject to conditions.