APPLICATION FOR HOUSEHOLDER PLANNING PERMISSION

DELEGATED REPORT

| APPLICATION REFERENCE: | PL/2026/00661 |
| --- | --- |
| APPLICATION TYPE: | Householder planning permission |
| SITE ADDRESS: | 46 The Street, All Cannings, Devizes, SN10 3PA |
| DEVELOPMENT DESCRIPTION: | Proposed Side & Rear Extension with Widened Access |
| RECOMMENDATION: | That Planning Permission be GRANTED |

### PROPOSAL

### The proposal seeks planning permission for the demolition of the existing timber framed, glazed rear extension and the roof canopy over the main entrance and the construction of a two-storey side extension and a single storey, lean-to, rear extension.  The windows and doors were originally proposed to be constructed from upvc, except for the bi-fold doors, which will be constructed from aluminium.  The application further proposes to widen the existing access.

### The application has been the subject of various revisions including; replacing the originally proposed 2no. first floor windows in the front elevation, with a single window with 5 window lights, with matching small window panes; the windows and doors to be framed in either painted timber or aluminium; the barge boards to be timber with a painted finish; and for the hanging clay tiles to be added to the principal elevation.

### SITE DESCRIPTION/CONTEXT

### The application site relates to a two-storey, clay brick built dwelling, under clay tiled, steeply pitched roofs, with a monopitched canopy roof over the main entrance and two, lean-to single storey extensions adjacent to the rear elevation, one with render to the walls, under a clay tiled roof and the other constructed from render, timber framed glazing, under a corrugated roof.

### There are gardens to the front, side and rear and an unmade driveway area to the front/side of the dwelling, with a tarmac, existing access with dropped kerb.  Hedging encloses part of the front garden.

### The application site is subject to the following planning policy constraints:

- It is a significant unlisted building in the Conservation Area; is in the setting of a listed building; is in the North Wessex Downs National Landscape; is within the River Avon Catchment; and is within Flood Zone 1.

### PLANNING HISTORY

Proposal / Summary

E/10/0501/FUL - Full planning permission Decision - Withdrawn Date: 16/6/2010 – Erection of Dwelling

E/10/1271/FUL - Full planning permission Decision - Approve with Conditions Date: 15/12/2010 – Erection of dwelling and formation of vehicular access and parking court

RELEVANT POLICIES AND GUIDANCE

Section 72(1) of the Planning (Listed Building and Conservation Areas) Act 1990

Sections 16 and 66 of the Planning (Listed Building and Conservation Areas) Act 1990

Wiltshire Core Strategy:

- Core Policy 57 Ensuring High Quality Design and Place Shaping

- Core Policy 50: Biodiversity and Geodiversity

- Core Policy 51: Landscape

- Core Policy 58: Ensuring the Conservation of the Historic Environment

- Core Policy 61: Transport and New Development

- Core Policy 64: Demand Management

- Core Policy 67: Flood Risk

Wiltshire Design Guide: Appendix D – Design Guidance for Householder extensions

The National Planning Policy Framework (NPPF)

The Planning Practice Guidance

National Design Guide

CONSULTATION RESPONSES

All Cannings Parish Council: No objection

Highways Officer: No objections, subject to conditions.

REPRESENTATIONS

No comments were received.

ASSESSMENT

Section 38(6) of the Planning and Compulsory Purchase Act 2004 states:

"If regard is to be had to the development plan for the purposes of any determination to be made under the planning Acts, the determination must be made in accordance with the plan unless material considerations indicate otherwise."

In accordance with this statutory requirement, this application has been assessed against the relevant policies of the Development Plan, including the adopted Wiltshire Core Strategy (2015) and, where applicable, any made Neighbourhood Plans.

Design and Impact upon the character and context of the area and heritage impacts:

The Council’s Core Strategy CP 57: Ensuring high quality design requires a high quality of design in all new developments, reflecting local context, character and distinctiveness. The policy requires applicants to “demonstrate how the proposal will make a positive contribution to the character of Wiltshire through (amongst other things) responding to the value of the natural and historic environment and being sympathetic to and conserving historic buildings and landscapes. Development is required to respond positively to the existing in terms of building layouts, built form, height, mass, scale, building line, plot size, elevational design, materials, streetscape and rooflines to effectively integrate the building into its setting.

From the point of view of the historic environment the main statutory tests are set out within the Planning (Listed Building and Conservation Areas) Act 1990.

Section 66 requires that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

Section 72(1) of the Act requires the Council to pay special attention to the desirability of preserving or enhancing the character or appearance of designated Conservation Areas.

The application dwelling inclusive of the row of dwellings which it forms part of are identified in the All Cannings Conservation Area Statement as being Significant Unlisted Buildings.

The Council’s Core Strategy Policy CP58 ‘Ensuring the conservation of the historic environment’ requires that designated and non-designated heritage assets and their settings will be conserved, and where appropriate enhanced, in a manner appropriate to their significance and encourages proposals which improve the condition of heritage assets at risk.

The NPPF outlines government policy for planning. Chapter 16 of the NPPF 'Conserving and enhancing the historic environment' sets out the Government's high-level policies concerning heritage and sustainable development.

The application site lies within the setting of the grade II listed buildings including Rustic Farmhouse on the opposite side of the road, and the greenhouse at The Old Forge on Greenhouse Road.

The revisions to the replace the first floor windows with a single five light window, ensuring its subdivision into small panes; the addition of tile hanging to the front elevation, which need to be clay tiles; timber or aluminium windows and doors; timber painted barge boards were secured to ensure that the proposals did not destroy the architectural unity of the group/row of dwellings with the same design.  These are good quality materials in line with conservation policies and are an integral part of securing high quality design.

The material pallet will match the existing dwelling and those in the row of properties.  Due to the constraints of the site, the external materials will be conditioned.  Therefore, the high quality design and materials of the extensions will not cause any harm to the setting of these listed buildings, particularly as the greenhouse at The Old Forge is separated from the site by the rear garden of No. 50 The Street, and the public right of way, ACAN14 running across the rear boundaries of the other dwellings in the terrace.

The proposal, by virtue of its scale, design and use of appropriate materials, would preserve the character and appearance of the Conservation Area and the setting of nearby listed buildings. It would not result in harm, including less than substantial harm, to the significance of any designated or non-designated heritage assets.

The development is therefore considered to comply with Core Policy 58 of the Wiltshire Core Strategy and Sections 66 and 72 of the Planning (Listed Buildings and Conservation Areas) Act 1990, as well as the relevant provisions of the NPPF.

Having regard to the above, the proposal complies with the design principles set out in Core Policy 57 of the Wiltshire Core Strategy and Appendix D of the Wiltshire Design Guide.

Amenity

Point vii of Core Policy 57 - Ensuring High Quality Design and Place Shaping of the Wiltshire Core Strategy seeks to ensure that developments do not adversely affect neighbouring properties.

The National Planning Policy Framework (NPPF), particularly Paragraph 135(f), stresses that planning decisions should ensure developments provide a high standard of amenity for existing and future users.

The proposed development has been assessed in terms of its potential impact on the residential amenity of neighbouring properties, with particular regard to loss of light, overshadowing, loss of privacy, and overbearing effects.

The proposed single storey rear extension due to its siting, adjoining the existing single storey lean-to extension, already built up to the shared boundary with No. 47, will extend it across the width of the original rear elevation of the dwelling.

The two-storey extension will be positioned adjacent to the existing gable end of the property, which is on the opposite side of the dwelling to No. 47.  Therefore, it will have no adverse impact upon this adjoining neighbour.

The neighbouring pair of semi-detached dwellings to the north-east side (Nos. 48 and 50 The Street) are almost perpendicular to the application site.  Due to the position and orientation of these dwellings, their rear elevations will face the rear garden of the application property, rather than the north-eastern side elevation of the proposed two-storey side extension.  There are also no windows at first floor level in rear elevation of the two-storey side extension at No. 48 next door, which will further minimise any overbearing impact or loss of light from the proposed two-storey side extension.  In addition to the position and orientation of these dwellings in relation to the proposed side extension, circa 10m will separate the north-eastern side elevation of No. 46 with the rear elevation of No. 48 at an angle.

No windows are proposed in the north-eastern side elevation of the proposed two-storey side extension.  It is not necessary to impose a condition removing Permitted Development rights for any upper floor windows in side elevations, as the Class A of Part 1 of the General Permitted Development (GPDO) 2015 As Amended, already restricts these openings to be obscure glazed and non-opening, unless parts of the window which can be opened are more than  1.7m above the floor of the room in which the window is installed.  This will be drawn to the applicant’s attention by an informative.

In view of the above, the proposals, are not considered to cause any loss of light; overshadowing; overlooking; or have an overbearing impact upon these occupiers neighbouring amenity.

The proposal therefore accords with Core Policy 57 point vii in respect of safeguarding residential amenity and Appendix D of the Wiltshire Design Guide.

Highways Impact

Core Policy 57 (xiv) of the Wiltshire Core Strategy seeks to ensure that development must meet the requirements of Core Policy 61. This policy requires proposals to be capable of being served by safe access to the highway network. Core Policy 64 seeks to ensure compliance with parking standards set out in the Local Transport Plan.

The National Planning Policy Framework (NPPF), particularly Paragraph 115 and 116, requires that development proposals do not have an unacceptable impact on highway safety and that safe and suitable access to the site can be achieved for all users.

The Council’s Highways Officer has commented:

“The proposal includes 3 off street parking spaces which is adequate for the extended property.  The vehicle access is proposed to be widened which will require a vehicle access application, I will attach an informative for this. The foliage across the site frontage must be removed or cut down to provide adequate visibility from the vehicle access.   I raise no highway objection providing the following conditions are imposed…”.

These relate to ensuring that visibility is maintained across the site frontage at or above 900mm; the first 5m measured from the edge of the carriageway is has been consolidated and surfaced (this has been amended to 2m, to reflect the proposed plans), (not loose stone or gravel); works have been implemented to avoid surface water entering the highway; that the gradient of the access at any point will not be steeper than 1m in 6m; that the driveway will remain ungated; that the access and parking spaces have been completed in accordance with the submitted details and shall remain free from storage of materials; and an informative relating to the need for a vehicle crossing licence for the extensions to the dropped kerb.

The driveway will be surfaced in gravel with a consolidated, block paved surface at the entrance to the highway.

The proposed development would not result in any adverse impact on highway safety. Adequate parking and turning provision are provided in accordance with Wiltshire’s adopted parking standards, and access arrangements are acceptable.

The proposal is therefore considered to comply with Core Policies 57 (xiv), 61 and 64 of the Wiltshire Core Strategy, the NPPF, and the Wiltshire Local Transport Plan.

National Landscape

Core Policy 51 of the Wiltshire Core Strategy requires all development to protect, conserve, and where possible enhance the character and quality of Wiltshire’s landscapes, giving particular weight to designated areas such as National Landscapes.

The National Planning Policy Framework (NPPF), in Paragraph 189, states that “great weight” should be given to conserving and enhancing landscape and scenic beauty in National Landscapes (formerly Areas of Outstanding Natural Beauty). Development within or affecting the setting of a National Landscape must demonstrate that it will not result in unacceptable harm to the special qualities of that area.

The application site is well contained to the boundaries of the property is seen within the built form of the Small Village of All Cannings.

The proposal would not result in harm to the special qualities or scenic beauty of the North Wessex Downs National Landscape, given its modest scale and containment within the existing built form of the village. The development therefore accords with Core Policy 51 of the Wiltshire Core Strategy and relevant national policy.

Drainage and Flood Risk

Core Policy 67 of the Wiltshire Core Strategy and paragraph 181 of the NPPF requires that all new development avoids increasing flood risk on-site or elsewhere and, where appropriate, incorporates sustainable drainage measures. Development must also comply with national policy on flood risk management and drainage design. Paragraph 176 (via footnote 62) of the NPPF states that householder development should not be subject to the sequential test, nor the exception test.

The application site falls within Flood Zone 1, according to the submitted Flood Map from the Environment Agency, which results in area with a low probability of flooding.  Groundwater levels are identified as within 0.5m of ground level.  The supporting flood map and risk documents show that a mitigation measure includes installing electrical sockets to be raised a minimum of 400mm above floor level within the rooms they will be installed in at ground floor level.

In terms of surface water run-off, the driveway is proposed to be surfaced in gravel, which is permeable, with a consolidated surfacing, comprising block paving at a depth of 2m, measured back from the carriageway edge.

The proposal therefore complies with Core Policy 67 of the Wiltshire Core Strategy and the policy in the NPPF, ensuring that flood risk is not increased and that surface water drainage is appropriately managed.

Biodiversity Net Gain

Householder planning applications are exempt from the requirement to deliver Biodiversity Net Gain under Schedule 7A, Section 4(2)(a) of the Town and Country Planning Act 1990, as inserted by the Environment Act 2021.

CONCLUSION

The proposed development is considered to be acceptable in terms of its design, scale, and relationship with the surrounding area. It would not result in any unacceptable harm to the character of the locality, the amenities of neighbouring occupiers, or the interests of highway safety, heritage, or ecology.

The proposal is therefore considered to comply with the relevant policies of the Wiltshire Core Strategy and the National Planning Policy Framework. As such, the application is recommended for approval, subject to the conditions.

RECOMMENDATION:  That Planning Permission be GRANTED subject to conditions.