APPLICATION FOR HOUSEHOLDER PLANNING PERMISSION

DELEGATED REPORT

| APPLICATION REFERENCE: | PL/2026/01397 |
| --- | --- |
| APPLICATION TYPE: | Householder planning permission |
| SITE ADDRESS: | 8 Barnack Close, Trowbridge, BA14 9DB |
| DEVELOPMENT DESCRIPTION: | Proposed single storey rear extension |
| RECOMMENDATION: | That Planning Permission be GRANTED |

### PROPOSAL

### The proposal seeks planning permission for a single storey extension to be erected on the rear elevation of the host dwelling.

### SITE DESCRIPTION/CONTEXT

This application relates to a semi-detached dwelling two storey dwelling, 8 Barnack Close, located within an established residential area in Trowbridge. The host dwelling benefits from an off-road parking area to the front elevation and an attached garage. The surrounding properties are all similar in style comprising of semi-detached pairs.

The application site sits within the Bath and Bradford on Avon Special Area of Conservation for bats and the Trowbridge and Westbury Bat Mitigation Strategy (TWBMS)

### PLANNING HISTORY

Proposal / Summary

W/03/01760/FUL - Full planning permission Decision - Approve with Conditions Date: 19/11/2003 – Single storey extension

RELEVANT POLICIES AND GUIDANCE

Wiltshire Core Strategy:

Core Policy 2 - Delivery Strategy

Core Policy 29 – Spatial Strategy for the Trowbridge Community Area

Core Policy 50 – Biodiversity and Geodiversity

Core Policy 57 - Ensuring High Quality Design and Place Shaping

Emerging Policy – Wiltshire Local Plan Review

The examination of the emerging Wiltshire Local Plan has been paused and the weight to be afforded to any part of the emerging plan can be no more than very limited. A decision on how to proceed will be made by the Council in May 2026.

### Trowbridge Bat Mitigation Strategy SPD, adopted February 2020

Wiltshire Design Guide: Appendix D – Design Guidance for Householder extensions

The National Planning Policy Framework (NPPF)

The Planning Practice Guidance

National Design Guide

CONSULTATION RESPONSES

Trowbridge Town Council: No objection

REPRESENTATIONS

One letter of support was received.

ASSESSMENT

Section 38(6) of the Planning and Compulsory Purchase Act 2004 states:

"If regard is to be had to the development plan for the purposes of any determination to be made under the planning Acts, the determination must be made in accordance with the plan unless material considerations indicate otherwise."

In accordance with this statutory requirement, this application has been assessed against the relevant policies of the Development Plan, including the adopted Wiltshire Core Strategy (2015) and, where applicable, any made Neighbourhood Plans.

Design and Impact upon the character and context of the area

Core Policy 57 - Ensuring High Quality Design and Place Shaping of the Wiltshire Core Strategy seeks to ensure that all new development is of high design quality, reflecting local context, character, and distinctiveness.

The extension would be positioned on the northwest elevation, extending to the rear of the host dwelling by a distance of 2.5m spanning a width of 6.7 across the rear elevation. The extension would employ a mono-pitch roof that would sit at a maximum height of 3.5m.

To the rear elevation a set of patio doors will be inserted alongside a window with two rooflights inserted within the roofscape.

Existing (left) and proposed (right) rear elevation of the host dwelling

The use of matching materials (brick elevations under Redland roof tiles) with matching white UPVC fenestration would ensure the proposal would integrate into the built form of the host dwelling and surrounding setting.

It is considered that the extension would by virtue of its height, scale and design constitute a complementary addition to the host dwelling that would appear commensurate to the size of the property and plot.

Having regard to the above, the proposal complies with the design principles set out in Core Policy 57 of the Wiltshire Core Strategy and Appendix D of the Wiltshire Design Guide.

Amenity

Point vii of Core Policy 57 - Ensuring High Quality Design and Place Shaping of the Wiltshire Core Strategy seeks to ensure that developments do not adversely affect neighbouring properties.

The National Planning Policy Framework (NPPF), particularly Paragraph 135(f), stresses that planning decisions should ensure developments provide a high standard of amenity for existing and future users.

The proposed development has been assessed in terms of its potential impact on the residential amenity of neighbouring properties, with particular regard to loss of light, overshadowing, loss of privacy, and overbearing effects.

The proposal relates to a single storey extension to the rear of the host dwelling that would extend over the footprint of an existing extension to the rear that would be removed to facilitate the build. All windows would overlook the applicants private residential rear garden therefore no overlooking concerns are raised.

The extension would be of limited height and mass and due to the single storey nature of the proposal no overshadowing, overbearing or loss of light impacts to neighbouring occupiers are raised.

The proposal therefore accords with Core Policy 57 point vii in respect of safeguarding residential amenity and Appendix D of the Wiltshire Design Guide.

Ecology Impact

Core Policy 50 of the Wiltshire Core Strategy seeks to protect and enhance biodiversity and geodiversity, including through ensuring that development avoids harm to protected species and habitats.

The National Planning Policy Framework (NPPF), in Paragraphs 193, requires that development minimises impacts on and provides net gains for biodiversity, including ensuring mitigation is secured where harm cannot be avoided.

The application site is located within one of the ‘consultation zones’ for the Bath and Bradford-on-Avon Bat Special Area of Conservation (SAC), as well as the grey, hatched zone of the Trowbridge and Westbury Bat Mitigation Strategy (TWBMS).

The proposed development comprises an extension to existing residential development consisting of limited built form close to the existing dwelling. Given the established high density urban residential nature of the location and the absence of any works affecting the roof of the existing dwelling or resultant loss of vegetation, it is considered highly unlikely that any bats would be affected by the proposal and significant effects relating to the disturbance to bats can therefore be ruled out.

As a precautionary measure an informative would be added to any decision that in event any evidence of bat activity is detected that the applicant should follow the advice of a professional ecologist or contact Natural England.

As such, the development complies with Core Policy 50 of the Wiltshire Core Strategy and the relevant provisions of the NPPF.

Biodiversity Net Gain

Householder planning applications are exempt from the requirement to deliver Biodiversity Net Gain under Schedule 7A, Section 4(2)(a) of the Town and Country Planning Act 1990, as inserted by the Environment Act 2021.

CONCLUSION

The proposed development is considered to be acceptable in terms of its design, scale, and relationship with the surrounding area. It would not result in any unacceptable harm to the character of the locality, the amenities of neighbouring occupiers, or the interests of ecology.

The proposal is therefore considered to comply with the relevant policies of the Wiltshire Core Strategy, the Wiltshire Local Plan, and the National Planning Policy Framework. As such, the application is recommended for approval, subject to the conditions set out below.

RECOMMENDATION:  That Planning Permission be GRANTED subject to conditions.

Conditions

- The development hereby permitted shall be begun before the expiration of three years from the date of this permission.

REASON: To comply with the provisions of Section 91 of the Town and Country Planning Act 1990 as amended by the Planning and Compulsory Purchase Act 2004.

- The development hereby permitted shall be carried out in accordance with the following approved plans and documents:

Existing & Proposed Ground Floor Plans & Elevations DWG No AH2026/12 1 of 2

Location Plan & Proposed Block Plan DWG No AH2026/12 1 of 2

All Plans Received 05/03/2026

REASON: For the avoidance of doubt and in the interests of proper planning.

Informative

If bats or evidence of bats are found during the works, the applicant is advised to stop work and follow advice from an independent ecologist or to contact Natural England’s Bat Line through the internet.