APPLICATION FOR FULL-PLANNING PERMISSION

CASE OFFICER REPORT

| APPLICATION REFERENCE: | PL/2025/09296 |
| --- | --- |
| APPLICATION TYPE: | Full planning permission |
| SITE ADDRESS: | The Library, Manton House, Marlborough, SN8 1PN |
| DEVELOPMENT DESCRIPTION: | Internal layout modifications alongside external changes, including the installation of conservation rooflights, a flue for a wood burner, and new dormer window to match existing design |
| RECOMMENDATION: | Approve with Conditions |

### SITE DESCRIPTION/CONTEXT

### The application relates to The Library Flat, a first-floor residential unit located within the south‑west range of Manton House Stable Yard, Manton, near Marlborough. The building forms part of a former Victorian stable complex dating from the late nineteenth century, arranged around a quadrangle with Manton House forming the fourth side.

### The stable range is Grade II listed, reflecting its historic and architectural significance as a largely intact courtyard group. The application site lies within the North Wessex Downs National Landscape. The flat is accessed from the outer (south‑west) elevation of the range and benefits from an associated garden area. The inner courtyard elevation, which is of greater heritage sensitivity, remains largely unaltered.

### The flat has been in continuous residential use since the early 1990s following historic permissions for conversion, and is currently configured as a three‑bedroom dwelling.

### The Historic England listing description reads as follows:

### Stable range at Manton House GV II Range of buildings enclosing courtyard on north-west side of Manton House. c1870-80. Brick, with tiled roof. Domestic revival style with central gatehouse with jettied and timber framed first floor with half hipped roof. Either side, ranges of single storey stables with stable doors and windows. Remains mainly as built - a high quality courtyard group.

### PROPOSAL

### Full planning permission and listed building consent are sought for the remodelling of the existing first‑floor residential flat. The proposals comprise:

- Reconfiguration of the internal layout, largely affecting modern partitions and finishes;

- Alterations to the outer elevation including:

- One additional dormer window to match an existing dormer;

- Revised and rationalised rooflights;

- A limited number of new or altered window openings;

- A discreet flue associated with a woodburning stove.

### No alterations are proposed to the inner courtyard elevation. The works would not increase the footprint of the building and would not result in the creation of an additional dwelling.

### PLANNING HISTORY

Proposal / Summary

K/84/1086 - Conversion of existing first floor premises to form off. veterinary lab., library etc – Approved with conditions – 17/01/1985

K/10605 - Conversion of laboratory to one flat and 7-bedroomed multiple accommodation – Approved with conditions – 25/06/1987

RELEVANT LEGISLATION, POLICIES AND GUIDANCE

Statutory Legislation:

Town and Country Planning Act 1990

Planning (Listed Buildings and Conservation Areas) Act 1990

National Planning Policy:

- National Planning Policy Framework (NPPF)

- Planning Practice Guidance (PPG)

- National Design Guide

Development Plan:

The development plan for Wiltshire comprises the Wiltshire Core Strategy (WCS) (adopted January 2015) and any made neighbourhood plans, which remains the principal statutory planning document guiding development across the county. In line with Section 38(6) of the Planning and Compulsory Purchase Act 2004, planning decisions must be made in accordance with the development plan unless material considerations indicate otherwise.

Though the development plan is considered as a whole, those parts deemed to be particularly relevant to this application are listed below:

The adopted Wiltshire Core Strategy (WCS), with particular regard to:

- Core Policy 1 Settlement Strategy;

- Core Policy 2 Delivery Strategy;

- Core Policy 14 Spatial Strategy Marlborough Community Area;

- Core Policy 50 Biodiversity and Geodiversity;

- Core Policy 51 Landscape

- Core Policy 57 Ensuring High Quality Design and Place Shaping;

- Core Policy 58 Ensuring the Conservation of the Historic Environment;

- Core Policy 61 Transport & Development;

- Core Policy 64 Demand Management

Draft Wiltshire Local Plan Policies

The examination of the emerging Wiltshire Local Plan has been paused and the weight to be afforded to any part of the emerging plan can be no more than very limited.  A decision on how to proceed will be made by the Council in May 2026.

Neighbourhood Plan:

Preshute Neighbourhood Plan 2021-2036 Made February 2023

Supplementary Planning Documents and other Guidance:

- Wiltshire Design Guide (adopted April 2024)

ISSUES

The main issues that need to be addressed in this case include:

- Whether the development is acceptable in principle (CP 1 and 2)

- Whether the scheme constitutes high quality design (CP 57)

- Whether the scheme would have an acceptable impact upon the reasonable living conditions of the adjoining occupants (CP 57)

- Whether the scheme would preserve or enhance the historic environment (CP 58)

- Whether the scheme would have an acceptable landscape impact (CP 51)

- Whether the proposal would have a negative effect upon highway safety, including if there is sufficient parking for the proposed development (CP 61 and 64)

- Whether there would be any harmful impacts upon protected species or priority habitats (CP 50)

CONSULTATION RESPONSES

WC Conservation: Initial objection, no further objections raised post revisions.

WC Archaeology: No archaeological issues are raised and no conditions are requested.

WC Public Protection: No objections are raised and no conditions are considered necessary.

Preshute Parish Council: No comments received at the time of writing this report.

REPRESENTATIONS

In accordance with the requirements of the Town and Country Planning (Development Management Procedure) (England) Order 2015 (as amended), the application was publicised by way of:

- Neighbour notification letters sent to adjoining and nearby properties;

- A site notice displayed near the site;

- An advertisement placed in the local press.

These methods were used to inform interested parties and provide a period of 21 days for comments to be submitted.

No letters of representation had been received at the time of writing this report.

ASSESSMENT

Section 38(6) of the Planning and Compulsory Purchase Act 2004 states:

"If regard is to be had to the development plan for the purposes of any determination to be made under the planning Acts, the determination must be made in accordance with the plan unless material considerations indicate otherwise."

In accordance with this statutory requirement, this application has been assessed against the relevant policies of the Development Plan, including the Wiltshire Core Strategy (2015) and, where applicable, any made Neighbourhood Plans. Consideration has also been given to the policies of the National Planning Policy Framework (NPPF) as a material consideration in the assessment.

Principle of Development

Policy Context

Core Policy 1: Settlement Strategy of the WCS establishes a hierarchy of settlements to guide where development should be focused. The policy  prioritises growth in:

- Principal Settlements (e.g., Salisbury, Chippenham, Trowbridge) – main focus for strategic growth.

- Market Towns – support local services and employment; suitable for sustainable development.

- Local Service Centres – smaller towns and villages with a range of services; limited development supported.

- Large and Small Villages – development should be limited and respond to local needs.

- Outside the defined settlements – development is strictly controlled and generally not supported unless exceptional circumstances apply.

.

Core Policy 2: Delivery Strategy of the WCS sets out how development will be distributed and delivered across Wiltshire. The policy emphasises:

- Sustainable patterns of growth, reducing the need to travel and supporting local services.

- Development should be plan-led, aligning with the settlement strategy and supported by infrastructure.

- Windfall development (not allocated in plans) must still comply with the principles of sustainable development.

- Development outside settlement boundaries is generally not permitted, unless it meets specific policy exceptions (as set out in paragraph 4.25 of the WCS).

Together, CP1 and CP2 ensure that new development is strategically located, sustainable, and supports the character and function of Wiltshire’s communities.

Assessment

The application relates to the remodelling of an existing residential flat, the lawful use of which is long established. The proposal would not intensify the use of the site, create additional dwellings or result in development beyond the existing envelope of the building.

Conclusion

It is therefore considered that the principle of the development is acceptable.

Design and Impact on the Character and Appearance of the Area

Policy Context

Core Policy 57 of the WCS requires high-quality design that:

- Enhances local character.

- Responds to the natural, historic, and built environment.

- Retains important features (e.g., trees, hedges).

- Supports sustainable construction and compatibility with surrounding uses.

- Creates safe, legible, and accessible spaces.

Section 12 of the NPPF emphasises design that is:

- Visually attractive.

- Functionally effective.

- Contributes to the quality of the built environment.

The Wiltshire Design Guide encourages sensitivity to local character, materials, and architectural styles and supports modern living standards while maintaining aesthetic and functional quality.

Assessment

The proposal relates to the remodelling of an existing residential flat within a historic building that has been subject to previous alteration and conversion. The scheme would not introduce any new built form beyond the existing envelope of the building, and the external changes proposed are limited in scale and confined to a secondary elevation.

The design approach has evolved through dialogue with the Local Planning Authority and Conservation Officer, resulting in amendments that reduce the number of roof interventions, rationalise openings and introduce a dormer that replicates an existing feature in terms of scale, form and detailing. This has resulted in a more coherent and balanced elevation, reducing visual clutter and ensuring that new elements read as subordinate to the host building.

Materials and detailing would reflect those of the existing building, and the inner courtyard elevation, which is of greater sensitivity, would remain unaltered. Given the contained nature of the site and the absence of clear public views, the proposed alterations would not appear visually intrusive or discordant within their setting.

Conclusion

On balance, it is considered that the proposal represents a proportionate and context‑led design response that respects the character of the building and its surroundings, and would not result in harm to visual amenity. The proposal therefore accords with Core Policy CP57 of the Wiltshire Core Strategy.

Impact on Heritage Assets

Policy Context

From the point of view of the historic environment the main statutory tests are set out within the Planning (Listed Building and Conservation Areas) Act 1990.

Section 66 (Planning Permission) requires that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

Paragraph 208 requires LPAs should “avoid or minimise any conflict” between the heritage asset’s conservation (which includes achieving its optimum viable use) and any aspect of a proposal.

Paragraph 210 requires local planning authorities should take account of the desirability of sustaining and enhancing the significance of heritage assets and putting them to viable new uses and of new development making a positive contribution to local character and distinctiveness.

Paragraphs 212 requires “great weight” to be given to an asset’s conservation “irrespective of whether any potential harm amounts to substantial harm, total loss or less than substantial harm to its significance.”

Paragraph 213 advises that harm to the significance of a designated heritage asset should require “clear and convincing justification”.

Paragraphs 213-215(214-216) require a balanced approach with any harm which would be caused being weighed against the potential public benefits which might be achieved.

The Planning Practice Guidance provides guidance on interpreting the NPPF.

The Council’s Core Strategy Policy CP58 ‘Ensuring the conservation of the historic environment’ requires that designated and non-designated heritage assets and their settings will be conserved, and where appropriate enhanced, in a manner appropriate to their significance and encourages proposals which improve the condition of heritage assets at risk.

The Council’s Core Strategy CP 57: Ensuring high quality design requires a high quality of design in all new developments. The policy requires applicants to “demonstrate how the proposal will make a positive contribution to the character of Wiltshire through (amongst other things) responding to the value of the natural and historic environment and being sympathetic to and conserving historic buildings and landscapes. Development is required to respond positively to the existing in terms of building layouts, built form, height, mass, scale, building line, plot size, elevational design, materials, streetscape and rooflines to effectively integrate the building into its setting.

Assessment

The section of the building subject to the application has undergone significant alteration during twentieth‑century conversion works and contains limited surviving historic fabric internally. The proposed internal remodelling would largely affect modern partitions and finishes and would not result in the loss of features that contribute to the significance of the listed building.

Externally, the proposal would introduce a limited number of alterations to the outer elevation, which is acknowledged to have experienced previous unsympathetic change. Initial concerns were raised regarding the cumulative impact of roof interventions and new openings; however, these concerns have been addressed through amendments that reduce the overall level of intervention and improve the symmetry and coherence of the elevation.

The inner courtyard elevation, which is of higher heritage sensitivity, would remain entirely unaltered. The Conservation Officer has confirmed that, following revisions, the proposal would result in no harm to the overall significance of the listed building, subject to appropriate conditions relating to detailing and implementation.

Having regard to the limited nature of the works, the existing condition of the building, and the amended design, it is considered that the proposal would preserve the significance of the listed building and would comply with Core Policy CP58 and the statutory duty under Section 66 of the Act.

Conclusion

On this basis, it is considered that the proposal would result in no harm to the significance of the heritage asset, subject to appropriate conditions.

Amenity

Policy Context

Core Policy 57 (Point vii) of the Wiltshire Core Strategy requires development to respect the amenity of existing occupants and ensure appropriate amenity levels for future residents.

The National Planning Policy Framework (NPPF), particularly Paragraph 135, reinforces the need for development to provide a high standard of amenity for both existing and future users.

These policies guide the assessment of amenity impacts.

The application relates to the remodelling of an existing single residential flat and would not result in the creation of additional dwellings or an intensification of the established residential use of the site.

The proposed alterations would not introduce new relationships with neighbouring properties beyond those that already exist. Any additional or altered openings would be orientated towards land within the applicant’s control and would not result in unacceptable overlooking, loss of privacy or overbearing impacts to neighbouring occupiers.

Given the enclosed nature of the site, the established residential context, and the limited scale of the external works, it is considered that the proposal would not give rise to adverse impacts in terms of noise, disturbance or general residential amenity.

Furthermore, the internal reconfiguration would improve the quality and functionality of the accommodation for future occupiers without giving rise to planning harm. On this basis, it is considered that the proposal would maintain acceptable living conditions for both existing and future occupants, and would accord with the objectives of Core Policy CP57 insofar as it relates to residential amenity.

Highways Impacts and Parking

Policy Context

Transport implications are a key factor in assessing planning applications, especially for larger developments. Proposals must demonstrate:

- Safe and suitable access for all users, including pedestrians, cyclists, and those with mobility impairments.

- No severe residual impacts on the surrounding highway network.

- Promotion of sustainable travel, reducing reliance on private vehicles.

- Adequate parking and cycle provision, in line with adopted standards.

- Protection and enhancement of public rights of way and active travel routes.

Relevant policies include Core Policies 60–64 of the Wiltshire Core Strategy, supported by the Wiltshire Local Transport Plan, Car Parking Strategy, and Cycling Strategy. The Local Cycling and Walking Infrastructure Plan (LCWIP) and LTN 1/20 (DfT) provide further design guidance for active travel infrastructure.

Proposals must also ensure safe access for refuse collection vehicles and comply with the Wiltshire Council Waste Storage and Collection SPD (2017).

Where necessary, Transport Assessments or Statements should be submitted to evaluate impacts and identify mitigation. Planning conditions or legal agreements may be used to secure off-site improvements, travel plans, or contributions to sustainable transport infrastructure.

Overall, the proposal would maintain the existing parking and access arrangements and would not give rise to any highway safety concerns or parking shortfall.

Ecology and Biodiversity

Policy Context

The ecological impact of development is a key planning consideration, especially for larger schemes. Proposals must assess and address potential effects on:

- Protected species

- Priority habitats

- Ecological networks

- Green and blue infrastructure

Core Policy 50 of the Wiltshire Core Strategy requires development to:

- Avoid harm to ecological assets.

- Deliver measurable Biodiversity Net Gain (BNG), in line with the Environment Act 2021.

- Support habitat restoration and connectivity.

- Integrate biodiversity into site design.

The Green & Blue Infrastructure Strategy (2022–2030) promotes nature recovery, ecosystem services, and habitat connectivity through development.

Unless exempt, National policy requires a minimum of 10% Biodiversity Net Gain (BNG) for major developments, demonstrated via:

- Biodiversity Metric calculations

- Ecological assessments (e.g. PEAs, Phase 1 surveys, protected species surveys)

Assessment

The application relates to the remodelling of an existing residential flat within a historic listed building located in the countryside. The proposal does not involve any ground disturbance, loss of habitat, works beyond the footprint of the existing building, or changes to the surrounding land.

The development is limited to internal alterations and modest external works to the existing building, including interventions to the roof. The building does not contain a traditional loft space or roof void capable of supporting bat roosts. A site visit confirmed that the building is in a sound condition, with no obvious crevices, lifted or loose tiles, gaps, or features that would be likely to offer suitable roosting opportunities for bats or nesting opportunities for other protected species. On this basis, it is considered that the likelihood of protected species being present is low, and the proposal does not trigger the requirement for the submission of ecological surveys.

Notwithstanding this conclusion, it is recognised that bats and other protected species are afforded protection under the Wildlife and Countryside Act 1981 (as amended) and the Conservation of Habitats and Species Regulations 2017. These legislative protections apply irrespective of whether surveys have been submitted or planning permission has been granted. Accordingly, the applicant is advised that should any evidence of bats or other protected species be discovered during the course of works, all works must cease immediately and a suitably qualified ecologist must be engaged to provide advice and, where necessary, to agree appropriate mitigation or licensing with Natural England. Failure to do so could constitute a criminal offence.

In respect of Biodiversity Net Gain, the proposal falls within the scope of the statutory requirement introduced by the Environment Act 2021. However, the development is considered to fall within the de minimis exemption, as it does not affect more than 25 square metres of on‑site habitat and does not involve any works to land beyond the existing built footprint of the building. The application relates solely to the remodelling of an existing dwelling and does not result in the creation of new habitat impacts.

Conclusion

On this basis, it is considered that the proposal would not result in harm to biodiversity or protected species, that the BNG requirement does not apply, and that no ecological conditions are necessary.

Landscape and Visual Impact

Policy Context

Core Policy 51 of the Wiltshire Core Strategy requires development to:

- Protect, conserve, and enhance landscape character and quality.

- Respect local distinctiveness, key views, and visual receptors.

- Give particular weight to impacts on National Landscapes (formerly AONBs) and locally valued landscapes.

The Wiltshire Design Guide SPD (2024) promotes a context-led approach, encouraging integration with natural features and settlement patterns while avoiding visual intrusion.

The Green & Blue Infrastructure Strategy (2022–2030) supports landscape protection through green infrastructure, habitat connectivity, and nature-based solutions.

The NPPF emphasises the need to protect and enhance valued landscapes, particularly those with scenic, historic, or ecological significance. Planning decisions should:

- Recognise the intrinsic character and beauty of the countryside.

- Safeguard key views, local distinctiveness, and landscape settings.

- Ensure development is well-integrated into its surroundings and avoids visual intrusion, especially in sensitive or designated areas.

Landscape impact must be considered alongside biodiversity, heritage, and green infrastructure objectives, with mitigation measures encouraged where harm cannot be avoided.

Paragraph 189 of the NPPF requires that great weight should be given to conserving and enhancing landscape and scenic beauty in National Landscapes which have the highest status of protection in relation to these issues.

Under Section 245 of the Levelling-up and Regeneration Act 2023, LPAs and other relevant authorities must now “seek to further” the statutory purposes of National Landscapes (formerly Areas of Outstanding Natural Beauty), replacing the previous duty to merely “have regard to” them.

This is an active duty, requiring authorities to take reasonable and proportionate steps to support the conservation and enhancement of the natural beauty, special qualities, and key characteristics of these designated areas when exercising any function that affects them.

Assessment

The application site lies within the North Wessex Downs National Landscape, where great weight is afforded to the conservation of landscape character and scenic beauty.

The proposal relates to modest alterations to an existing, established building within a long‑standing built complex. The development would be wholly contained within the footprint and envelope of the existing listed building and would not result in any encroachment into the wider countryside. No changes are proposed to the surrounding land, and the scheme would not introduce any new built form into the landscape.

The site is located within a contained and developed setting, surrounded by other buildings associated with the former Manton House stable complex and adjacent development. As such, the building does not present as an isolated structure within an open landscape context. The proposed works would not alter the established pattern of development or erode the rural character of the surrounding area.

In visual terms, the proposed external alterations are minor in scale and relate to the rationalisation of openings and roof features on an existing elevation. The proposals would not be clearly visible from public vantage points beyond the site, and there are no long‑range public views of the development from within the wider National Landscape. The inner courtyard elevation, which is of greater sensitivity, would remain unaltered.

Given the limited nature of the proposals, the contained site context, and the absence of any landscape encroachment or prominent visual impact, it is considered that the development would preserve the character and scenic qualities of the National Landscape.

Conclusion

On this basis, it is concluded that the proposal accords with Core Policy CP51 of the Wiltshire Core Strategy and would not result in material harm to landscape character or visual amenity.

PLANNING BALANCE AND CONCLUSION

At the heart of the NPPF there is a presumption in favour of sustainable development requiring local planning authorities to approve development proposals that accord with an up-to-date development plan without delay; and where there are no relevant development plan policies, or the policies which are most important for determining the application are out-of-date, permission should be granted unless (taken from paragraph 11d of the NPPF):

- The application of policies in the NPPF that protect areas or assets of particular importance provides a clear reason for refusing development proposed; or

- any adverse impacts of doing so would significantly and demonstrably outweigh the benefits, when assessed against the policies in the NPPF taken as a whole;

The principle of development is considered acceptable when assessed against the settlement strategy of the Wiltshire Core Strategy and the emerging Local Plan.

The proposal is broadly compliant with relevant local and national planning policies in respect of design, scale, residential amenity, heritage, landscape, highway safety, ecology and biodiversity, subject to the recommended conditions.

No material considerations have been identified that would significantly and demonstrably outweigh these benefits or indicate that permission should be refused, having regard to the assessment above and the representations received.

Accordingly, the proposal is considered acceptable in planning terms and in accordance with the Wiltshire Core Strategy, the National Planning Policy Framework, and all other relevant material considerations.

RECOMMENDATION:
That Planning Permission be GRANTED, subject to conditions.