PL/2026/01298 Planning

Site: Manor House, Church Place, Lydiard Millicent, Swindon, SN5 3LS

Proposal: Proposed swimming pool, pool house and external works

Conservation Officer: Alex Marsh

________________________________________________________________________

Recommendations:

| X | Comment |
| --- | --- |
|  | Amendments to the scheme (as set out below) |
|  | Support subject to conditions (as set out below) |
|  | Object (for reasons set out below) |
|  | No objections |

Scope of comments: the following comments relate to the built historic environment.

Policy: From the point of view of the historic environment the main statutory tests are set out within the Planning (Listed Building and Conservation Areas) Act 1990. Sections 16 (LBC) and 66 (PP) require that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

The NPPF outlines government policy, including its policy in respect of the historic environment. Section 16 of the NPPF 'Conserving and enhancing the historic environment' sets out the Government's high-level policies concerning heritage and sustainable development.

Paragraphs 213 - 215 of the NPPF 2024 require a balanced approach with any harm which would be caused being weighed against the potential public benefits which might be achieved.

Paragraph 210 requires local planning authorities should take account of the desirability of sustaining and enhancing the significance of heritage assets and of new development making a positive contribution to local character and distinctiveness.

Paragraph 219 Local Planning Authorities should look for opportunities for new development ,,, within the setting of heritage assets to enhance or better reveal their significance.

The Council’s Core Strategy Policy CP58 ‘Ensuring the conservation of the historic environment’ requires that “designated heritage assets and their settings will be conserved, and where appropriate enhanced, in a manner appropriate to their significance.”

The Council’s Core Strategy CP 57: Ensuring high quality design requires a high quality of design in all new developments. Proposals are required to “demonstrate how the proposal will make a positive contribution to the character of Wiltshire through (amongst other things) “enhancing local distinctiveness by responding to the value of the natural and historic environment, relating positively to its landscape setting and the existing pattern of development” and “responding positively to the existing townscape and landscape features in terms of building layouts, built form, height, mass, scale, building line, plot size, elevational design, materials, streetscape and rooflines to effectively integrate the building into its setting.”

Historic England Advice Note 3 – The Setting of Heritage Assets set out advice for managing change within the setting of heritage assets.

Heritage Assets:

Dovecote at Manor Farm Lydiard Millicent grade II

Lydiard Millicent Conservation Area

Site and assets considered:

The current Manor House is a modern 1965 – 66 replacement dwelling on the site of a former Manor House that had origins on the site since the 11th century. The latest version was 18th and early 19th century two storey, with rendered elevations and slate roof with three parallel ranges that burnt down in 1880.

The gardens were enclosed with the red brick walls in the 18th century and along with the former Threshing Barn, Dovecote and Lodge have survived the fire and later demolition.

The Manor House site boarders the to the north and west of All Saints Church Lydiard Millicent at the centre of the village. The modern house is centrally located, the lodge to the entrance at the right-hand side of the Church, the Dovecote, associated garden walls and Threshing Barn are located to the north of the house with a substantial stone wall in poor condition and partly collapsed forms the north boundary.

The red brick wall attached to the listed Dovecote curves from the Threshing Barn ruin to the north west with the Dovecote to the apex and continues to meet the stone boundary wall where historic mapping suggests further former farm building range was located adjoining the Threshing barn to the north west.

The site retains significant interest from the intact Dovecote and attached curved brick wall but also the remaining standing ruins and associated walling.

Proposal: Proposed swimming pool, pool house and external works.

Relevant planning history:

N/A

Issues: The impact of new development on the setting of the listed buildings.

Significance of assets and contribution of setting: The NPPF defines significance as the “value of a heritage asset to this and future generations because of its heritage interest.

The interest may be architectural, or historic as set out in the Town and Country Planning (Listed Building and Conservation Areas) Act 1990 S 1.

Significance derives not only from a heritage asset’s physical presence, but also from its setting.” Historic England assesses significance in a similar manner, referring to evidential, historic, aesthetic, and communal values of a place.

In this case the significance of the Dovecote and attached walls are remnants from the former Manor House Lydiard Millicent. The Dovecote is a substantial 3 storey building is a good example of an 18th century rural farm building in plan form, layout and architectural expression/detailing and use of local design approaches and extensive use of brick bonding has a significant aesthetic and architectural interest and makes a positive contribution to the character and appearance of the Conservation Area.

Assessment:

The proposal is for a domestic swimming pool building located to the north west corner of the rear walled garden, of the current dwelling house about 3m in from the wall and 7.5 m away from the Dovecote.

The rectangular pool (10m x 5 m) will run parallel to the Pool Building (12.5 m x 4.4m) roughly north /south in orientation, with the Pool Building running parallel to the side garden boundary with the post and rail fence boundary of neighbour Manor Farm.

The Pool Building is proposed to be single storey with eaves at 2.5 m and ridge height at 4.2 m. It is and oak framed structure with 3 bays to the west, and north elevation with timber horizontal cladding inset allowing the frame to be expressed, with the south elevations open with shutters and east elevations with two open bays with roller blind shutters to the open plan seating and bar area and a third infilled with timber horizontal boarding for the service area changing room, toilet shower and plant rooms. The roof is proposed as hipped end and slate clad.

The pool area was to be completed with a slabbed patio area between and around the pool and Pool buildings. The patio area stops short of the curved brick wall – about 1m but at its closest point about 250 mm for a flower bed area abutting the wall.

In general terms the impact on the setting and the change in perception of the setting from the rear the domestic garden with the new pool and pool buildings and hard surface area on the Dovecote and the listed curved wall is a change but this level of impact on significance of these structures is minimal to neutral as the buildings and their appearance is something expected in a domestic garden possibly at the  more developed end of the spectrum.

In addition the listed structures are of a scale 3m wall and three storey Dovecote to have sufficient scale and massing to counter the potential harm from the proposed pool structures which are in somewhat less hard materials palate of timber framing and cladding and slate roof and hardstand.

Therefore, my only comments are in relation to the subtlety of the proposed new development and how it works with the existing listed curved wall and Dovecote in proximity.

I think that it’s important to leave the listed wall and base of the wall free from hard surface and would prefer to see a simpler grassed boundary fringe between the patio and the wall base. I would ask that a fringe of at least 1m is continuous with the curving base of the wall either a grassed fringe or flower bed as proposed, possibly with the curved reflected in the hardstand edge. This may require a very minor moving of the pool structures about 1m further away from the wall to enable this fringe to be achieved.

The application lacks a Heritage Statement and Design and Access Statement to explain and justify the proposal and the design however as the impacts are low harm to neutral it is something we can work around with our assessment under the national and local heritage polices that allow for the Local Planning Authority to make these assessments based on the information provided.

I would also prefer to see a few significant views as existing and proposed pool and buildings using 3D visual or photomontage contextual views developed to include views of the listed Dovecote and curved brick wall to better assist in understanding and explain the impact of such a large garden building in respect to the setting of the listed building. That said there is a contextual section that gives some idea of the context and setting.

Conservation Balancing

The NPPF paragraph 213 advises any harm to or loss of the significance of a designated heritage asset (from its alteration or destruction or from development within its setting) should require clear and convincing justification

And Paragraph 219 advises Local Authorities when considering new development …. within the setting of heritage assets should seek opportunities to enhance or better reveal their significance.

The listed structures are of a scale 3m wall and three storey Dovecote have sufficient scale and massing to counter the potential harm from the proposed pool structures which are in somewhat less hard materials palate.

Therefore, I consider the scale and massing of the proposals will have a very limited impact on the setting of the listed structures and they are domestic in appearance to be expected in a residential garden. My only comments are in relation to the subtlety of the proposed new development and how it works with the existing listed curved wall and Dovecote in its close proximity.

I think that it’s important to leave the listed wall and base of the wall free from hard surface and would prefer to see a simpler grassed boundary fringe between the patio and the wall base. I would ask that a fringe of at least 1m is continuous with the curving base of the wall either a grassed fringe or flower bed as proposed, possibly with the curved reflected in the hardstand edge. This may require a very minor moving of the pool structures about 1m further away from the wall to enable this fringe to be achieved.