APPLICATION FOR A CERTIFICATE OF LAWFULNESS FOR PROPOSED USE OR DEVELOPMENT

| APPLICATION REFERENCE: | PL/2026/01799 |
| --- | --- |
| APPLICATION TYPE: | Lawful development: Proposed use |
| SITE ADDRESS: | 8 Wellington Drive, Devizes, SN10 2JL |
| DEVELOPMENT DESCRIPTION: | Enclosure of an existing gap between two lawful structures within the residential curtilage of the property, using materials that match the existing dwelling and without increasing the height of either building. |
| RECOMMENDATION: | Refuse |

PROPOSAL

This application seeks a Certificate of Lawfulness (CLP) under Section 192 of the Town and Country Planning Act 1990 (as amended) for a proposed enclosure of an existing gap between two lawful structures within the residential curtilage.

The planning merits of the proposal are not under consideration. The only issue is whether the proposed works constitute “permitted development” (PD) under the provisions of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended) [hereafter referred to as "the GPDO"].

RELEVANT LEGISLATION AND POLICY

The statutory framework governing CLP’s is set out in Section 192 of the Town and Country Planning Act 1990 (as amended). This section establishes that a proposed development or use is "lawful" if it would not constitute a breach of planning control. A CLP can be granted if the Local Planning Authority is satisfied, on the balance of probabilities, that the proposed development or use would not require planning permission.

- The application has been assessed against the relevant criteria set out in Schedule 2, Part 1, Classes A–H of the GPDO 2015 (as amended).

REPRESENTATIONS

There is no legal obligation to consult third parties, including Town/Parish Councils or neighbouring properties, for this type of application.

RELEVANT SITE HISTORY

The site has the following relevant planning history:

Proposal / Summary

15/08719/FUL - Full planning permission Decision - Approve with Conditions Date: 17/11/2015 – Erection of rear single storey extension

ASSESSMENT AND CONCLUSION The proposal seeks to utilise permitted development rights under Part 1 of Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended).

Having completed the detailed assessment checklist at Appendix 1 and 2 and based on the submitted plans, supporting information, and a review of relevant constraints (including planning history, Article 4 Directions, and any planning conditions), it is concluded that:

The proposed development does not fall within the scope of permitted development, as it fails to meet one or more of the limitations or conditions set out within the relevant class(es) of the GPDO. Specifically: Class A, (e) to (j) and ((ja)

The proposed development is therefore not considered lawful under Section 192 of the Town and Country Planning Act 1990 (as amended).

RECOMMENDATION

It is recommended that the Certificate of Lawfulness for Proposed Development be REFUSED for the following reasons:

Reason for refusal: The proposed development does not meet the criteria for permitted development under Schedule 2, Part 1, Class A, (e) to (j) and (ja) of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended). Therefore, planning permission is required.

APPENDIX 2 –

Checklist for Householder permitted development under Part 1 of Schedule 2 of the Town and Country Planning (General Permitted Development) Order 2015 (as amended)

|  | Class [F37G,] M, [F38MA,] N, P [F39, PA] or Q of Part 3 of this Schedule |  |
| --- | --- | --- |
| ### Class A – Enlargement, Improvement or alteration of a dwellinghouse |  | Yes/No |
| ### If yes answer questions below, if no skip to Class B |  |  |
| Criteria (A.1) – Development is not permitted if: |  |  |
| (a) | Was the permission to use the dwellinghouse granted Class G, M, MA, N, P, PA or Q of Part 3 of this Schedule? | no |
| (b) | Would the ground covered by extensions to the original dwellinghouse exceed 50% of the total area of the curtilage? | no |
| (c) | Would the height of the proposal exceed the height of the highest part of the roof of the existing dwellinghouse? | no |
| (d) | Would the height of the eaves exceed the height of the existing dwellinghouse eaves? | no |
| (e) | Is the principal elevation of the original dwellinghouse being extended or Is the elevation being extended fronting a highway and forming a side elevation? | no |
| (f) | Would the single storey extension extend beyond the rear wall of the original dwellinghouse house by more than 4m (detached) / 3m (others) or exceed 4m in height? | no |
| (g) | For dwellinghouses not in Article 2(3) land or SSI, would it extend beyond the rear wall of the original dwellinghouse house by more than 8m (detached) / 6m (others) or exceed 4m in height? Please note that the proposed development is subject to a prior approval process. As such, a Certificate of Lawfulness cannot be granted unless prior approval has been applied for and either determined as not required or formally approved. | n/a |
| (h) | Would it have more than a single storey and extent beyond the rear wall of the original dwellinghouse by 3m or be within 7m of any boundary of the curtilage of the dwellinghouse being enlarged which is opposite the rear wall of that dwellinghouse? | n/a |
| (i) | Would the extension be within 2m of a boundary with eaves exceeding 3m in height? | no |
| (j) | Would it extend beyond a side elevation of the original dwellinghouse and exceed 4m in height, be more than single storey or have a width greater than half the original dwellinghouse? | no |
| (ja) | Does the total enlargement (the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceed or would exceed the limits set out in sub-paragraphs (e) to (j)? | yes |

Please see the below section of the General Permitted Development Order and drawing demonstrating how the proposed extension fails to comply with Part 1, Class A.

Rear and side extensions

Where an extension fills the area between a side elevation and a rear wall, then the restrictions on extensions beyond rear walls and side walls will both apply. The extension must:

- extend no more than 3 metres beyond the rear wall, or 4 metres for a detached house.

(ii) be a single storey and must not exceed 4 metres in height;

(iii) have a total width that does not exceed more than half the width of the house (measured at its widest point).

In some situations, it may be that development is undertaken in separate stages. For example, a side extension may be built first, and then a rear extension added at a later date. In this case, under paragraph (ja) the size of the total enlargement (being the proposed extension together with the previous extension to which it will be joined) will be taken into account.

For example: