PL/2026/02018

Site: Rixon Farm, Rixon Gate, Ashton Keynes, Swindon, SN6 6QW

Proposal: Vary Condition 2 (Plans) of 14/03493/FUL - Full Planning Permission for the Demolition of Existing Agricultural Buildings and Erect 2 Dwellings with Parking and associated Landscaping. Conversion of Barn to Form Single Dwelling. Change of Use of Ground Floor Barn and Extension to Form Single Dwelling including Existing First Floor Residential Use (Pursuant to 05/02731/FUL) Erection of Garage for Farmhouse

_______________________________________________________________________

Recommendations:

| X | Comment |
| --- | --- |
|  | Support in principle subject to amendments to the scheme (as set out below) |
|  | Support subject to conditions (as set out below) |
| X | Object (for reasons set out below) |
|  | No objections |

Scope of comments: the following comments relate to the built historic environment.

Site Description:

Rixon Farmhouse and associated farm buildings are located on Rixon Gate a lane that runs east out of the village of Ashton Keynes and about 1km east of the centre of the village. There are lakes 50 to the north and 150 to the south of the farm part of the Cotswold Water Park.

The farmhouse is a two storey with attic built in the late 18th century in limestone rubble with ashlar quoins. The roof is hipped to left with gable stack and stone slates. It has 3 bays with left rear wing of 2 bays. A central door with tripartite sash windows either side and 12 pane sashes to first floor and two hipped dormers to attic.

To the north of the Farmhouse are separately listed linear range of barns are located parallel and abutting Rixon Gate Lane. They are 17th and 18th century and built in limestone rubble.

The main barn is central with ashlar quoins and stone slate roofs. It is 4 bays with midstrey to south and half hipped roof. It has three tiers of ventilation ports on the north side and doors in the second bay.

Attached to the west directly north of the application garage is a two storey 2 bay stables built in limestone rubble with concrete roof tile.

Attached to the stables at the west end is simple open sided structure, a 6 bay stone cowhouse subject of the application with stone tile roof supported on circular rubble columns to the open front to the south on to the farmyard.

This building had at least partly collapsed and has been substantially rebuilt.

To the east of the main barn is a 6-bay structure with limestone lower half raised in in 19th century brick work to provide an attic.

Heritage Assets:

Rixon Farm Barn and Associated Farm Buildings Rixon Gate Ashton Keynes grade II

Rixon Farmhouse Rixon Gate Ashton Keynes grade II

Relevant planning History: 14/03672/LBC

The Proposal: Permission is being sought to amend the Cow shed conversion as approved in the previous permission ref PL/2024/01252 granted on 16th August 2024.

The approved design approach introduced a mix of glazed and timber clad cavity walls behind columns as the original permission, with an end bay kept open to reflect the original from of the building. This was a s73 application to the original planning permission re 14/03493/FUL.

Policy: From the point of view of the historic environment the main statutory tests are set out within the Planning (Listed Building and Conservation Areas) Act 1990. Sections 16 (LBC) and 66 (PP) require that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

The NPPF outlines government policy, including its policy in respect of the historic environment. Section 16 of the NPPF 'Conserving and enhancing the historic environment' sets out the Government's high-level policies concerning heritage and sustainable development.

Paragraphs 213-15 of the NPPF require a balanced approach with any harm which would be caused being weighed against the potential public benefits which might be achieved.

Paragraph 210 requires local planning authorities should take account of the desirability of sustaining and enhancing the significance of heritage assets and of new development making a positive contribution to local character and distinctiveness.

National Planning Practice Guidance provides guidance on interpreting the NPPF.

The Council’s Core Strategy Policy CP58 ‘Ensuring the conservation of the historic environment’ requires that “designated heritage assets and their settings will be conserved, and where appropriate enhanced, in a manner appropriate to their significance.”

Wiltshire Council’s Core Strategy Policy CP57 ‘Ensuring high quality design and place shaping’: A high standard of design is required in all new developments, including extensions, alterations, and changes of use of existing buildings. Development is expected to create a strong sense of place through drawing on the local context and being complementary to the locality.

Historic England Advice Note 2 – Making changes to Heritage Assets illustrates the application of policies set out in the NPPF in determining applications for PP and LBC.

Assessment: The NPPF confirms that when considering the impact of a proposed development on the significance of a designated heritage asset, great weight should be given to the asset’s conservation (and the more important the asset, the greater the weight should be). It makes clear that any harm to a designated heritage asset requires clear and convincing justification.

The approved design approach introduced a mix of glazed and timber clad cavity walls set behind the principal feature to significance of the Cowshed barn, the rounded stone columns which create the six open bays as the original permission, with an end bay kept open to reflect the original form of the building.

The applicant now considers that the ‘openness’ of the original structure is compromised by this solution.

The applicants principal issue with the approved approach is that the new eaves details is compromised as it creates an ‘unbalanced roof structure’ with the proposed eaves details and some minor cleaning and installation of internal blinds issues.

The applicant has therefore proposed to continue the glass along the full length of the south elevation. This the application suggests can be achieved without compromising the buildings original form and appearance and preserves the simplicity of the original building.

The applicant also provides a detail for the new eaves details with external full height glazing and a very specific example with the glazing outside the line of the columns or posts that has been approved elsewhere and this relates to a conversion C16th grade II listed barn conversion where the original timber posts and ceiling beams and braces were integrated (not the roof trusses) .

In this case the majority of the structure, stone columns and timber trusses are all new and therefore the need to preserve the original historic fabric appearance has not the same weight in consideration.

Each case needs to be considered on its own merits.

The starting point is normally the Historic England guidance on Adapting Historic Farm Buildings where it discusses approaches to ‘Openings’ and ‘Glazing’.

Openings

The historic pattern of openings is related to the function of the building over time and often makes a fundamental contribution to its mass and character. …..Where new openings are added or new windows inserted within existing door openings, great care needs to be given to their placing and design….

The large cart doors that lead into the threshing bays of barns pose a particular design challenge in adaptation schemes…..New entrance screens which are slightly recessed with a simple vertical emphasis can be effective.

Full glazing can alter the scale of the building and can give too much prominence to the opening. However, if the cart doors still survive or replacements are designed this can successfully reduce the impact of glazing.

Alternatively, this bay can be left open and unconverted or used for storage or garaging. This allows the large doors and the space behind, which may still retain its original floor finish, to be left unaltered.

Glazing

The glazing of openings is a particularly subtle aspect of design in farm building adaption work. In masonry structures setting glazing deep in the reveal of existing openings (where were really glazed) creates shadow lines and minimises reflections and impact.

The guidance has a number of examples of how to design appropriate glazing responses to barn conversions based on the above principles keeping it simple and setting back the glazing in deep reveals.

The applicants’ modernist clean lines architectural approach is at odds with the organic and rugged circular stone columns which are considered the significant structural element of the Cowshed that form the 6 open bays. It is therefore a strong consideration in any design response to preserve the stone columns as visible and not impinged

To achieve this involves setting the new external elevation as deep as possible, set back behind the columns and that the new elevation treatments should be more solid than glazed as full glazing can alter the scale of the building and can give too much prominence to the large open elevation.

There is a specific need therefore in conversion for open sided farm buildings to equally reflect solid through reinterpretation of cart doors to work together with glazing elements designed to reduce the adverse impacts of full glazing.

The glazing of openings is a particularly subtle aspect of design in farm building adaption works and the benefit from setting glazing deep behind the eaves of existing openings to create deep shadow and minimise reflections and adverse visual impact needs to be carefully considered and understood.

The minor compromise that has resulted in this case is the impact on the new trusses and connection with the stone columns. However, this is considered much less harmful than the now proposed full glazing to the eaves detail.

In addition, a non-heritage issue but one that is of concern and growing issue is the impact on Birds striking glazing a cause of avian mortality. The birds cannot perceive the glass instead seeing reflections of the sky and foliage as clear flyways.

Bird strikes and mortality can be reduced by significantly reducing and breaking up the glazing and reflections to signal a barrier to the birds.

Summary: The glazing of openings is a particularly subtle aspect of design in farm building adaption works and the benefit from setting glazing deep behind the eaves of existing openings to create deep shadow and minimise reflections and adverse visual impact needs to be carefully considered and understood.

The minor compromise that has resulted in this case is the impact on the new trusses and connection with the stone columns. However, this is considered much less harmful than the now proposed full glazing to the eaves detail.

I would add a non-heritage issue there is  a significant potential to increase bird strikes with a full glazed approach as proposed.