APPLICATION FOR HOUSEHOLDER PLANNING PERMISSION

DELEGATED REPORT

| APPLICATION REFERENCE: | PL/2026/01657 |
| --- | --- |
| APPLICATION TYPE: | Householder planning permission |
| SITE ADDRESS: | 4 Jubbs Lane, Ogbourne St George, Marlborough, SN8 1SR |
| DEVELOPMENT DESCRIPTION: | Two storey side/rear extension. |
| RECOMMENDATION: | That Planning Permission be GRANTED |

### PROPOSAL

### The proposal seeks planning permission for the erection of a two-storey side and rear extension. The extension would project approximately 3.3 metres to the side of the property and would span the entire depth of the property before projecting a further 1.5 metres to the rear.

### SITE DESCRIPTION/CONTEXT

### The application relates to 4 Jubbs Lane in Ogbourne St George, Marlborough. It is a two-storey, end-of terrace house located on a residential street. The house comprises brick exterior walling under a hip-to-gable roof with a hip-to-gable style front porch. The architectural style of the property forms a uniform appearance with the other properties on this row of houses. The site is bounded by residential properties to the east, south and west, with open fields lying to the north of the property. The house is set within a relatively large garden and is accessed via a footpath, with shared parking located approximately 45 metres to the east of the site.

### The application site is subject to the following planning policy constraints:

- Washed over by the North Wessex Downs National Landscape.

- Located outside any defined limits of development.

- Flood Zone 1.

### PLANNING HISTORY

No relevant planning history.

RELEVANT POLICIES AND GUIDANCE

Wiltshire Core Strategy:

- Core Policy 50: Biodiversity and Geodiversity

- Core Policy 51: Landscape

- Core Policy 57: Ensuring High Quality Design and Place Shaping

- Core Policy 61: Transport and New Development

- Core Policy 64: Demand Management

Wiltshire Design Guide: Appendix D – Design Guidance for Householder extensions

The National Planning Policy Framework (NPPF)

The Planning Practice Guidance

National Design Guide

Wiltshire Local Transport Plan 2011-2026, adopted 2015 - including the Car Parking Strategy and Cycling Strategy, adopted 2015

CONSULTATION RESPONSES

Ogbourne St George Parish Council: No response received at the time of writing.

Highways: No objection.

REPRESENTATIONS

No comments were received.

ASSESSMENT

Section 38(6) of the Planning and Compulsory Purchase Act 2004 states:

"If regard is to be had to the development plan for the purposes of any determination to be made under the planning Acts, the determination must be made in accordance with the plan unless material considerations indicate otherwise."

In accordance with this statutory requirement, this application has been assessed against the relevant policies of the Development Plan, including the adopted Wiltshire Core Strategy (2015) and, where applicable, any made Neighbourhood Plans.

Design and Impact upon the character and context of the area

Core Policy 57 - Ensuring High Quality Design and Place Shaping of the Wiltshire Core Strategy seeks to ensure that all new development is of high design quality, reflecting local context, character, and distinctiveness.

The proposed extension would be two-storeys high and would be located to the side of the property, also extending to the rear. The front section of the extension would act as a continuation of the existing building, with the same hip-to-gable style roof set at the same pitch and ridge/eaves height as the existing roof, and the front wall sitting flush with the existing front elevation. The rear section of the extension would maintain the same width as the side elevation and would also have a hip-to-gable style roof, but this would be set at a lower ridge height and would run perpendicular to the main roof.

The proposed openings are generally in keeping with the size, style and placement of those found on the existing property; the ground floor door on the front elevation is somewhat awkwardly placed but it is apparent this serves a utility purpose and is not considered materially harmful to the overall appearance of the building given the lack of architectural sensitivity. All other openings are generally placed centrally in their respective elevations and appear proportionate/in keeping with the existing property.

The external walling is proposed as stock facing brick in a brown/orange colour, the windows are proposed as white UPVC, and the roof tile is proposed as concrete plain roof tiles in a brown colour. A condition will be imposed to ensure the materials match the existing property in the interest of protecting visual amenity.

Overall, the proposed extension would retain the same architectural style and scale as the existing property, and while it would see a notable addition of built form to the dwellinghouse, this can be comfortably accommodated within the plot and a subservient appearance to the main dwellinghouse would be retained.

Having regard to the above, the proposal complies with the design principles set out in Core Policy 57 of the Wiltshire Core Strategy and Appendix D of the Wiltshire Design Guide.

Amenity

Point vii of Core Policy 57 - Ensuring High Quality Design and Place Shaping of the Wiltshire Core Strategy seeks to ensure that developments do not adversely affect neighbouring properties.

The National Planning Policy Framework (NPPF), particularly Paragraph 135(f), stresses that planning decisions should ensure developments provide a high standard of amenity for existing and future users.

The proposed development has been assessed in terms of its potential impact on the residential amenity of neighbouring properties, with particular regard to loss of light, overshadowing, loss of privacy, and overbearing effects.

The proposal would involve a relatively large amount of built form being added to the property at the first-floor level. However, the site is relatively generous, and the additional two-storey built form would retain a separation distance of approximately 2.6 metres from the shared boundary with 18 Liddiard’s to the west and a separation distance of approximately 10 metres from the neighbouring house itself. The host dwelling is located to the east of No.18 so any light blocking would be limited to the early hours of the day and would only be slightly increased by the section of the extension which projects to the rear, which is only 1.5 metres with a roof form sloping away from the boundary. Given the limited change from existing levels of light blocking and the separation distance, the proposed additional built form would not give rise to any unacceptable levels of overshadowing or overbearing impact for 18 Liddiard’s.

There would also be a modest impact upon 3 Jubbs Lane which adjoins to the east. The extension only projects by 1.5 metres to the rear and would have a roof form which slopes away from the neighbouring property; therefore, given the separation distance of approximately 8 metres between the extension and the shared boundary with No.3, this relationship is also considered acceptable.

The only newly proposed first floor openings would be sited on the front (south) and rear (north) elevations of the property. There are already first floor openings on the front elevations with a similar outlook to the newly proposed window, and there would be a separation distance of approximately 20 metres between the new opening and the closest neighbouring property, so this is not considered to give rise to any unacceptable levels of overlooking. The proposed rear first floor window would have oblique views towards the garden of no.18 to the west, but the existing first floor windows already have a similar outlook, and this window would not have any direct views to the property or the immediate outdoor amenity space, so this is also considered an acceptable relationship. All other newly proposed openings are situated on the ground floor where they raise no cause for concern regarding overlooking.

The proposal therefore accords with Core Policy 57 point vii in respect of safeguarding residential amenity and Appendix D of the Wiltshire Design Guide.

Highways Impact

Core Policy 57 (xiv) of the Wiltshire Core Strategy seeks to ensure that development must meet the requirements of Core Policy 61. This policy requires proposals to be capable of being served by safe access to the highway network. Core Policy 64 seeks to ensure compliance with parking standards set out in the Local Transport Plan.

The National Planning Policy Framework (NPPF), particularly Paragraph 115 and 116, requires that development proposals do not have an unacceptable impact on highway safety and that safe and suitable access to the site can be achieved for all users.

No alterations are proposed to the access or number of bedrooms on site. The existing car parking arrangement comprises unallocated spaces off Jubbs Lane, no on-site parking is currently provided or proposed.

In consideration of the proposed development, the council’s Highways Officer has raised no objection, commenting “The number of bedrooms in the property is to remain the same and therefore no change to the parking arrangements is necessary.

I raise no highway objection.”

In light of the above and as the Highways Officer has raised no objection, it is concluded that the proposed development would not give rise to any concerns in terms of highway safety and the existing parking arrangements are considered acceptable.

The proposal is therefore considered to comply with Core Policies 57 (xiv), 61 and 64 of the Wiltshire Core Strategy, the NPPF, and the Wiltshire Local Transport Plan.

National Landscape

Core Policy 51 of the Wiltshire Core Strategy requires all development to protect, conserve, and where possible enhance the character and quality of Wiltshire’s landscapes, giving particular weight to designated areas such as National Landscapes.

The National Planning Policy Framework (NPPF), in Paragraph 176, states that “great weight” should be given to conserving and enhancing landscape and scenic beauty in National Landscapes (formerly Areas of Outstanding Natural Beauty). Development within or affecting the setting of a National Landscape must demonstrate that it will not result in unacceptable harm to the special qualities of that area.

Given the proposal’s modest domestic scale and its siting within the established residential curtilage of the dwellinghouse, it would not encroach into the open countryside or erode the wider landscape character. The extension would be read in the context of existing built form and would not harm the special qualities or scenic beauty of the North Wessex Downs National Landscape.

The proposal, by virtue of its scale, siting, and design, would not harm the special qualities or scenic character of the North Wessex Downs National Landscape.

The development therefore complies with Core Policy 51 of the Wiltshire Core Strategy and Paragraph 176 of the NPPF in conserving and enhancing the National Landscape.

Biodiversity Net Gain

Householder planning applications are exempt from the requirement to deliver Biodiversity Net Gain under Schedule 7A, Section 4(2)(a) of the Town and Country Planning Act 1990, as inserted by the Environment Act 2021.

CONCLUSION

The proposed development is considered to be acceptable in terms of its design, scale, and relationship with the surrounding area. It would not result in any unacceptable harm to the character of the locality, the amenities of neighbouring occupiers or the interests of highway safety.

The proposal is therefore considered to comply with the relevant policies of the Wiltshire Core Strategy, the Wiltshire Local Plan, and the National Planning Policy Framework. As such, the application is recommended for approval, subject to the conditions set out below.

RECOMMENDATION:  That Planning Permission be GRANTED subject to conditions.