WALLS & ROOF

Rectangular wall structure – wood, 44mm thickness

Roof – Flat with EPDM rubber covering

WINDOWS (PJO7) x 3

1530x340

Type: Fixed window with an integrated ventilation grate.

Material: Constructed from high-quality laminated solid timber to prevent warping and ensure durability.

Glazing: Fitted with 24mm HR++ double glazing (4 mm glass + 16 mm argon gas filled + 4 mm glass).

DOOR (PE46) x1 Single

1800x770

Laminated Timber: To prevent warping and shrinking, the door frame and posts are constructed from layered (laminated) European spruce.

Double Glazing: The door features 24mm HR++ double glazing (4mm glass + 16mm argon gas + 4mm glass)

Finish – Painted wood – Pebble grey – RAL 7032 – to complement adjacent stone façade of house.

Window and door trims painted white.