APPLICATION FOR HOUSEHOLDER PLANNING PERMISSION

DELEGATED REPORT

| APPLICATION REFERENCE: | PL/2026/01767 |
| --- | --- |
| APPLICATION TYPE: | Householder planning permission |
| SITE ADDRESS: | The Rowans, Orchard Close House, Wye Lane, Marlborough, SN8 1PJ |
| DEVELOPMENT DESCRIPTION: | Erection of garage |
| RECOMMENDATION: | That Planning Permission be GRANTED |

### PROPOSAL

### The proposal seeks planning permission for the erection of a detached single bay garage.

### SITE DESCRIPTION/CONTEXT

### The application relates to the Rowans, Orchard Close House in Wye Lane, Marlborough. It is a two-storey, detached house set within a relatively large and discreet plot at the end of a residential road. The site is situated just outside of the Conservation Area, although it is not visible from any key views, and it is set within proximity to multiple listed buildings, including 10 London Road (List Entry Number: 1258767), Stables and Coach House to Five Alls Public House (List Entry Number: 1034208), and 11 London Road (List Entry Number: 1034206).

### The application site is subject to the following planning policy constraints:

- Located just outside the Marlborough Conservation Area.

- Situated within close proximity to multiple listed buildings.

- Washed over by the North Wessex Downs National Landscape.

- Located within the limits of development for Marlborough.

- Flood Zone 2.

### PLANNING HISTORY

K/54163/F – Two storey extension to provide ancillary residential annex. Approved with Conditions 28th June 2006.

K/55644/VAR – Variation of Planning Permission K/54163/F to include 1 no additional first floor window on the south elevation. Approved with Conditions 5th January 2007.

17/07175/CLE - Certificate of Lawfulness for the occupancy of part of the approved residential annexe as a self contained dwelling. Approved 26th September 2017.

17/12201/FUL - Extensions to existing house (containing a separate 1 bedroom flat) to provide a utility room with bedroom over and a garden room and the division of the house into two, 3 bedroom, dwellings. Associated works. Approved with Conditions 9th February 2018.

RELEVANT POLICIES AND GUIDANCE

Wiltshire Core Strategy:

- Core Policy 50: Biodiversity and Geodiversity

- Core Policy 51: Landscape

- Core Policy 57: Ensuring High Quality Design and Place Shaping

- Core Policy 58: Ensuring the Conservation of the Historic Environment

- Core Policy 61: Transport and New Development

- Core Policy 64: Demand Management

- Core Policy 67: Flood Risk

Marlborough Area Neighbourhood Plan 2021-2036

Wiltshire Design Guide: Appendix D – Design Guidance for Householder extensions

The National Planning Policy Framework (NPPF)

The Planning Practice Guidance

National Design Guide

CONSULTATION RESPONSES

Marlborough Town Council: No objection.

Highways: No objection.

Arboriculture: No objection.

REPRESENTATIONS

No comments were received.

ASSESSMENT

Section 38(6) of the Planning and Compulsory Purchase Act 2004 states:

"If regard is to be had to the development plan for the purposes of any determination to be made under the planning Acts, the determination must be made in accordance with the plan unless material considerations indicate otherwise."

In accordance with this statutory requirement, this application has been assessed against the relevant policies of the Development Plan, including the adopted Wiltshire Core Strategy (2015) and, where applicable, any made Neighbourhood Plans.

Design and Impact upon the character and context of the area

Core Policy 57 - Ensuring High Quality Design and Place Shaping of the Wiltshire Core Strategy seeks to ensure that all new development is of high design quality, reflecting local context, character, and distinctiveness.

The proposed works are relatively minor and would not have a significant visual impact upon the street scene. The proposal comprises the addition of a single storey detached garage to the north of the existing property, infilling a relatively large gap to the side of the house. The front elevation of the garage would broadly align with the front elevation of the property, and it is proposed to have a hip-to-gable style roof which would reflect the style of roofing on the existing property. The scale/height is modest whereby it is considered a subservient addition to the existing dwellinghouse.

The external walling is proposed as render to match existing; the roofing is proposed as plain tile to match existing; the window/pedestrian door are proposed as UPVC; and the garage door is proposed as a roller shutter door. The proposed  materials would generally match with the existing property, with the only new element being the roller garage door which is not considered to give rise to any material visual harm.

Overall, the proposed works comprise a subservient addition, sited sensibly and designed to reflect the architectural style/material palette of the main dwellinghouse. The proposed works are therefore considered acceptable on visual amenity grounds.

Having regard to the above, the proposal complies with the design principles set out in Core Policy 57 of the Wiltshire Core Strategy and Appendix D of the Wiltshire Design Guide.

Heritage Impact

From the point of view of the historic environment the main statutory tests are set out within the Planning (Listed Building and Conservation Areas) Act 1990.

Section 66 requires that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

Section 72(1) of the Act requires the Council to pay special attention to the desirability of preserving or enhancing the character or appearance of designated Conservation Areas.

The Council’s Core Strategy Policy CP58 ‘Ensuring the conservation of the historic environment’ requires that designated and non-designated heritage assets and their settings will be conserved, and where appropriate enhanced, in a manner appropriate to their significance and encourages proposals which improve the condition of heritage assets at risk.

The Council’s Core Strategy CP 57: Ensuring high quality design requires a high quality of design in all new developments. The policy requires applicants to “demonstrate how the proposal will make a positive contribution to the character of Wiltshire through (amongst other things) responding to the value of the natural and historic environment and being sympathetic to and conserving historic buildings and landscapes. Development is required to respond positively to the existing in terms of building layouts, built form, height, mass, scale, building line, plot size, elevational design, materials, streetscape and rooflines to effectively integrate the building into its setting.

The NPPF outlines government policy for planning. Chapter 16 of the NPPF 'Conserving and enhancing the historic environment' sets out the Government's high-level policies concerning heritage and sustainable development.

While the site is situated within proximity to the Marlborough Conservation Area, it is situated at the end of a dead-end road where it is not visible from key views within the conservation area, so it does not have a significant impact upon it’s setting. The same is true for the relationship with the nearby listed buildings.

In any case, as discussed in the previous visual amenity section, the proposed works are relatively minor and would comprise the addition of a subservient outbuilding which would match the material pallet and architectural style of the existing property. There would therefore be no visual harm associated with the proposed works regardless of their visibility within the setting of nearby heritage assets.

The proposal, by virtue of its scale, design, and use of materials, is therefore considered to preserve the setting of the conservation area and setting of the nearby listed buildings.

The development is therefore considered to comply with Core Policy 58 of the Wiltshire Core Strategy and Sections 66 and 72 of the Planning (Listed Buildings and Conservation Areas) Act 1990, as well as the relevant provisions of the NPPF.

Amenity

Point vii of Core Policy 57 - Ensuring High Quality Design and Place Shaping of the Wiltshire Core Strategy seeks to ensure that developments do not adversely affect neighbouring properties.

The National Planning Policy Framework (NPPF), particularly Paragraph 135(f), stresses that planning decisions should ensure developments provide a high standard of amenity for existing and future users.

The proposed development has been assessed in terms of its potential impact on the residential amenity of neighbouring properties, with particular regard to loss of light, overshadowing, loss of privacy, and overbearing effects.

Given the nature of the proposal as a single storey garage sited away from any shared boundaries, there are no concerns raised regarding residential amenity. There is no risk of any adverse overlooking, overshadowing, or overbearing impact arising as a result of the proposed works.

The proposal therefore accords with Core Policy 57 point vii in respect of safeguarding residential amenity and Appendix D of the Wiltshire Design Guide.

Highways Impact

Core Policy 57 (xiv) of the Wiltshire Core Strategy seeks to ensure that development must meet the requirements of Core Policy 61. This policy requires proposals to be capable of being served by safe access to the highway network. Core Policy 64 seeks to ensure compliance with parking standards set out in the Local Transport Plan.

The National Planning Policy Framework (NPPF), particularly Paragraph 115 and 116, requires that development proposals do not have an unacceptable impact on highway safety and that safe and suitable access to the site can be achieved for all users.

The proposed garage would be sited on an existing open-air parking space and may result in a slight reduction in parking capacity at the site due to its central placement within a space which could currently accommodate two cars. However, there remains adequate parking elsewhere to comfortably accommodate the needs of the property so there are no concerns raised regarding highways impacts.

Wiltshire Council’s Highways department has been consulted on the application and have responded with no objections.

The proposed development would not result in any adverse impact on highway safety. Adequate parking and turning provision is provided in accordance with Wiltshire’s adopted parking standards, and access arrangements are acceptable.

The proposal is therefore considered to comply with Core Policies 57 (xiv), 61 and 64 of the Wiltshire Core Strategy, the NPPF, and the Wiltshire Local Transport Plan.

National Landscape

Core Policy 51 of the Wiltshire Core Strategy requires all development to protect, conserve, and where possible enhance the character and quality of Wiltshire’s landscapes, giving particular weight to designated areas such as National Landscapes.

The National Planning Policy Framework (NPPF), in Paragraph 176, states that “great weight” should be given to conserving and enhancing landscape and scenic beauty in National Landscapes (formerly Areas of Outstanding Natural Beauty). Development within or affecting the setting of a National Landscape must demonstrate that it will not result in unacceptable harm to the special qualities of that area.

Due to the proposal’s nature as the erection of a single-storey garage within the established residential curtilage of an existing residential dwellinghouse within an urbanised area, there would be no encroachment into the open countryside associated with the proposed development and there is no risk of harm to the special qualities of the national landscape.

The proposal, by virtue of its scale, siting, and design, would not harm the special qualities or scenic character of the North Wessex Downs National Landscape.

The development therefore complies with Core Policy 51 of the Wiltshire Core Strategy and Paragraph 176 of the NPPF in conserving and enhancing the National Landscape.

Drainage and Flood Risk

Core Policy 67 of the Wiltshire Core Strategy and paragraph 181 of the NPPF requires that all new development avoids increasing flood risk on-site or elsewhere and, where appropriate, incorporates sustainable drainage measures. Development must also comply with national policy on flood risk management and drainage design. Paragraph 176 (via footnote 62) of the NPPF states that householder development should not be subject to the sequential test, nor the exception test.

The site is located within Flood Zone 2 and has been submitted with a Householder Minor Extensions Flood Risk Assessment which sets out that floor levels within the proposed development will be set no lower than existing levels and electrical sockets will be raised a minimum of 400mm above floor level. Compliance with the submitted mitigation measures details will be conditioned as part of the approval.

Given the relatively limited scope of the additional built form and the proposed mitigation measures, for which compliance will be conditioned, the proposal is not considered to give rise to any unacceptable levels of increased flood risk at the site.

The proposal therefore complies with Core Policy 67 of the Wiltshire Core Strategy and the policy in the NPPF, ensuring that flood risk is not increased and that surface water drainage is appropriately managed.

Biodiversity Net Gain

Householder planning applications are exempt from the requirement to deliver Biodiversity Net Gain under Schedule 7A, Section 4(2)(a) of the Town and Country Planning Act 1990, as inserted by the Environment Act 2021.

CONCLUSION

The proposed development is considered to be acceptable in terms of its design, scale, and relationship with the surrounding area. It would not result in any unacceptable harm to the character of the locality, the amenities of neighbouring occupiers, or the interests of highway safety, heritage, or increase flood risk.

The proposal is therefore considered to comply with the relevant policies of the Wiltshire Core Strategy, the Wiltshire Local Plan, and the National Planning Policy Framework. As such, the application is recommended for approval, subject to conditions.

RECOMMENDATION:  That Planning Permission be GRANTED subject to conditions.