APPLICATION FOR HOUSEHOLDER PLANNING PERMISSION

DELEGATED REPORT

| APPLICATION REFERENCE: | PL/2026/01497 |
| --- | --- |
| APPLICATION TYPE: | Householder planning permission |
| SITE ADDRESS: | 1 Meadow Croft, Chilton Foliat, Hungerford, RG17 0UA |
| DEVELOPMENT DESCRIPTION: | Loft conversion, front porch and single storey side extension, creation of new parking area. |
| RECOMMENDATION: | That Planning Permission be GRANTED |

### PROPOSAL

### The proposal seeks planning permission for a loft conversion and single storey side extension, alongside the creation of a new parking area.

### SITE DESCRIPTION/CONTEXT

### The application relates to 1 Meadow Croft, Chilton Foliat in Hungerford. It is a single storey, semi-detached bungalow located within the Chilton Foliat Conservation Area. The house is of relatively modern construction with a traditional unassuming appearance within the street scene, consisting of red brick exterior walling under a hip-to-gable plain clay tile roof. It forms part of a row of bungalows of the same design, although it abuts the other three bungalows at a perpendicular angle and is accessed via a private dead-end road which adjoins the B4192, so it is the least prominent property within this collection of bungalows from key views within the conservation area. Three Grade II listed buildings are sited approximately 30 metres to the north of the site on the other side of the B4192.

### The application site is subject to the following planning policy constraints:

- Located within the Chilton Foliat Conservation Area.

- Located in proximity to several listed buildings.

- Washed over by the North Wessex Downs National Landscape.

- Located in the open countryside for planning purposes.

- Part of the site is located within Flood Zone 2.

### PLANNING HISTORY

No relevant planning history.

RELEVANT POLICIES AND GUIDANCE

Wiltshire Core Strategy:

- Core Policy 50: Biodiversity and Geodiversity

- Core Policy 51: Landscape

- Core Policy 57: Ensuring High Quality Design and Place Shaping

- Core Policy 58: Ensuring the Conservation of the Historic Environment

- Core Policy 61: Transport and New Development

- Core Policy 64: Demand Management

- Core Policy 67: Flood Risk

Chilton Foliat Conservation Area Statement December 2003

Wiltshire Design Guide: Appendix D – Design Guidance for Householder extensions

The National Planning Policy Framework (NPPF)

The Planning Practice Guidance

National Design Guide

CONSULTATION RESPONSES

Chilton Foliat Parish Council: No objection.

Highways: No objection subject to conditions.

Drainage: (Final response) No objection.

Arboriculture: No objection.

REPRESENTATIONS

No comments were received.

ASSESSMENT

Section 38(6) of the Planning and Compulsory Purchase Act 2004 states:

"If regard is to be had to the development plan for the purposes of any determination to be made under the planning Acts, the determination must be made in accordance with the plan unless material considerations indicate otherwise."

In accordance with this statutory requirement, this application has been assessed against the relevant policies of the Development Plan, including the adopted Wiltshire Core Strategy (2015) and, where applicable, any made Neighbourhood Plans.

Design and Impact upon the character and context of the area

Core Policy 57 - Ensuring High Quality Design and Place Shaping of the Wiltshire Core Strategy seeks to ensure that all new development is of high design quality, reflecting local context, character, and distinctiveness.

The proposed extension would be sited on the southwest corner of the existing dwellinghouse, extending to the rear and to the side of the existing property. It would be set back approximately three metres from the front elevation of the property so it would have a limited visual impact despite its visibility upon the street scene. The external walling would consist of blackened timber cladding external walling above a brick plinth, and the roof is proposed as a green flat roof. The proposed materials would contrast with the existing property but given its limited visibility upon the street scene, this would not have a detrimental visual impact. The extension would essentially wrap-around a small gable-roofed section of the property; given the competing roof forms there would be some complexity in the roof form which would not be perceptible from public viewpoints.

A loft conversion is also sought as part of the application; this would involve the addition of five rooflights to the front of the property and a first-floor window in the south gable end. The addition of these openings would not have a significant visual impact and is considered acceptable.

A relatively modest gable-roofed front porch is proposed on the front elevation; this would have a traditional appearance, with a timber frame and concrete roof tiles to match the existing roof. The side walls are proposed as blackened timber cladding to match the rear extension; this would tie together the two additional elements visually and would not be particularly prominent due to the small scale of these walls, so this is also considered acceptable on visual amenity grounds.

The proposal also involves the addition of a parking space to the front of the property; this would involve the removal of a relatively small section of walling from the front boundary and the surfacing of half the front garden with permeable bound gravel. This area is currently gravelled, so the addition of the driveway would not see any particular loss of greenery to the front of the property and the loss of the wall would be relatively modest. A large section of walling on the front boundary and green garden space would remain to the south of the driveway, so the appearance of the property within the street scene would be largely retained.

The proposal reflects the guidance set out in Appendix D by remaining subservient to the host dwelling and respecting the scale and character of the surrounding built form.

Having regard to the above, the proposal complies with the design principles set out in Core Policy 57 of the Wiltshire Core Strategy and Appendix D of the Wiltshire Design Guide.

Heritage Impact

From the point of view of the historic environment the main statutory tests are set out within the Planning (Listed Building and Conservation Areas) Act 1990.

Section 66 requires that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

Section 72(1) of the Act requires the Council to pay special attention to the desirability of preserving or enhancing the character or appearance of designated Conservation Areas.

The Council’s Core Strategy Policy CP58 ‘Ensuring the conservation of the historic environment’ requires that designated and non-designated heritage assets and their settings will be conserved, and where appropriate enhanced, in a manner appropriate to their significance and encourages proposals which improve the condition of heritage assets at risk.

The Council’s Core Strategy CP 57: Ensuring high quality design requires a high quality of design in all new developments. The policy requires applicants to “demonstrate how the proposal will make a positive contribution to the character of Wiltshire through (amongst other things) responding to the value of the natural and historic environment and being sympathetic to and conserving historic buildings and landscapes. Development is required to respond positively to the existing in terms of building layouts, built form, height, mass, scale, building line, plot size, elevational design, materials, streetscape and rooflines to effectively integrate the building into its setting.

The NPPF outlines government policy for planning. Chapter 16 of the NPPF 'Conserving and enhancing the historic environment' sets out the Government's high-level policies concerning heritage and sustainable development.

The site is located in the Chilton Foliat Conservation Area and is sited in proximity to several listed buildings. The property is of relatively modern construction, so it does not offer any heritage value to the conservation area, but it does represent a modest, unassuming building which comfortably blends into the street scene and reflects the verdant nature of the locality.

The proposed works would be relatively modest and while they would be visible upon the street scene, they would not have a significant impact upon the visual appearance of the building and would have no material impact upon any key views within the Conservation Area or within the setting of any of the nearby listed buildings due to the discreet nature of the plot.

The proposed addition of parking to the front of the property would involve the demolition of a small section of the front boundary wall; this is not a historic wall and while it contributes positively to the street scene, it would only be a small section which would be lost with a large amount retained to the front of the property, so this is not considered to have a materially harmful impact upon the conservation area.

Therefore, while the proposed extension would incorporate materials which represent a departure from the main dwellinghouse and the addition of the driveway would see a small loss of the boundary wall, the proposal would not result in any material harm to designated heritage assets.

Having had special regard to the desirability of preserving the setting of nearby listed buildings (Section 66(1)) and having paid special attention to the desirability of preserving or enhancing the character or appearance of the Chilton Foliat Conservation Area (Section 72(1)), it is concluded that the proposal would result in no harm to the significance of designated heritage assets. Great weight has been given to the conservation of heritage assets in this assessment. The proposal therefore accords with Core Policy 58 and Chapter 16 of the NPPF.

Amenity

Point vii of Core Policy 57 - Ensuring High Quality Design and Place Shaping of the Wiltshire Core Strategy seeks to ensure that developments do not adversely affect neighbouring properties.

The National Planning Policy Framework (NPPF), particularly Paragraph 135(f), stresses that planning decisions should ensure developments provide a high standard of amenity for existing and future users.

The proposed development has been assessed in terms of its potential impact on the residential amenity of neighbouring properties, with particular regard to loss of light, overshadowing, loss of privacy, and overbearing effects.

The proposed side/rear extension would be relatively modest in scale and would be limited to the ground-floor level with a flat roof. Despite is siting close to the shared boundary with 2 Meadow Croft, it would not be sited close to the neighbouring properties, only their garden area. Given its limited height, the extension would not give rise to any unacceptable levels of overshadowing or overbearing impact. The front porch would also be single storey and modest in scale, being sited away from any shared boundaries, so this would also result in no overshadowing or overbearing impact.

Five rooflights are proposed in the front (east) roofscape; the closet neighbouring properties to the east are set 23 metres away, so these would not give rise to any unacceptable overlooking. A new first-floor window is also proposed in the south side elevation; there are no neighbouring properties to the south, so this is considered acceptable on residential amenity grounds.

All other proposed openings would be located on the ground floor where they pose no risk of introducing any unacceptable levels of overlooking.

The proposal therefore accords with Core Policy 57 point vii in respect of safeguarding residential amenity and Appendix D of the Wiltshire Design Guide.

Highways Impact

Core Policy 57 (xiv) of the Wiltshire Core Strategy seeks to ensure that development must meet the requirements of Core Policy 61. This policy requires proposals to be capable of being served by safe access to the highway network. Core Policy 64 seeks to ensure compliance with parking standards set out in the Local Transport Plan.

The National Planning Policy Framework (NPPF), particularly Paragraph 115 and 116, requires that development proposals do not have an unacceptable impact on highway safety and that safe and suitable access to the site can be achieved for all users.

Whilst the proposal would not achieve full compliance with Wiltshire’s parking standards for a three-bedroom dwelling, the site currently provides no off-street parking. The introduction of a single space therefore represents a material improvement over the existing situation and, having regard to the consultation response from the Highway Authority raising no objection, the residual level of under-provision is not considered to result in a severe impact on highway safety or parking conditions in accordance with paragraph 115 of the NPPF.

Wiltshire Council’s Highways Department has been consulted on the application and have responded with no objections subject to conditions, stating that the property currently benefits from no off-street parking so the addition of a parking space would offset the additional accommodation on site and there would therefore be no additional strain on on-street parking in the local vicinity. The requested conditions relates to the access consisting of a consolidate material for the first 5 metres, no gates being added to the access, the implementation of surface water drainage measures, and the completion/retention of the proposed parking space for the lifetime of the development, alongside an informative alerting the applicant’s attention to the requirement for a dropped kerb licence. All requested conditions and the informative will be included as part of the approval.

The proposed development would not result in any adverse impact on highway safety. Whilst the level of parking provision would remain below adopted standards, the introduction of a single space represents a material improvement over the existing situation and is considered acceptable in this instance. Access arrangements are acceptable.

The proposal is therefore considered to comply with Core Policies 57 (xiv), 61 and 64 of the Wiltshire Core Strategy, the NPPF, and the Wiltshire Local Transport Plan.

National Landscape

Core Policy 51 of the Wiltshire Core Strategy requires all development to protect, conserve, and where possible enhance the character and quality of Wiltshire’s landscapes, giving particular weight to designated areas such as National Landscapes.

The National Planning Policy Framework (NPPF), in Paragraph 176, states that “great weight” should be given to conserving and enhancing landscape and scenic beauty in National Landscapes (formerly Areas of Outstanding Natural Beauty). Development within or affecting the setting of a National Landscape must demonstrate that it will not result in unacceptable harm to the special qualities of that area.

The proposed works would be limited to relatively modest additions to an existing dwellinghouse within the established residential curtilage. As there would be no encroachment into the open countryside and the proposed works would be viewed in context with the existing site, there is no risk of any landscape harm associated with the proposed works.

The proposal, by virtue of its scale, siting, and design, would not harm the special qualities or scenic character of the North Wessex Downs National Landscape.

The development therefore complies with Core Policy 51 of the Wiltshire Core Strategy and Paragraph 176 of the NPPF in conserving and enhancing the National Landscape.

Drainage and Flood Risk

Core Policy 67 of the Wiltshire Core Strategy and paragraph 181 of the NPPF requires that all new development avoids increasing flood risk on-site or elsewhere and, where appropriate, incorporates sustainable drainage measures. Development must also comply with national policy on flood risk management and drainage design. Paragraph 176 (via footnote 62) of the NPPF states that householder development should not be subject to the sequential test, nor the exception test.

Part of the site is located in Flood Zone 2, the house is located 19 metres to the north of a river, and groundwater levels at the site are between 0.5m and 5m below the ground surface so there is a risk of flooding to subsurface assets.

Wiltshire Council’s Drainage department has been consulted on the application and initially requested further information; following this request, exceedance plans have been submitted by the applicant, and the drainage officer has confirmed that they now hold no objections to the proposal.

The conclusion of the Drainage officer is agreed; the proposed works would be relatively modest and adequate consideration has been afforded to reducing water runoff and ensuring adequate drainage at the site. The submitted exceedance plan will be added to the plans list to ensure compliance with the submitted details.

The proposal constitutes minor (householder) development and is therefore appropriate in principle within Flood Zone 2. The submitted exceedance information and the absence of objection from the Lead Local Flood Authority confirm that the development would not increase flood risk on or off site.

The proposal therefore complies with Core Policy 67 of the Wiltshire Core Strategy and the NPPF, ensuring that flood risk is not increased and that surface water drainage is appropriately managed.

Biodiversity Net Gain

Householder planning applications are exempt from the requirement to deliver Biodiversity Net Gain under Schedule 7A, Section 4(2)(a) of the Town and Country Planning Act 1990, as inserted by the Environment Act 2021.

CONCLUSION

Overall, the proposal would deliver improved residential accommodation with no identified harm to heritage assets, landscape character, neighbouring amenity, or highway safety. Whilst there is a minor shortfall in parking provision relative to adopted standards, this is outweighed by the existing baseline position and the absence of any objection from the Highway Authority. The development is therefore considered acceptable when assessed in the round.

The proposed development is considered to be acceptable in terms of its design, scale, and relationship with the surrounding area. It would not result in any unacceptable harm to the character of the locality, the amenities of neighbouring occupiers, or the interests of highway safety, heritage, or ecology.

The proposal is therefore considered to comply with the relevant policies of the Wiltshire Core Strategy, the Wiltshire Local Plan, and the National Planning Policy Framework. As such, the application is recommended for approval, subject to conditions.

RECOMMENDATION:  That Planning Permission be GRANTED subject to conditions.