2026/03735 Permission in Principle

Site: Praters Farm, Easton Town, Sherston, Malmesbury, SN16 0PS

Proposal: Permission in Principle (PIP) for the Proposed Erection of Up to Two (2) Dwellings

________________________________________________________________________

Recommendations:

| X | Comment |
| --- | --- |
|  | Support in principle subject to amendments to the scheme (as set out below) |
|  | Support subject to conditions (as set out below) |
|  | Object (for reasons set out below) |
|  | No objections |

Scope of comments: the following comments relate to the built historic environment.

Site Description:

Praters is a former farmhouse now country house dating from the early 17th century with later 17th century west wing addition. It is a two-storey property with L shaped plan after later 17th century addition.

The curtilage includes a separately listed former Threshing Barn located 15 m south west of the house. It has 17th century origins and was extensively restored from a ruinous state in the 1970’s to a photographic studio. It is constructed in rubble stone with flush rusticated dressed stone quoins. The roof was replaced in the 1970’s and is clad in concrete tiles. The barn has a cruciform plan with east and west midstreys.

Praters Farmhouse is in Easton Town which is at the eastern edge of Sherston and situated along a rural lane 50m south of the B4040 east-west road that forms the spine of Sherston.

The farmhouse is positioned outside the development boundary and Sherston Conservation Area; however, it is located within the Cotswolds AONB.

The PIP application site is the agricultural field 0.28 h immediately adjacent, west of the farmhouse and farm buildings including the Threshing Barn and formed part of the larger Praters farm.

The field has a street frontage to the north onto the B4040 behind a traditional dry-stone retaining wall. The field is about 1m above the pavement level. It includes some established hedgerow and trees. The other boundaries are heavily established trees to a hedgerow with post and rail fence to the west and to the east boundary with Praters Farmyard a 2m dry stone walling that returns halfway across the south boundary. The rest of the south and east boundary is post and rail with established hedgerow and trees.

Heritage Assets:

Praters Farmhouse Easton Town Sherston grade II

Barn to SW of Praters Farmhouse Easton Sherston grade II

Home Farmhouse Easton Sherston grade II

Outbuilding to the North of Home Farmhouse grade II

Relevant planning History:

2025/02862 Householder and 2025/03134 LBC Conversio of Existing Studio

2024/11612 Householder and 2025/00178 Extension and alterations to Praters Farmhouse

2025/08695 Permission in Principle (PIP) for the Proposed Erection of Up to Five Dwellings - Refused

The Proposal: Permission in Principle (PIP) for the Proposed Erection of Up to Two (2) Dwellings

Policy: From the point of view of the historic environment the main statutory tests are set out within the Planning (Listed Building and Conservation Areas) Act 1990. Sections 16 (LBC) and 66 (PP) require that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

The NPPF outlines government policy, including its policy in respect of the historic environment. Section 16 of the NPPF 'Conserving and enhancing the historic environment' sets out the Government's high-level policies concerning heritage and sustainable development.

Paragraphs 213-15 of the NPPF require a balanced approach with any harm which would be caused being weighed against the potential public benefits which might be achieved.

Paragraph 210 requires local planning authorities should take account of the desirability of sustaining and enhancing the significance of heritage assets and of new development making a positive contribution to local character and distinctiveness.

National Planning Practice Guidance provides guidance on interpreting the NPPF.

The Council’s Core Strategy Policy CP58 ‘Ensuring the conservation of the historic environment’ requires that “designated heritage assets and their settings will be conserved, and where appropriate enhanced, in a manner appropriate to their significance.”

Wiltshire Council’s Core Strategy Policy CP57 ‘Ensuring high quality design and place shaping’: A high standard of design is required in all new developments, including extensions, alterations, and changes of use of existing buildings. Development is expected to create a strong sense of place through drawing on the local context and being complementary to the locality.

Historic England Advice Note 2 – Making changes to Heritage Assets illustrates the application of policies set out in the NPPF in determining applications for PP and LBC.

Issues: Impact on the setting of the Listed Buildings.

Significance of assets and contribution of setting: The NPPF defines significance as the “value of a heritage asset to this and future generations because of its heritage interest.

The interest may be architectural, or historic as set out in the Town and Country Planning (Listed Building and Conservation Areas) Act 1990 S 1.

The NPPF defines significance as the “value of a heritage asset to this and future generations because of its heritage interest. The interest may be, architectural, or historic. Significance derives not only from a heritage asset’s physical presence, but also from its setting.” Historic England assesses significance in a similar manner, referring to evidential, historic, aesthetic, and communal values of a place.

In this case the significance of the listed buildings comes from their historic fabric and vernacular architecture - its form, layout and architectural expression/detailing and use of local design approaches and materials which have some significant aesthetic and architectural interest.

The dry-stone boundary walling to site frontage onto street whilst not curtilage listed is of significance as historic and architectural feature that connects with adjoining dry stone walls and forms part of character of entrance to village and Conservation Area  from Easton Town and are considered to be Non Designated Heritage Assets.

Praters Farmhouse is significant, and the Barn was listed for its contribution to the group with the Farmhouse.

Assessment:

Following a refusal of the previous application for Permission in Principle for 5 (five) dwellings the proposal under PIP has been amended during the application form 3 (three) to 2 (two) dwellings sited at the north end of the side either side of the new central access integrated into the dry stone walling.

‘The permission in principle consent route is an alternative way of obtaining planning permission for housing-led development which separates the consideration of matters of principle for proposed development from the technical detail of the development. The permission in principle consent route has 2 stages: the first stage (or permission in principle stage) establishes whether a site is suitable in-principle and the second (‘technical details consent’) stage is when the detailed development proposals are assessed.’

The Planning (Listed Building and Conservation Areas) Act 1990. Sections 16 (LBC) and 66 (PP) require that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

This ‘special regard’ obligation requires a Local Planning Authority to consider the setting of listed buildings. This means the standard assessment for a PIP is overridden by this legal duty to protect the heritage assets affected.

The application has been amended during the consideration of the application to 2 (two) dwellings sited to the north end of the field site adjoin the street and dry stone walling.

The removal of the third dwelling deeper into the site has significantly reduced the harm to the setting of the heritage assets Praters Farmhouse and Praters Threshing Barn and boundary dry stone walls.

It is considered that the revised proposal to two dwellings sited to road frontage behind existing dry-stone wall, will have more limited setting impact on the Heritage Assets and on balance may now be acceptable to progress to the second (‘technical details consent’) stage where the detailed development proposals including detailed design of dwellings, layout and access are assessed.

During the detailed design phase specific care is required to design dwellings that would integrate with the local architectural distinctiveness, including materials and streetscape and character and appearance of Sherston/ Easton Town. This should not necessarily mean following traditional designs and can be contemporary architectural design, but it must respond and reflect the local distinctiveness and materials.

Specific care is also required in the design and layout to integrate the site access with the existing dry-stone walls to maintain the consistency of this NDHA that contributes positively to the local character and appearance fronting the road at the entrance of the Conservation Area and village.

In addition, the half the application site field (south half) to the west of Praters Farmhouse and Barn should be left for Biodiversity Net Gain or an Orchard extension to assist in enhancing and better revealing the setting of the heritage assets.

Heritage Conclusion and Balancing: The NPPF confirms that when considering the impact of a proposed development on the significance of a designated heritage asset, great weight should be given to the asset’s conservation (and the more important the asset, the greater the weight should be). It makes clear that any harm to a designated heritage asset requires clear and convincing justification.

The ‘special regard’ under Town and Country Planning (Listed Building and Conservation Areas) Act 1990. S 16 and S.66 (PP) require that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

This obligation requires a Local Planning Authority to consider the setting of listed buildings and to protect the heritage assets affected.

This revised PIP could progress beyond stage one as the harm to heritage assets has sufficiently reduced and the assets settings should be enhanced and better revealed through landscape/BNG and architectural design as part of the Second stage of the process.