| T5 | Irish Yew | 15M | 1000MM | M | - Significant landscape feature. - Main stem bifurcates @1.6M on south side and 0.1M on north side. - Tree again bifurcates @2M, narrow mouthed included union. Stem – stem wielding evident. - Multiple large diameter dead primary branches in crown. - Pruning stubs/snags, lower crown – indicator of poor pruning practice. - Crossing/rubbing branches throughout crown. - Secondary and tertiary branches preventing spread of streetlight pattern. - Crown clean – remove dead primary branches. - Prune to clear streetlight by 2M. | <1/M N/A N/A | 1 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |