Council Planning Consultation Response

ECOLOGY

| Officer name: | Niall Machin |
| --- | --- |
| Date: | 29.07.26 |
| Application No:. | PL/2026/03045 |
| Proposal:. | Outline planning application for the erection of up to 49 dwellings with associated access, public open space, landscaping, drainage and associated infrastructure (all matters reserved except access). |
| Site Address:. | Land off Damask Way, Warminster |
| Case Officer: | Steven Sims |

Recommendations:

|  | No Comment |
| --- | --- |
|  | Support |
|  | No objection |
|  | Condition (no objection subject to conditions) |
| X | Objection - further information required |
|  | Objection in principle |

Further Information Required:

|  | Issue | Policy/Legislative Compliance | Date information requested & Further information required | Satisfactorily addressed (Document & Date submitted) |
| --- | --- | --- | --- | --- |
| 1 | HRA | Conservation of Habitats and Species Regulations 2017 | Lack of information in order to produce an Appropriate Assessment |  |
| 2 | Impact to adjoining LNR | NPACA 1949 Core Policy 50 | Local Nature Reserve Mitigation and Maintenance Plan |  |
| 3. | Ecological Impact Assessment | Core Policy 50 NPPF | The EcIA is incomplete (see text below): lack of evaluation; lack of survey data; incomplete assessment of grasslands/habitats |  |
| 4. | Protected Species data | Core Policy 50, NPPF, WCA 1981 (as amended) | The following is required: - Emergence surveys of trees with bat potential impacted by the development - Heat maps or similar showing bat usage of the site, including foraging and commuting routes/areas - Bat Mitigation Strategy - Toad Mitigation Strategy - Invertebrate Survey |  |
| 5. | Phosphate mitigation | Conservation of Habitats and Species Regulations 2017 | Lack of appropriate/inconsistent sustainable urban drainage approach; lack of evidence for P mitigation and credit availability. The current drainage submission does not yet provide sufficient evidence to demonstrate that runoff quantity, runoff quality, exceedance flows and construction-phase drainage risks have been fully addressed and that there will be no significant impact to the SAC and LNR. |  |
| 6. | Biodiversity Net Gain | Environment Act 2021 CP50, NPPF | Lack of evidence, incomplete condition sheets, uncertainty of 10% gain compliance |  |
| 7. | Ecological mitigation | CP50, NPPF | Lack of evidence that valued features and habitats would be retained and enhanced. A CEMP is required pre approval. |  |
| 8. | CP50 | CP50 | Currently, the development does not comply with Policy CP50 as it lacks information on key ecological features, lacks a proper assessment of the value of such features and fails to adequately demonstrate how sites, habitats and features would be protected and enhanced. |  |

The outstanding document(s) listed above are needed to enable the council’s ecology team to consider all the relevant impacts and benefits of the proposed scheme.   The council’s ecologists will provide a further response once all the above information has been submitted via the case officer.

Please Note:  When resubmitting a revised document, ALL changes must be highlighted to enable review.

Thank you for your consultation on the above application. I have reviewed the submitted documents against OS maps, aerial photographs of the site and surrounding area, together with GIS layers of statutory and non-statutory designated sites and existing records of protected species.

Information considered:

- Ecological Impact Assessment, NPA 31.03.26, ref NPA 30142 0400

- Ecology Parameters Plan NPA 30.03.26 ref 30142

- Nutrient Neutrality Assessment and Mitigation Strategy, SLR 30.03.26 ref 402.066106.00001, Rev3

- Statutory BNG metric NPA dated 30.03.26 and associated condition assessments

- Lighting impact assessment, QODA, 01.04.25, NOR00866D-QODA-XX-XX-RP-E-0001-02

COMMENTS:

Designated sites

The River Avon SAC and SSSI is around 80m to the south of the site. Smallbrook Meadows LNR is just 45m to the north of the site. The application site is on a raised hill that slopes down to both these wetland sites meaning that there is the potential for construction sediment disturbance, stormwater discharge, overground flows and pollution to enter the SAC/SSSI and LNR. The site is also within the Salisbury Plain SPA Mitigation Strategy buffer zone

We note Natural England’s (NE) comments (dated 16th July 2026) which states:

On the basis of information provided, Natural England’s advice is that this proposed development will require mitigation measures intended to avoid or reduce the likely harmful effects on a European site which cannot be taken into account when determining whether or not a plan or project is likely to have a significant effect on a site and requires an appropriate assessment (following the People Over Wind ruling by the Court of Justice of the European Union). For this reason, the proposed mitigation measures need to be formally checked and confirmed by your Authority, as the competent authority, via an Appropriate Assessment (AA).

In relation to the phosphorous budget, NE state:

The NNAMS has used a water use of 95 l per person per day (ppd) in the phosphorus calculations. Natural England has no objection to this approach, however, the final water use standard for the development must incorporate the 10 l ppd buffer required by the standard methodology. Any permission must therefore ensure proposed new development is required to limit water use to 85 l ppd and not the 95 l ppd set out in the NNAMS. Provided this is acceptable to the applicant then I can confirm Natural England has no further concerns relating to the calculations or mitigation measures proposed. Please note as the Competent Authority it is your responsibility to ensure that you are confident that there is sufficient information to support the values used in the phosphorus calculation, and that the nutrient budget calculation is correct. Your authority should however ensure that all the necessary mitigation measures are secured, including:

- Ensuring that the development limits water use to 85 l ppd.

- That the permission secures SuDS identified and ensures appropriate maintenance in perpetuity.

- That the scheme delivers and thereafter retains the requisite areas of greenspace (1.05 ha) as set out in the phosphorus calculations.

- That the requisite phosphorus credits are secured prior to commencement.

In relation to potential run off to the SAC, NE state:

- River Avon SAC – Surface water runoff The proposed development site sits on high ground with poor drainage, steep slopes and in close proximity to the River Wylye (notified as the River Avon System SSSI) that feeds into the River Avon SAC. It is therefore likely that the development will pose a high risk to the designated watercourses from diffuse pollution, both during construction and via urban runoff from the completed development. Your Authority’s AA will therefore need to carefully consider the measures proposed and their likely effectiveness for preventing contaminated water entering the designated sites.

- Runoff during construction Given the high risk posed from the site Natural England advises that a detailed Construction Environmental Management Plan (CEMP) is provided prior to determination. The CEMP will need to address how soil runoff will be prevented throughout the construction phases including during periods of heavy rain. Measures might include minimising the areas of turf stripping as the development progresses and ensuring high risk works are completed during drier periods. In addition, measures for controlling and treating construction run off should also be secured.

- Urban run off The scheme proposes to collect surface water from permeable pavements in tanks before discharge to the drainage ditch network to the south of the site, which connects to the River Wylye. Natural England welcomes the use of vortex separators to intercept oil and silt as set out in the submitted Flood Risk Assessment & Drainage Strategy. Although the drainage strategy concludes that the measures prosed will provide “acceptable” levels of attenuation of pollutants further consideration should be given to the conservation status of the receiving water courses with a view to providing the enhanced pollution prevention standards. Further consideration should be given to a greater use of SuDS features on site, in particular the incorporation into the layout of a comprehensive system of filter strips, vegetated swales, bioretention areas (rain gardens) and detention basins to facilitate as far as possible discharge to ground and effectively treat the surface water that leaves the site. Such features will require sufficient space within the layout and so designs should be considered at the Outline application stage, so as to ensure the necessary measures can be incorporated into the final development.

Habitat Regulations Assessment (HRA)

The application site is located adjacent to the statutorily designated River Avon SAC and therefore has potential to result in significant adverse impact on the special features of that site. Appropriate Assessment (AA) under Section 63 of the Conservation of Habitats and Species Regulations 2017 (as amended) must be carried out by the relevant Competent Authority (the LPA) to determine the potential significant effects and the suitability of any measures proposed to avoid or mitigate those effects.

Insufficient information has so far been submitted to enable the AA to be undertaken, which must be completed and the conclusion agreed with Natural England prior to the application being determined.

Salisbury Plain SPA

This application lies within the 6.4km zone of influence of the Salisbury Plain SPA and in light of the HRA for the Wiltshire Core Strategy and the HRA for the Wiltshire Housing Site Allocations Plan it is screened into Appropriate Assessment due to the potential impact of recreational pressure on stone curlew in combination with other plans and projects. A strategic Appropriate Assessment (AA) has been prepared pertaining to development within the 6.4km zone of influence that could result in a likely significant effect on the SPA as a result of an increase in recreational / visitor pressure. The AA reached a conclusion of no adverse effects on the integrity of the SPA or its qualifying features provided that the council’s mitigation strategy continues to be implemented. Annual stone curlew monitoring and protection measures continue to be secured by the Council. In January 2026 Natural England (NE) confirmed that the strategic AA for Salisbury Plain SPA continues to be supported by NE until end of December 2026. We note that NE have stated that they are ‘satisfied that provided the contribution to the agreed Salisbury Plain SPA Mitigation Strategy is secured then the proposals will not result in adverse effects on the integrity of the Salisbury Plain SPA / SSSI’.

Smallbrook Meadows LNR

We note NE’s comments in relation to the LNR:

Impacts on the Smallbrook Meadows Nature Reserve The development lies in very close proximity and has a direct footpath connection to the Smallbrook Meadows Wiltshire Wildlife Trust Nature Reserve and will therefore inevitably lead to greater recreational use of the reserve. Increased visitor numbers are likely to increase the ongoing management costs for the nature reserve (e.g. through increased requirement for wardening visits, increased replacement infrastructure costs, etc.). The scheme should therefore provide additional measures to ensure it does not lead to a deterioration of the reserve’s wildlife interests or result in significant additional management costs for the reserve managers. This might be achieved through the delivery of an agreed annual costed access management plan for the reserve that provides long-term support for additional wardening and the maintenance of access infrastructure (e.g. paths, gates, interpretation) on the reserve. Natural England would be happy to discuss with the Council or applicant how this might be achieved.

We agree with NE’s comments and therefore request a Mitigation and Maintenance Plan for the LNR which addresses:

- Modelling of predicted visitor/footfall increases and urban development pressures to the LNR as a result of the scheme

- Mitigation to address increased management costs for the LNR in relation to wardening, repairing and reinstating infrastructure, potential pollution/run -off impacts etc

- A costed Adaptive Plan to ensure the LNR is not adversely affected by the scheme

Ecological Impact Assessment

An Ecological Assessment (EcIA) has been submitted in support of this application however this contains insufficient information to determine potential impacts on the following habitats and protected/ notable species:

- It is noted that the EcIA does not assess the level of impact significance for the species affected: the EcIA needs to be revised to assess level of impact as per Chapter 4 of CIEEM guidance.

- Table 4.4 of the EcIA lists 6 trees with bat roost potential which could be impacted by the proposals: emergence bat surveys of these trees is required in order for the LPA to assess impact.

- Lack of data and information on bat use of site – where and how bats are using the site including foraging and commuting routes and a lack of heat maps

- Given the presence of notable invertebrate species and the mosaic of habitats present, including NERC Priority Habitats, we recommend that an invertebrate survey is undertaken to fully assess the value of the site for this group.

- Insufficient evidence has been supplied in relation to toad presence and mitigation.

- Lack of evidence to confirm status, extent and condition of grassland habitats on site

In carrying out its statutory function, the LPA must be reasonably sure that the proposal will not result in significant adverse effects on protected habitats or species. The information outlined above must therefore be submitted and reviewed by the LPA’s ecology team prior to determination of the application. Supporting text for Policy CP50 states that ‘for the purposes of evaluation in ecological impact assessment, the value of undesignated habitats and species should be measured against published selection criteria where possible’.

Protected Species

Methodologies for protected species surveys were broadly acceptable. We note that the dormouse surveys spanned 4 months (August to November).

Bats

Table 4.4 of the EcIA lists 6 trees with bat roost potential which could be impacted by the proposals: emergence bat surveys of these trees is required in order for the LPA to assess impact.

We note NPA recorded greater horseshoe, lesser horseshoe and barbastelle bats using the site. It is also likely that Bechstein’s bats use the site having been recorded on the LNR to the north (and NPA did not identify any to species). The data shows that there is likely to be greater horseshoe, lesser horseshoe, Myotis and barbastelle roosts ‘within the vicinity of the site’.

We note that NE have asked for further information on bats (their comments dated 16th July):

Bat Activity – further information required The submitted survey results demonstrate that the site is well used by foraging and commuting bats, most notably pipistrelle species, but also regular use by greater and lesser horseshoe bats (Annex II species). The high activity levels are not surprising given the high quality of neighbouring habitats associated with the Smallbrook Meadows Nature Reserve and river corridors. The submitted information, however, has provided no indication of where on site the bats were recorded (e.g. activity heat maps). It is therefore not possible to determine whether the proposal will impact on commuting routes or features favoured by the bats using the site. This is information is important for ensuring the design of the scheme does not reduce ecological connectivity through the site.

We agree with NE’s comments that there is insufficient evidence in the EcIA regarding bat foraging, commuting and roosting on and across the site.

In terms of lighting, the EcIA states:

Coherent network of dark bat habitat connected to the wider landscape maintained through: buffers between built development and woodland edge; these buffers to be kept dark (<0.2 horizontal / 0.4 lux vertical) – see Lighting Impact Assessment to see how this could be achieved; tree and hedge/scrub planting at the site entrance to provide for continued connectivity; planting species-rich native hedgerows (e.g. to rear of northern gardens).

The Lighting Strategy states: ‘Following coordination with the ecologist, the external lighting provision shall be minimised to not exceed 0.2lx on the horizontal plane and 0.4lx on the vertical plan at 7.5m away from where sensitive inhabitants live or commute’.

We recommend that these dark buffers are 10m. As designed, the Lighting Strategy shows no dark corridors across the site, meaning that any current commuting or foraging routes across the site will be lost.

A Bat Mitigation Strategy is required to demonstrate how bats will use the site for foraging, commuting and roosting. A bat sensitive Lighting Strategy is also required.

Toads

Common toads are listed as a priority species under the Natural Environment and Rural Communities (NERC) Act (2006). The site is located on a key migratory route for a large population of common toad. We note NE’s comments in relation to toads:

The locality is well known for supporting significant toad populations with migratory routes though the site and neighbouring roads, with significant road causalities occurring each year. It is disappointing therefore that the submitted ecological assessment has not fully considered any likely impacts on the local toad population from obstructing toad movement across the site and through increased mortality on local roads due to increased local traffic. Further, the mitigation measures proposed are limited and provide no mechanism for reducing mortality on the new roads serving the development or on the local road network. In our view given the sensitivity of the locality a comprehensive toad mitigation strategy should be agreed and secured. In additional to the measures set out in the submitted ecological assessment the strategy should consider the following potential mitigation measures:

- Increased use of SuDS throughout the site designed to provide habitats that will facility the movement of toads through the development. E.g. vegetated swales running north south through the development.

- Design of garden boundaries, roadside curbs etc to allow movement of amphibians.

- Incorporation of road tunnels / culverts to new and existing local designed to allow passage by amphibians.

- Provision of road signage warning of toad crossings.

- Support for local toad patrol groups.

Increased housing and traffic in this area is likely to have an adverse impact on the local toad population. Off-site, the toad population along Smallbrook Road will face additional pressures from traffic associated with approximately 50 new homes, with additional vehicle movements along this narrow single-track lane.

We agree with NE’s view and recommendations and require robust and comprehensive mitigation measures for toads set out in a Toad Mitigation Strategy.

Invertebrates

The EcIA highlights the presence of several important invertebrate species on site including two Priority Species as listed under Section 41 of the Natural Environment and Rural Communities (NERC) Act 2006, the Nationally Scarce rugged oil beetle Meloe rugosus and the Vulnerable small heath butterfly Coenonympha pamphilus. We also note that a Nationally Scarce solitary wasp Lestiphorus bicinctus, the bee-wolf Philanthus triangulum and the rare digger wasp Lestiphorus bicinctushas (nationally notable) have all been recorded in the grassland on site. Theses are all rare species for Wiltshire and reflect the unusual habitats and situation on site. Other notable and priority invertebrates occur in Smallbrook Meadows Local Nature Reserve (LNR) adjacent to the north. Given the presence of these species and the mosaic of habitats present, including NERC Priority Habitats, we recommend that an invertebrate survey is undertaken to fully assess the value of the site for this group.

Habitats

The EcIA conducted by NPA found an area of lowland dry acid grassland on site, which is a Priority Habitat under the NERC Act 2006.

I visited the site on 30th June 2026. I also note the comments from Wiltshire Senior Ecologist Louisa Kilgallen on the previous application for this site (17/12348/OUT) who stated in her response (dated 15th May 2018, amended 5th June 2018):

From walking around the area is clear that the application site cannot be viewed in isolation. It is completely surrounded by species rich habitats, including scrub, woodland, wetlands, road verges, mature trees, grazed pastures and regenerating woodland habitats. Together they form a matrix which is likely to support high levels of biodiversity. The field itself is not without value. It supports a wider range of grass species than identified in the EcIA as well as large swathes of sheep’s sorrel and field woodrush indicating the site is has not been subject to reseeding and fertilizer treatment. Contrary to the conclusions in the EcIA, the invertebrate abundance and biodiversity associated with such vegetation can be expected to be high especially with light levels of sheep grazing and this will contribute to the numbers and range of birds, mammals, amphibians and reptiles in the local area.

I agree with Ms Kilgallen’s assessment. In particular, I note that an area of acid grassland occurs on site – this Priority Habitat is uncommon in Wiltshire, with small areas occurring in the south of the County around the New Forest.

I note that a phase 1 habitat survey was undertaken by Sharon Pilkington, a consultant judged by the Council to be an expert on Wiltshire’s vegetation and habitats, in 2020. This showed that the eastern field is classified as semi-improved grassland with acid grassland indicators and there is a small area of unimproved lowland dry acid grassland, a habitat of principal importance for conserving biodiversity. This report stated:

The floristic composition of the acid grassland places it within the U1 Festuca ovina – Agrostis capillaris – Rumex acetosella grassland of the National Vegetation Classification (NVC). This is the characteristic lowland acid grassland community of southern England but in many places (including Wiltshire) it is now rare, having been lost to intensification of agricultural management or neglect. Lowland dry acid grassland is listed as Section 41 Habitat of Principal Importance in England under the Natural Environment and Rural Communities (NERC) Act 2006. Even if it is not directly within the development footprint, development nearby is likely to scare off the rabbits that maintain the vegetation in its current condition.

In addition to direct loss of grassland habitat from development, there is a significant risk that the remaining areas of retained grassland will be degraded/reduced through:

- Construction activities including vehicle tracking and storage of materials, site cabins etc

- Change in long term management •

- Other measures aimed at creating specific non acidic grassland types for BNG.

The 2020 report also categorised the remaining whole of the eastern grassland as ‘other neutral grassland’: this contradicts the NPA habitat survey which classifies a large central area as only ‘modified grassland’. The photo used to show this modified grassland by NPA shows an area close to the existing housing on the west of the site, not the central eastern area. The condition assessment for the BNG metric indicates that the northern and eastern areas of ‘other neutral grassland’ have an ‘acid tendency’, indicating that the extent of acid grassland on site is probably underestimated. This is also an indication that with appropriate management the extent of acid grassland on this site could be increased.

Further information (photographs, species lists, condition assessments) is required to justify NPA’s categorisation of the central western area as modified grassland.

We note that the Wiltshire and Swindon Local Nature Recovery Strategy classifies this land as an area to ‘create more mixed habitats’ especially for bats, plus maintenance and enhancement of the areas of woodland.

Phosphate mitigation and drainage/run off

We have the following comments on the SLR Nutrient Neutrality Assessment and Mitigation Strategy (‘the NN strategy’).

As per NE’s comments, water usage should be 85 l/person/day.

SLR state that ‘All of the surface water runoff from the proposed site will be collected in tanked permeable pavement before discharge to sewer. Further on-site mitigation is not possible, due to space constraints’. A reduction in housing units would allow for better surface water mitigation and should be explored. This is also requested by NE who state:

Further consideration should be given to a greater use of SuDS features on site, in particular the incorporation into the layout of a comprehensive system of filter strips, vegetated swales, bioretention areas (rain gardens) and detention basins to facilitate as far as possible discharge to ground and effectively treat the surface water that leaves the site. Such features will require sufficient space within the layout and so designs should be considered at the Outline application stage, so as to ensure the necessary measures can be incorporated into the final development.

This also appears inconsistent with the EcIA which states that permeable routes through the site in the form of ‘swale/ditches, culverts…’ will be provided to ensure toad dispersal. Also, the illustrative layout shows house gardens accounting for over half of the edge location where presumably surface water run off would not be captured by tanked permeable pavements.

SLR state: ‘The nutrient loading from the development will be reduced to 3.69 kg total phosphorus (TP) per year to 2030, and to 2.57 kg TP/year from 2030. This remaining nutrient loading will require off-site mitigation. It is therefore proposed that the developer purchases nutrient credits from a credit provider within the Hampshire Avon catchment’.

Further the NN Strategy states ‘Sewer records have been obtained from Wessex Water and are enclosed in Appendix C. These indicate that a 150mm diameter public foul water sewer is located immediately south of the site. This flows to the south-east. In an email dated the 16th April 2025, Wessex Water confirmed that foul water from this sewer is discharged to Warminster Wastewater Treatment Works (WWTW)’.

The site straddles two River Avon SAC sub-catchments – the northern half lies in the Upper Wylye, whilst the southern half lies in the Upper Wylye Headwaters. Given the discharge to Warminster STW, the required P credits would need to be sourced from the Upper Wylye Headwaters.

The NN Strategy states that ‘It is envisaged that some additional nutrient mitigation will be secured via the purchase of credits, and that this will be secured via a planning condition and a legal agreement with the credit provider’. Any P mitigation scheme must have prior approval from Natural England and should be signed off in order to satisfy the HRA appropriate assessment. Please note that the P mitigation scheme cited in para 5.7 has not been approved by Wiltshire Council and no S106 agreement between this supplier and the Council exists.

The use of tanked permeable pavements to prevent surface water run off and pollution entering the adjacent river systems requires further justification, including details of the locations of tanked permeable pavements and how P mitigation has been calculated, how will these areas be maintained, how will the tanks be emptied and how they will deal with severe storm events predicted to be more frequent with climate change? How will run off from gardens be dealt with? It is proposed to discharge post development surface water runoff to a surface water drainage ditch network to the south of the site, which connects to the River Wylye. We recommended enhanced pollution prevention standards/measures to prevent pollution to the SAC river and the LNR river and a redesign of the scheme to include greater use of swales, ditches and water features.

We also note concerns from Wiltshire Council drainage colleagues citing inconsistencies and insufficient information on the drainage and run off strategy.

We note that the LLFA is currently maintaining a holding objection because there is insufficient information to demonstrate that the proposed surface water strategy will function as intended. Key outstanding matters include the absence of infiltration testing and groundwater monitoring, uncertainty over whether infiltration has been maximised in accordance with the drainage hierarchy, a lack of detailed hydraulic calculations, uncertainty regarding potential catchment transfer, and the absence of an exceedance flow plan showing how runoff would be managed during extreme events.

These drainage uncertainties are directly relevant to the ecological concerns raised above. Further information is required including:

- how runoff will be managed across the site

- where runoff will ultimately discharge

- how the system performs when downstream water levels are elevated

- how exceedance flows are routed during severe storm events, and

- what level of water quality treatment is being provided.

Currently, is difficult to conclude that there would be no adverse effects on downstream receptors, including Smallbrook Meadows LNR and, ultimately, the River Avon SAC.

We also note that the drainage consultee has requested confirmation that surface water disposal via infiltration has been maximised and that detailed evidence is provided to support this conclusion. Given the ecological sensitivity of the receiving environment, this ties in closely with Natural England's comments regarding the need to maximise opportunities for above-ground SuDS and provide robust water quality treatment.

Biodiversity Net Gain (BNG)

The BNG Metric attached is incomplete and inaccurate. In relation to the BNG condition sheets, these lack crucial data on species compositions and ‘notes’ not present. The Unredacted EcIA contents lists BNG as section 4 when it is section 5. The redacted version has the correct section reference but provides no further detail. It provides a baseline score but no other detail. The Mycelia screenshot is not helpful and shows 8 metric errors. There is therefore no indication of how the development will deliver a 10% BNG in line with legislation. Given our concerns about the classification of the grassland types on site, the revised BNG submission should provide sufficient detail to confirm the grassland types, extents and condition status.

A revised metric plus set of appropriately detailed condition sheets is required.

Mitigation

More information is required in relation to the following mitigation elements:

- Phosphate mitigation: evidence that tanked permeable paving is appropriate (see above) and evidence that P credits required are available in the required sub catchment (see above);

- A Construction Environment Management Plan is required prior to planning approval given the sensitivities of expected run off and potential pollution into the River Avon SAC to the south and the LNR river to the north.

- A revised drainage strategy to provide greater assurance of pollution prevention to sensitive river systems and to better enhance biodiversity on site, including toad migration

- A Toad Mitigation Strategy

- A Bat Mitigation Strategy, including protection of sensitive trees, sensitive lighting for 10m buffers

- Modelling of expected greater recreational pressure on LNR to the north and measures to fully mitigate this impact. Modelling of recreational use of retained and enhanced grassland public open space, including acid grassland Priority Habitat, to prevent degradation of habitat and to enhance its biodiversity value. A detailed Habitat Management Plan required to provide assurance and demonstrate that the acid grassland can be protected and managed appropriately and that the other areas of retained neutral grassland will also be protected and enhanced.

- A revised Ecology Parameters Plan to show the appropriate dark buffer zones of at least 10 metres to the adjacent woodland along the north, east and southern edges; and how retained open space grassland and woodland will be protected and enhanced; illustrative design of amphibian corridor;

Council Core Policy 50

All planning applications are currently expected to comply with Core Policy 50 on Biodiversity and Geodiversity. This policy applies in addition to any requirement for biodiversity net gain. You should demonstrate how the ecological function of any important habitat features will be maintained by your proposal. All developments are expected to demonstrate no net loss of biodiversity. Currently, the development does not comply with Policy CP50 as it lacks information on key ecological features, lacks a proper assessment of the value of such features and fails to adequately demonstrate how sites, habitats and features would be protected and enhanced.

DRAFT CONDITIONS:

Note: we currently object to this application: the following conditions are indicative at this stage and provided for information. Identification of such conditions at this stage does not indicate that the Council would approve this scheme subject to such conditions, rather that any approval is likely to include similar conditions.

LIGHTING

Prior to commencement of development, a sensitive bat lighting plan that includes a Lux Plot, shall be submitted to the LPA for approval. The plan will demonstrate that bat habitat (grassland, trees, scrub and hedgerows) on the perimeter of the site and commuting/foraging routes through the site will remain below 0.5 lux. All lighting provided on site shall be in accordance with the appropriate Environmental Zone standards set out by the Institute of Lighting Engineers in their publication GN01:2021, ‘Guidance for the Reduction of Obtrusive Light’ (ILP, 2021), and Guidance note GN08/23 “Bats and artificial lighting at night”, issued by the Bat Conservation Trust and Institution of Lighting Professionals.

REASON: To minimize impacts on bats caused by light spillage to areas through, above and around the buffers of the development site.

NUTRIENT NEUTRALITY AND WATER USE

The Scheme must ensure the following:

- Ensuring that the development limits water use to 85 l ppd.

- That the permission secures SuDS identified and ensures appropriate maintenance in perpetuity.

- That the scheme delivers and thereafter retains the requisite areas of greenspace (1.05 ha) as set out in the phosphorus calculations.

- That the requisite phosphorus credits are secured prior to commencement.

Reason: To comply with nutrient neutrality policy and protect the River Avon SAC.

CEMP

Given the high risk posed from the site to adjacent SAC and LNR habitats, a detailed Construction Environmental Management Plan (CEMP) is required prior to determination. The CEMP will need to address how soil runoff will be prevented throughout the construction phases including during periods of heavy rain. Measures might include minimising the areas of turf stripping as the development progresses and ensuring high risk works are completed during drier periods. In addition, measures for controlling and treating construction run off should also be secured

Reason: To prevent damaging impact to designated sites during construction. Given the potential to impact the SAC (European Site), the CEMP must be provided pre determination to enable an Appropriate Assessment to be completed.

SUSTAINABLE URBAN DRAINAGE

No development shall commence until a detailed Sustainable Urban Drainage Scheme, including maintenance plan has been submitted to and agreed in writing by the Local Planning Authority. The scheme should include greater use of SuDS features on site, in particular the incorporation into the layout of a comprehensive system of filter strips, vegetated swales, bioretention areas (rain gardens) and detention basins to facilitate as far as possible discharge to ground and effectively treat the surface water that leaves the site.  The Sustainable Urban Drainage scheme will accord with CIRIA C808 guidance and demonstrate the proposal will deliver a reduction in surface water phosphorus load to xxx kgP/year and accord with the Nutrient Neutrality and Mitigation Strategy dated xxx and Shadow Habitats Regulation Assessment dated xxx.

Reason: To comply with SuDS guidance, deliver phosphate mitigation and protect the River Avon SAC and adjacent wildlife habitats.

ECOLOGICAL ENHANCEMENTS

Prior to the commencement of any works, including vegetation removal and demolition, details of the number, design and locations of ecological enhancement features including the fencing off and proposed management and maintenance of buffer areas and grassland (including acid grassland), measures for toad commuting and conservation and other required ecological measures shall be submitted to the Local Authority for approval. The approved details shall be implemented for the lifetime of the development and completed before development commences.

REASON: To provide mitigation /enhancement for biodiversity. The ecology works are required ahead of development commencement given the narrow nature of retained grassland areas, their sensitivity and their potential risk to construction works and impact from new residents.

HABITAT MANAGEMENT AND MONITORING PLAN

No development shall commence on site until a Habitat Management and Monitoring Plan (the HMMP), prepared in accordance with the statutory Biodiversity Gain Plan and including:

- a non-technical summary;

- the roles and responsibilities of the people or organisation(s) delivering the HMMP;

- the planned habitat creation and enhancement works to create or improve habitat to achieve the biodiversity net gain in accordance with the statutory Biodiversity Gain Plan and schedule for implementation;

- the management measures to maintain habitat in accordance with the statutory Biodiversity Gain Plan for a period of 30 years from the completion of development; and

- the monitoring methodology and specification of a Monitoring Pack (to include but not exclusively up to date Management Actions Logs, Habitat Condition Assessment Reports, metric calculation; and corresponding post intervention Habitat Map) which shall be submitted to the Local Planning Authority in years 2 (two) 5 (five) 10 (ten) 15 (fifteen) 20 (twenty) and 30 (thirty) of the Maintenance Period,

has been submitted to, and approved in writing by, the local planning authority. The created and/or enhanced habitat shall be managed and maintained in accordance with the statutory HMMP at all times thereafter.

REASON: To ensure the development delivers a biodiversity net gain on site in accordance with Schedule 7A of the Town and Country Planning Act 1990.

LOCAL NATURE RESERVE MITIGATON PLAN

No development shall commence until a scheme to conserve and enhance the adjacent Smallbrook Meadows LNR is submitted to the LPA for approval.

Reason. To ensure that the adjacent LNR is not adversely impacted by the development in terms of (but not limited to) visitor pressure, water and pollution run off, impacts of urban development (rubbish tipping, cat predation, fires, vandalism etc).

Sign off by WC Ecologist will only be given once ALL outstanding information has been submitted and agreed sufficient.

| Ecologist | Date |
| --- | --- |
| Niall Machin |  |

Guidelines for Ecological Impact Assessment in the UK and Ireland, Version 1.3 updated Sept 2024.

Land at Damask Way, Warminster 20/07214/REM, Botanical Inventory of Grassland and Woodland, Sharon Pilkington Dec 2020