PL/2026/03769 LBC and PL/2026/03496 FULL

Site: Reybridge Farm Stables, 32 Reybridge, Lacock, SN15 2PF

Proposal: Change of use and conversion from stables to dwelling house.

________________________________________________________________________

Recommendations:

|  | No comment |
| --- | --- |
| X | Support in principle subject to amendments to the scheme (as set out below) |
|  | Support subject to conditions (as set out below) |
|  | Object (for reasons set out below) |
|  | No objections |

Scope of comments: the following comments relate to the built historic environment.

Policy: From the point of view of the historic environment the main statutory tests are set out within the Planning (Listed Building and Conservation Areas) Act 1990. Sections 16 (LBC) and 66 (PP) require that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

The NPPF outlines government policy, including its policy in respect of the historic environment. Section 16 of the NPPF 'Conserving and enhancing the historic environment' sets out the Government's high-level policies concerning heritage and sustainable development.

Paragraphs 213 - 215 of the NPPF 2024 require a balanced approach with any harm which would be caused being weighed against the potential public benefits which might be achieved.

Paragraph 210 requires local planning authorities should take account of the desirability of sustaining and enhancing the significance of heritage assets and of new development making a positive contribution to local character and distinctiveness.

The Council’s Core Strategy Policy CP58 ‘Ensuring the conservation of the historic environment’ requires that “designated heritage assets and their settings will be conserved, and where appropriate enhanced, in a manner appropriate to their significance.”

The Council’s Core Strategy CP 57: Ensuring high quality design requires a high quality of design in all new developments. Proposals are required to “demonstrate how the proposal will make a positive contribution to the character of Wiltshire through (amongst other things) “enhancing local distinctiveness by responding to the value of the natural and historic environment, relating positively to its landscape setting and the existing pattern of development” and “responding positively to the existing townscape and landscape features in terms of building layouts, built form, height, mass, scale, building line, plot size, elevational design, materials, streetscape and rooflines to effectively integrate the building into its setting.”

Historic England Advice Note 3 – The Setting of Heritage Assets set out advice for managing change within the setting of heritage assets.

Historic England guidance on the Adapting Traditional Farm Buildings.

Heritage Assets:

Reybridge House, Reybridge Lacock grade II

The associated barns are curtilage listed.

31 – 33 Reybridge Reybridge Lacock grade II

Site and assets considered:

Reybridge Farmhouse (formerly New End Farmhouse) is mid-18th century two storey with attic house. It has two parallel ranges with the front (south) elevation in rubble stone with flush quoins and cyma moulded flush mullion windows and stone slate roof and gable stone stacks.

The house is set back 40 m from the lane in the loose hamlet of Reybridge about 750 m north east of Lacock village church.

There is a single storey range of barns framing the access drive to the house orientated north south that are attached at right angles to a further range of single storey stables /barns  located fronting a courtyard about 10 m deep set back from the lane behind a dry stone rubble wall.

This singel storey range of barns is subject of the application and are evident in the 1880 1st Editon OS Mapping but not on the tithe map of 1840.

The range of barns/stables are considered to be curtilage listed. They are mostly open fronted with timber posts on saddle stones fronting the courtyard many are now infilled with block work and stable door openings. The rear and gable elevation is rubble stone walling and the roof is roman clay tiles.

Proposal: Change of use and conversion from stables to dwelling house.

Relevant planning history:

ENQ/2023/1078 for Conversion of the existing equestrian stables to a single storey 3 bedroom dwelling house

The Conservation Comments In this case advised:

The significance of the building lies largely in its historic association with Reybridge House. It has an ancillary function to the house and the historic fabric, architecture, its plan form, layout and architectural expression/detailing are of a functional service building.

The pre application looks to convert and extend and existing stable block dating from the 19th century. The application alludes to certain building fabric of significance including a king post truss roof and ….these relate to the bays created by the timber posts on stone to the south elevation.

This stable building appears to have originally been open sided. The rear (north) elevation is also significant being constructed of local stone rubble. The roof is clad in double roman tiles to the front and stone tiles to the rear. It is proposed to clad the new roof in slate tiles however

I consider that a more appropriate and subservient appearance is gained form the double roman profile tiles to the front elevation, and these would be preferred to all roof slopes.

Historic England guidance on Adapting Traditional Farm Buildings advises as the applicant has suggested that the retention of as much significant historic fabric as possible is fundamental to any good adaption together with compatible material and methods of repair. It also seeks to retain features that give evidence of the former use of the building.

In this case the timber posts on plinths and associated timber king post trusses suggest an original cart shed or open sided building that was later adapted for stables. The posts should be retained and emphasised as significant features by setting back the infill stone walling at least 50 mm and preferably behind/inside the posts to better reveal them.

A further issue is introducing daylight into the block however in this case with the large south openings this will not be considered a problem. The pre app proposal has several rooflights to north roofs slope and to the kitchen /utility return block. These rooflights except maybe 1 or 2 small rooflights to the rear corridor to serve the bedrooms should be removed.

On extensions the guidance suggests extension should work with and enhance the historic plan form of the farmstead and can be used to take uses that require subdivision. In this case while the extension is at the upper extent of what additional floor area would be considered appropriate, they follow the primary form of the stable block in a linear form while introducing a return block to improve separation from the other neighbouring uses and create an enclosed amenity space.

The extensions will need to be justified. The openings currently are not consistent with the subdivision of either the existing stables or more importantly the previous open sided block, timber post subdivision.

The living room, office and bedroom window openings appear excessive and open to privacy issues and light spill and should be amended and be based on the proportions of the stable door openings.

All new windows should be simple, contemporary and timber framed.

The solar panels have public benefits but are a very visible feature on the roof slope and need justification within a heritage statement.

2024/04525 LBC and 2024/04056  FULL Conversion of existing stables into a single storey three-bedroom dwelling house, with extensions to the Eastern and Western ends. Refused.

The reasons for refusal of planning permission were that the development was outside a settlement boundary, and the building was not capable of conversion. The council was not satisfied the design of the proposed extensions were appropriate.

The concurrent Listed Building Consent application was refused on the grounds that the proposed alterations failed to respect the architectural and historic interest of the curtilage listed building.

Issues: The impact of alterations on the curtilage listed barns and setting of neighbouring listed Farmhouse

Significance of assets and contribution of setting: The NPPF defines significance as the “value of a heritage asset to this and future generations because of its heritage interest.

The interest may be architectural, or historic as set out in the Town and Country Planning (Listed Building and Conservation Areas) Act 1990 S 1.

Significance derives not only from a heritage asset’s physical presence, but also from its setting.” Historic England assesses significance in a similar manner, referring to evidential, historic, aesthetic, and communal values of a place.

In this case the significance of the building is provided in the list description stating Reybridge  Farmhouse merits designation on a national level as it is a good example of a mid Georgian stone farmhouse using good quality materials and craftmanship expressing local vernacular traditions. Its plan form has remained mostly intact and includes good quality feature that illustrate the historic development of the building.

Assessment:

The application seeks to convert the range of stable and open sided barn into a 3 bedroom dwelling.

The principle of the conversion is established as referenced in the pre application.

This revised application addresses the concerns /issues raised previously and specifically reference these responses in the Design and Access statement with the application.

The details previously lacking have also now been provided regarding alterations and repairs to existing significant fabric and is now not proposed to extend the property. The features which contribute to the significance of the curtilage listed building will be retained.

The historic king-post roof structures will remain visible and expressed internally and are shown in a detailed section along with new timber posts and saddle stones, new timber stud and rear rubble stone retaining wall and ground levels.

The applicants’ comments in Design and Access Statement

‘To address the Conservation officer’s requirement to further retain and preserve its architecture expression, character and detailing.

Design Amendment 1. To preserve the character and detailing of the existing structure, the existing ridge-line will remain as existing. The proposal will seek to retain the mono- pitched roof line and abutment wall detail to the storage room element at the western end, further preserving its agricultural character. By contrast, the pre app submission which sought permission to stretch the existing dual pitch roof to meet the western boundary may have risked the proposal becoming more domestic in its elevated form and character.

Design Amendment 2 As advised by the conservation officer, we have amended the design to include double Roman profiled tiles to both the front and rear elevations of the stable block and to retain and reuse the existing tiles where possible.

Design Amendment 3 As was the intention in the pre app design, the structural timber posts and concrete staddles to the front elevation will be retained and made good where structurally sound and replaced with ‘like for like’ where their structural integrity has been compromised. The proposed timber clad wall will be set back at least 50mm from the rear face of the timber posts to allow greater expression of their functionality and historic importance. This also serves to create a deeper soffit at eaves level which improves solar shading during the summer months, reducing unwanted heat gains.

Design Amendment 4 The Structural Engineer’s report has highlighted an uneven weight distribution of the roof loading, causing deflection to the roof timbers created by the heavier profiled clay tile located on the southern pitch. To address this issue, the slate tiles on the northern pitch will be replaced with Double Roman style tiles to match the weight and aesthetic of the slate found on the southern pitch which will help to protect the structure from further bowing and stress. To compensate for the additional weight strengthening of the truss members will be required. As mentioned in the accompanying Structural engineers report.

Design Amendment 5 We have reduced the number of roof lights required to that which is considered essential for light and ventilation.

Design Amendment 6 All windows and doors have been amended to be flush casement painted timber style fenestration. The patio doors have been reduced in width, and all windows to the front elevation have been repositioned centrally between the structural timber posts with all openings replicating the proportions of the stable doors.

Design Amendment 7 The solar panels have been reduced to 6 and designed to be recessed into the tile zone giving them a flush appearance, dressed in an all black, non-reflective matt finish to ensure they do not have an adverse impact of the surrounding environment and views over the valley are preserved. The panels in their utilitarian form, will also seek to strengthen the historic ancillary and subservient narrative of the barn while embracing 21st century green and renewable energy.

A heritage statement has been prepared assessing the significance of the existing fabric and evaluating the feasibility of the design proposal.’

The one issue that remains is the approach to damp penetration from the raised ground levels to the rear (north) elevation into the rubble stone wall. The rear ground levels are up to 1m below the roof eaves so that the north stone wall is part retaining.

The applicant has proposed a timber stud frame with 12.5 mm plasterboard and skim and painted finish over a tanking membrane to the stone wall. At the base of the wall the internal tanking and drainage channel is proposed.

I consider this detail and general response to this issue is not sympathetic to traditional solid stone wall construction and the benefits of lime mortar to allow the stone wall to breathe along with an internal lime plaster with lime wash finish. This would allow the water penetration in the wall to naturally vary to conditions both external water pressure and internal heating levels and allow as required any damp to move through the fabric.

Conservation Balancing

Whilst NPPF paragraph 215 allows that a level of harm may be offset by public benefits, which could include conservation benefits. In this case the design approach to conversion does not appear to be sufficiently conservation led and should look to follow the Historic England guidance Adapting Traditional Farm Buildings to assist in justification of the alterations and remove those alterations identified as being harmful. These proposals are limited to private benefits and do not currently outweigh the harm.

The principle of conversion of the existing barns has been established in earlier pre application, and the revised proposal now responds well to previous concerns and Historic England good conservation practice on Adapting Traditional Farm Buildings and to the applicants Structural Engineers Report.

The south elevation now better reveals the repaired timber posts on straddle stones and truss construction which is significant and better reflect the former open character of the stable barns by setting back infill panels in lighter material and has the Structural Engineer advice on roof structure and cladding issues.

The roof cladding is to remain a Roman clay tile as exits to the front (south) slope and the solar panels reduced to no more than 6 and justified.

The redesign for the proposed conversion includes improved drawing details, with detailed cross sections to show ground floor levels, structure including posts, trusses, insulation and stone wall.

However, the rear, part retaining wall should be left exposed and breathable and details need to be revised to remove the tanking system for a sympathetic and traditional solid stone wall construction using the benefits of lime mortar to allow the stone wall to breathe along with an internal lime plaster with lime wash finish. This would allow the water penetration in the wall to naturally vary to conditions both external water pressure and internal heating levels and allow as required any damp to move through the fabric.

With this revision to the section detail, I can support the application as it will be in accordance with the Historic England guidance and local and national policies.