APPLICATION FOR LISTED BUILDING CONSENT

OFFICER DELGATED REPORT

| APPLICATION REFERENCE: | PL/2026/00823 |
| --- | --- |
| APPLICATION TYPE: | Listed building consent (Alt/Ext) |
| SITE ADDRESS: | 113 High Street, Burbage, Marlborough, SN8 3AA |
| DESCRIPTION OF WORKS: | Demolition of existing lean-to utility room and replacement with new side extension to create kitchen, with associated internal alterations. |
| RECOMMENDATION: | Approve with Conditions |

### SITE DESCRIPTION/CONTEXT

The application relates to 113 High Street in Burbage, Marlborough. It is a Grade II listed building (List Entry Number: 1035916) located within the Burbage Conservation Area. The property forms one half of a pair of cottages and dates from the early eighteenth century. The principal structure comprises Flemish bond brickwork beneath a thatched roof and retains a traditional vernacular appearance which contributes positively to both its heritage significance and the character and appearance of the Conservation Area.

The submitted Heritage Statement explains that the building comprises solid brick walls beneath a thatched roof with a catslide roof form to the rear. Internally, the building incorporates a timber frame with cob infill panels. A timber lean-to utility extension has been added against the northern gable in more recent years and is described as being of rudimentary construction with limited architectural or historic value.

The list entry, for means of identification purposes only, provides the following description:

### BURBAGE HIGH STREET SU 26 SW (east side) 2/28 No 113 (Lothlorien) and 115 (Manor Cottage) GV II

### Two cottages in one block. Early C18. Flemish bond brickwork with thatched roof. Single storey and attic, each cottage of 2 bays with gable stacks and stack on party wall. No 113 has boarded "and thatched porch, and 4-panelled door. No 115 has boarded door in open gabled porch. Inner bay of each has small gable with attic window, and faced with mathematical tiling. Leaded windows to 113, paned windows to 115. Flush dormers to outer bays. Interior not seen, may be of interest.

### Listing NGR: SU2302061477

### PROPOSAL

### Listed Building Consent is sought for:

- Demolition of the existing lean-to utility room attached to the northern side elevation.

- Construction of a replacement side extension to provide a kitchen and cloakroom.

- Associated internal alterations including the formation of a new doorway opening within the existing building and infilling of an existing opening.

### The replacement extension would utilise a traditional design approach and matching materials, including brick walling, painted timber joinery and a slate roof.

### RELEVANT PLANNING HISTORY

E/10/0223/FUL - Change of use of former telephone exchange to ancillary residential accommodation. Approved with Conditions 12th April 2010.

13/04535/LBC - Replacement of rotting windows with new hardwood painted windows. Approved with conditions 28th November 2013.

RELEVANT LEGISLATION, POLICIES AND GUIDANCE

Legislation

From the point of view of the historic environment the main statutory tests are set out within the Planning (Listed Building and Conservation Areas) Act 1990.  Section 16 requires that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

Section 72(1) of the Planning (Listed Building and Conservation Areas) Act 1990 also requires the Council to pay special attention to the desirability of preserving or enhancing the character or appearance of designated Conservation Areas.

National Policy and Guidance

The National Planning Policy Framework (NPPF) sets out government policy, including in respect of the historic environment.

Chapter 16 of the National Planning Policy Framework sets out national policy for conserving and enhancing the historic environment. It requires decision-makers to take account of the desirability of sustaining and enhancing the significance of heritage assets and to give great weight to the conservation of designated heritage assets. Where harm is identified, that harm must be clearly justified and weighed in accordance with the relevant provisions of Chapter 16.

National Planning Practice Guidance provides guidance on interpreting the NPPF.

Historic England Advice Note 2: Making changes to Heritage Assets, illustrates the application of policies set out in the NPPF in determining applications for LBC (and planning permission).

Historic England’s publication ‘Traditional Windows – Their Care, Repair and Upgrading (2017)’ encourages the retention of windows that contribute to the significance of listed buildings but sets also out their approach to the alteration/replacement of windows where this is acceptable in principle.

Historic England Advice Note 18: Adapting Historic Buildings for Energy and Carbon Efficiency, provides advice in support of consistent decision making on approaches to improve the energy efficiency and support carbon reduction of historic buildings, whilst conserving their significance.

Wiltshire Core Strategy:

Whilst not directly relevant to listed building consent applications, the below policies give an indication of the Council’s approach to development affecting heritage assets:

- Core  Policy 57 ‘Ensuring high quality design and place shaping’: A high standard of design is required in all new developments, including extensions, alterations, and changes of use of existing buildings. Development is expected to create a strong sense of place through drawing on the local context and being complementary to the locality.

- Core Policy 58 ‘Ensuring the conservation of the historic environment’ requires that “designated heritage assets and their settings will be conserved, and where appropriate enhanced, in a manner appropriate to their significance.”

ISSUES

Impact upon the preservation of the listed building or its setting or any features of special architectural and/or historic interest.

CONSULTATION RESPONSES

Burbage Parish Council: No response received at the time of writing.

WC Conservation: No objection subject to conditions.

REPRESENTATIONS

In accordance with the requirements of Section 5(1) of the Planning (Listed Buildings and Conservation Areas) Regulations 1990 (as amended) the application for listed building consent was publicised by way of:

- A site notice displayed near the site;

- An advertisement placed in the local press.

These methods were used to inform interested parties and to allow a statutory consultation period of 21 days for representations to be submitted.

Below is a summary of the representations received during the consultation period. In accordance with the provisions of the Planning (Listed Buildings and Conservation Areas) Act 1990, only matters relevant to the consideration of the building’s special architectural or historic interest have been taken into account in the assessment of this Listed Building Consent application.

- No comments were received.

ASSESSMENT

In accordance with Section 16(2) of the Planning (Listed Buildings and Conservation Areas) Act 1990, special regard has been given to the significance of the listed building, including its architectural and historic interest, and the desirability of preserving both the building and its setting. Relevant policies within the National Planning Policy Framework (NPPF), particularly Section 16: Conserving and Enhancing the Historic Environment, have also been taken into account. These policies require that any harm to the significance of a designated heritage asset must be clearly justified and weighed against the public benefits of the proposal.

In order to be acceptable, the application needs to be assessed against the following criteria:

Information considered/submitted

Paragraph 207 requires that applicants describe the significance of any heritage assets affected, including any contribution made by their setting. This should include sufficient information to provide a clear understanding of the significance of the heritage asset and its setting and the potential impact of any proposals on that significance.

The application has been supported by a Design & Access Statement and Heritage Asset Statement prepared by A.L Associates, together with existing and proposed drawings. The submitted documents identify the significance of the listed building, describe the evolution of the site and assess the likely impacts of the proposed works. Additional information was also provided during the determination process in response to concerns raised regarding the internal alterations.

The Local Planning Authority is satisfied that sufficient information has been submitted to understand both the significance of the heritage asset and the likely impact of the proposals upon that significance.

Assessment of significance:

According to the National Planning Policy Framework (NPPF), significance is defined as:

“The value of a heritage asset to this and future generations because of its heritage interest. The interest may be archaeological, architectural, artistic or historic. Significance derives not only from a heritage asset’s physical presence, but also from its setting.”

Historic England further categorises significance into four key values:

- Evidential – the potential of the asset to yield evidence about past human activity.

- Historic – the ways in which the asset illustrates aspects of history.

- Aesthetic – the sensory and design qualities of the asset.

- Communal – the meanings and associations attached to the asset by people.

The significance of 113 High Street derives principally from its vernacular architectural form, traditional materials and surviving historic fabric. The building is an early eighteenth-century cottage constructed in Flemish bond brickwork beneath a thatched roof and forms part of a visually attractive pair of listed cottages which make a positive contribution to the Burbage Conservation Area.

The asset derives particular significance from its historic form, traditional construction, thatched roof, brickwork, plan form and surviving fabric. The Heritage Statement identifies both historic and aesthetic value arising from the age of the building, its local vernacular character and its contribution to the High Street.

The modern lean-to utility extension attached to the northern elevation is of considerably lesser significance. The submitted Heritage Statement describes it as a rudimentary timber structure of limited architectural and historic value and the Conservation Officer similarly considers it to be of little significance.

Impact of proposals on the significance and setting of the building/structure:

Understanding the impact of proposed works on a listed building requires a careful and proportionate assessment of how those changes affect the asset’s heritage significance and its setting. This includes both the physical alterations to the building itself and any changes to its spatial or visual relationship with surrounding structures or spaces.

The significance of a heritage asset is not static; it is shaped by its architectural, historic, and communal values, and by how these are experienced in context. Therefore, any intervention must be considered in light of the asset’s identified significance, with attention given to both potential harm and benefits. Benefits may include repairs that support the long-term conservation of the building, improved energy performance, or enhanced public access and understanding.

This assessment is grounded in the statutory duty under Section 16 of the Planning (Listed Buildings and Conservation Areas) Act 1990, which requires special regard to be given to the desirability of preserving the building and its features of special interest. It is also informed by national policy, and the relevant provisions of Chapter 16 of the National Planning Policy Framework (NPPF), and relevant local heritage policies and guidance.

The existing lean-to extension is of limited significance and there is no objection in principle to its removal. Its replacement with a more traditionally designed extension represents an improvement over the existing structure and follows amendments undertaken following pre-application discussions.

The Conservation Officer acknowledges that the proposed rear catslide arrangement is somewhat constrained by the existing building form but considers the solution to be functional and an appropriate response to the practical limitations associated with extending a historic thatched cottage. Whilst modern in character, the extension remains clearly legible as a later addition and would not result in physical harm to historic fabric. Consistent materials and detailing would assist in integrating the extension with the host building and limiting any visual impact upon its vernacular character.

With regard to the internal alterations, concerns were initially raised regarding the absence of sufficient supporting information. Additional information has subsequently been provided, including details of the affected wall, its previous alteration, later gypsum plaster and plasterboard finishes, the limited width of the proposed opening, and the proposed means of maintaining support to the upper masonry and larch pole purlin through the installation of lintels. The proposed doorway would involve localised intervention into existing fabric but having considered the submitted information and the Conservation Officer’s advice, the Local Planning Authority is satisfied that the works would not result in harm to the significance of the listed building, subject to a condition requiring approval of the internal doorway elevation, section, lintel, door and method details before those works are carried out.

The Conservation Officer concludes that the proposal would not cause harm to the significance of the listed building. The works would improve the functionality of the internal accommodation whilst preserving the building's special interest. A condition requiring submission of an internal elevation of the new doorway is considered necessary to safeguard historic fabric.

The Conservation Officer has also requested the inclusion of conditions relating to external materials, brick bond, joinery details, vents/flues and rainwater goods. These details are necessary to ensure that the final form and detailing of the works preserve the special architectural and historic interest of the listed building and can be satisfactorily secured by condition.

Overall, the proposed works would preserve the special architectural and historic interest of the Grade II listed building. The removal of the existing lean-to utility structure is acceptable given its limited significance, the replacement extension would remain subservient and legible as a later addition, and the internal alterations would involve only limited intervention into fabric that has already been altered. Subject to the recommended conditions, the proposal would satisfy the statutory duty under section 16(2) of the Planning (Listed Buildings and Conservation Areas) Act 1990 and the relevant provisions of Chapter 16 of the NPPF.

CONCLUSION

The NPPF confirms that great weight should be given to the conservation of designated heritage assets and that any harm requires clear and convincing justification. In this case, the proposals would not result in harm to the significance of the listed building or its setting. The replacement extension would replace a structure of limited significance with a more sympathetic addition, whilst the internal alterations would improve the functionality and long-term usability of the dwelling without adversely affecting its special architectural or historic interest.

Having had special regard to the desirability of preserving the listed building, its setting and those features of special architectural and historic interest which it possesses, the proposal is considered to satisfy the requirements of Section 16 of the Planning (Listed Buildings and Conservation Areas) Act 1990. The proposal also accords with the relevant provisions of Chapter 16 of the National Planning Policy Framework.

Listed Building Consent is therefore recommended for approval subject to conditions including the statutory time limit; compliance with the approved plans; details of external materials; brick bond to match; detailed internal elevation, section, lintel, door and method details for the proposed new doorway opening; joinery details; vents/flues; and rainwater goods.

RECOMMENDATION: That Listed Building Consent be GRANTED, subject to conditions.