MEMORANDUM

To: Development Management                  From:  Highways Development Management

Ref: 2025/09780                                                  Ref: PL/2025/09780

Date: 7th August 2026

_________________________________________________________________________

25/09780 Outline application for the construction of 295 homes, allotments, sustainable drainage systems, 100 place nursery and access points for pedestrians and cyclists. All matters reserved except access, the principal point of vehicular access is to be provided from a new northern arm onto an improved arrangement of the Eastern Way / A3102 roundabout, with an emergency access elsewhere onto the A3102

I provide highway comments on the above application as follows, and apologise for the delay in providing this response.

The development site is adjacent to the existing built-up area of Melksham town, and has a frontage to the A3102 class I road Sandridge Common. Halfway along the frontage there is the existing A3102 / Eastern Way  roundabout from which vehicular access is proposed. The minor road New Farm Road runs along the eastern site boundary. No vehicular access is proposed from New Farm Road.

These comments are provided bearing in mind the concurrent application for Full planning permission reference 24/10345 which proposes full planning permission for a similar development. Reference should be made to the highway policy and general transport comments provided on that application. But for clarity I confirm that the development  will have no severe and residual cumulative impacts on the highway network such that a refusal of planning permission could be justified under the terms of the NPPF.

There is therefore no highway objection in principle to this outline application.

Detailed planning permission is sought for access. The provided access details are satisfactory to provide, good, safe and convenient access arrangements to the development, subject to account being taken of my comments below.

It must be noted that the roundabout redesign has not been fully technically approved. Under the full technical approval process at S278 stage some further minor adjustments to the roundabout design may be required. Such adjustments will be minor in scope.

A satisfactory emergency access point which will double as a pedestrian and cycle access is detailed on the site frontage west of the access roundabout.

Formal pedestrian crossings are indicated west and east of the roundabout.

My expectation is that the construction vehicle access will be from the roundabout, although this can be discussed in more detail at a later stage.

The Masterplan and a plan at around page 44 of the TA indicates pedestrian and cycle access to the western boundary. While this would be very welcome it is unachievable as the land beyond the site boundary is not highway and is not in the control of the applicants.

I have no highway objection to this application subject to the following obligations and conditions.

Obligations

Public transport contribution of £375,000

Works in connection with footpath MELK103 £30,000.

Travel Plan £300 green travel voucher per dwelling

£7500 Travel Plan monitoring

Improving accessibility at train station £5042

Adding development details to local cycle maps £1000

Access roundabout alterations and the access, including any necessary resurfacing, alterations to highway drainage and to street lighting at the roundabout and also at the crossing locations.

Footway alterations near the access roundabout and on the site frontage.

Adoption of a main spine road through the estate to the northern site boundary.

Conditions

Implementation of the submitted Travel Plan.

Construction Management Plan.

Provision of the emergency access with its visibility.

Provision of the toucan crossing to the west and its visibility splays.

Provision of the puffin crossing to the east and its visibility splays.

Should you need detailed wording for the above conditions let me know.

Mark Wiltshire

Principal Transport and Development Manager

Highways Development Management

mark.wiltshire@wiltshire.gov.uk