# Heritage Statement

Greystones, Becketts Lane, Chilmark, Wiltshire SP3 5BD

Listing Information

Detached cottage. Late C17. Course dressed limestone, thatched roof, gable end ashlar and brick stacks. Gable end to road. Two-storey, 2-window front.

Central C20 glazed door, 3-light recessed chamfered mullioned window to either side. First floor has three 2-light mullioned windows. Right return is gable to road; 2 windows. Rear has 2 light mullioned windows to ground and first floors, late C19 two storey extension to right has 4-pane sashes and tiled roof.

Listing: NGR ST9691032613

Date of listing: 27-Jul-1985

Grade II

Site Background

Greystones is a mid 17th century cottage which has been substantially extended in several phases during the 1980’s and 90’s. The existing extensions to the original stone cottage wrap around the rear of the building and comprise of 2 storey extension with boot room, stairs to upper floors and kitchen on the ground floor and 1 x bedroom and landing on the upper floor. An additional single storey extension was added to the rear of the kitchen. The accommodation has further been extended with a loft conversion providing a further bedroom and shower room and externally a detached garage was added in 2002.