PL/2026/02097 Planning and PL/2026/02517/LBC

Site: The Holford Arms, Knockdown, Tetbury, GL8 8QY

Proposal: Redevelopment of ancillary barn and function building, together with works to the public house, to provide spa and gym facilities in association with the pub's enhanced attraction.

Conservation Officer: Alex Marsh

________________________________________________________________________

Recommendations:

| X | Comment |
| --- | --- |
|  | Support in principle subject to amendments to the scheme (as set out below) |
|  | Support subject to conditions (as set out below) |
| X | Object (for reasons set out below) |
|  | No objections |

Scope of comments: the following comments relate to the built historic environment.

Site Description:

The Holford Arms formerly the New Inn is a roadside Inn dating from the 17th century with 18th century and later alterations. It is two storeys with colour washed render elevations and a hipped stone slate roof.

There is a prominent stair tower projection to the front elevation that also has a 3 light casement fenestration to the ground floor with 16 pane hornless sashes to first floor and horned version to the stair tower.

The rear south elevation has two lateral stacks. The interior has interest with beams and inglenook fireplace in the ground floor public house single room plan.

The Public House is located to the south of the A433. Set back about 15 m with open parking to the front and garden. The frontage had established trees to the east side behind railing and dry-stone walling.

To the west is a single storey 18th or early 19th century former stable block, subject of the application and to the south a defined curtilage with dry stone walling and small stone outbuilding and pigsty.

Heritage Assets:

The Holford Arms, Knockdown. Tetbury, grade II.

Relevant planning History:

19/3888/FUL & 19/4528/LBC Works to curtilage listed building replacement of temporary outside bar with permanent outside bar election of demountable canopy over beer garden (retrospective)

19/07189/FUL & 19/07331/LBC Conversion of skittle alley to form staff accommodation and ancillary storage space to the public house (retrospective)

19/08744 & 19/09298 To Regularise Deviations from Works to Convert the Upper Floors to Guest Accommodation and Consent for a Replacement Porch

PL/2024/08465 Proposal: Variation of condition 6 of 19/03888/FUL - To enable retention of existing beer garden canopy for an additional temporary period of 3no. years.

The Proposal: Redevelopment of ancillary barn and function building, together with works to the public house, to provide spa and gym facilities in association with the pub's enhanced attraction.

National Policy and guidance: From the point of view of the historic environment the main statutory tests are set out within the Planning (Listed Building and Conservation Areas) Act 1990. Sections 16 (LBC) and 66 (PP) require that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

Section 72(1) of the Planning (Listed Building and Conservation Areas) Act 1990 also requires the Council to pay special attention to the desirability of preserving or enhancing the character or appearance of designated Conservation Areas.

The NPPF (August 2026) sets out Government planning policy for England. Chapter 20, Conserving and enhancing the historic environment, recognises heritage assets as an irreplaceable resource. Heritage assets should be conserved in a manner appropriate to their significance, with harm avoided unless there is a clear and convincing justification. Decision-makers are required to apply a balanced approach, giving substantial weight to the conservation of heritage assets and weighing any harm to significance against the public benefits of the proposal.

Chapter 5 of the NPPF sets out policies for “Meeting the challenge of climate change” and gives substantial weight to the benefits of improving the energy efficiency of existing buildings and/or drawing energy from district heat networks, renewable and low carbon sources (including through the installation of heat pumps and solar panels where these do not already benefit from permitted development rights).

National Planning Practice Guidance provides guidance on interpreting the NPPF.

Historic England’s Conservation Principles, Policies and Guidance (2008) provides policy and guidance for the “sustainable management of the historic environment”. Additional guidance on the application of policies set out in the NPPF is contained within their publications:

- Historic Environment Good Practice Planning Advice Note 2 – Managing Significance in Decision-Taking in the Historic Environment (2015)

- Historic Environment Good Practice Planning Advice Note 3 – The Setting of Heritage Assets 2nd edition (2017)

- Historic England Advice Note 2 – Making changes to Heritage Assets (2025)

- Historic England Advice Note 18 – Adapting Historic Buildings for Energy and Carbon Efficiency (2026)

Local Policy: The Council’s Core Strategy Policy CP58 ‘Ensuring the conservation of the historic environment’ requires that “designated heritage assets and their settings will be conserved, and where appropriate enhanced, in a manner appropriate to their significance.” It is also required that distinctive elements of Wiltshire’s historic environment, including non-designated heritage assets, which contribute to a sense of local character and identity will be conserved, and where possible enhanced.

The Council’s Core Strategy CP 57: Ensuring high quality design requires a high quality of design in all new developments. Proposals are required to “demonstrate how the proposal will make a positive contribution to the character of Wiltshire through (amongst other things) “enhancing local distinctiveness by responding to the value of the natural and historic environment, relating positively to its landscape setting and the existing pattern of development” and “responding positively to the existing townscape and landscape features in terms of building layouts, built form, height, mass, scale, building line, plot size, elevational design, materials, streetscape and rooflines to effectively integrate the building into its setting.”

Historic England Guidance on Adapting Traditional Farm Buildings

Historic England website The Public Houses of England

England Conservation Principles Policies and Guidance 2008

Issues: Impact on listed building and curtilage listed building and setting of the listed buildings, and design and materials.

Significance of assets and contribution of setting: The NPPF defines significance as the “value of a heritage asset to this and future generations because of its heritage interest.

The interest may be architectural, or historic as set out in the Town and Country Planning (Listed Building and Conservation Areas) Act 1990 S 1.

The NPPF defines significance as the “value of a heritage asset to this and future generations because of its heritage interest. The interest may be, architectural, or historic. Significance derives not only from a heritage asset’s physical presence, but also from its setting.” Historic England assesses significance in a similar manner, referring to evidential, historic, aesthetic, and communal values of a place.

Holford Arms significance comes from its vernacular architecture, building form, layout and architectural expression/detailing and use of high-quality design materials which have significant aesthetic and architectural interest.

The former Stable block is a linear building with a lean-to addition to the west end. It is evident on 1840 Tithe mapping and 1880 OS mapping It is built in rubble stone with tiles roof cladding. Internally it is subdivided by rubble stone walls into three rooms relating to the openings. The east end has two bays a small central single bay and room and three bays to west end. The majority of the roof trusses and purlins are original, and the building is considered a curtilage listed buildings along with the dry-stone walls and rear garden/yard small outbuilding (s).

Assessment:

This application has been amended in a positive manner taking on the Conservation Officer comments from the original submission. The site has complex planning history including significant unauthorised works combined with some temporary structures and proposed works to the listed building.

This background has been mostly responded to in these issues through the amendments and updated landscape proposal plans.

The reinstatement of the rear garden and front garden drystone boundary walls and metal railings and part demolition of the lean-to that have been removed are reinstated in the landscape plan albeit altered.

These walls need to be reinstated and will need to be managed through a condition to ensure reinstatement within 6 months of consent. An enforcement case will need to be opened and maintained until the walls are reinstated on site. The walls and railings are significant features and define the curtilage of the Public House.

In a similar way to the previous application for 19/4528/LBC Works to curtilage listed building replacement of temporary outside bar with permanent outside bar election of demountable canopy over beer garden (retrospective) this was subject to a pre-recommencement condition No. 4.

It is noted that the works to take down part of the stable block have been undertaken without discharge of this condition and are therefore unlawful. The revise plan shows the lean-to rebuilt as previously consented with a pitch roof.

In addition, under PL/2024/08465 a Variation of condition 6 of 19/03888/FUL - The event structure/outside beer garden cover will be removed and the land restored to its former condition by 15/12/2026.

The associated Beer Garden Bar against the back wall of the barn has already been removed.  These removals of temporary structures will benefit the setting of the listed building.

The current revised proposals are for:

Alteration to Public House - Replacement Windows.

Most of the windows in the property are either original or traditional in details and style and are of significance. The exceptions being WF 01 sprung load sash WF03 to the rear east bedroom. And WG 06, 07, 08, 10 and WG 11 to the rear single storey extensions.

The application had included the replacement of existing significant windows in good condition

However, this proposal has been removed, and the existing windows are to be repaired and repainted.

Alteration to Public House - External Elevations

The proposals to the side garden and rear courtyard elevations have been altered had included the timber framed pergola structures to the side garden (east) have been altered to freestanding structures in the garden adjacent to the rear single storey skittle alley and the timber framed covered walkway to the rear wing (kitchens and accommodation in former skittle alley) have been removed.

The corresponding modern extension to the former skittle alley is currently rendered and is proposed to be clad in timber and new windows to the rear addition to the Skittle Alley, now used as a staff accommodation. Subject to materials these proposals are acceptable.

Porch

The existing porch to the front of the Public House is a discrete and appropriately scale and designed small hood with possibly later single post and timber boarded panel. The front door is plain boarded in a large opening, that has been reduced in size with a side panel in timber boarding.

The proposal to replace the Hood porch with a large lean-to porch with slate roof on oak timber post and bracing on saddle stone plinths has been removed from the application.

The existing hood is a significant feature on the front of the listed Public House and is to be retained and repaired.

The front door is to be replaced with a single boarded door to fill the existing wide opening this would respond to the original dimensions of the opening and is subject to details is likely to be acceptable.

The Stable Block

The former Stable block is a linear building with a lean-to addition to the west end. It is evident on 1840 Tithe mapping and 1880 OS mapping. It is built in rubble stone with tiles roof cladding. Internally it is subdivided by rubble stone walls into three rooms relating to the openings. The east end has two bays a small central single bay and room and three bays to west end. It is considered curtilage listed.

The Stable Block conversion is based on a previous approval not implemented under a pre commencement condition.

I maintain an objection to the proposed alterations to the barn that were previously based on the planning officer balancing the justification based on the contribution it would have to the Public House continued business during the exceptional Covid pandemic period.

I do not consider the appropriate weight was given to the harm the fabric of the building as against the benefits form the alterations to provide for a new extended use of the Public House into the Stable block.

While, a well-considered conversion of the Stable block based on Historic England guidance on Adapting Historic Farm Buildings would be supported this would require a much less invasive and harmful approach retaining the partitions walls and features, evidence in drawings including sections that the principal trusses and purlins of the roof structure are retained.

The proposed lean-to, to the rear courtyard is a finely balanced decision but with appropriate detail of how it is to be integrated into the existing structure this may be acceptable.

The new Spa use can be achieved within a revised scheme preserving the significance of the plan form and fabric of this curtilage listed building to the benefit of both the building and setting of the Public House and the new Spa use.

The Stable barn extension previously proposed has been removed from the application however this does not resolve the above concerns about the proposed conversion.

As the Spa use is core of the proposal, we need to revise the proposal along the lines of above to mitigate the identified harm can be resolved. This can either be achieved either through this application or refusal and resubmission.

Camping Barn Events Building

It is my understanding that the Camping Functions Building was built specifically and intrinsically for the neighbouring Camping use. It is outside the site and proposed to be transferred to an extension of the curtilage of the Public House site.

The proposal includes a large extension to the Functions building to provide an additional 60 sq m floor area. This would extend the existing building to the west and north adjacent to the listed building stone wall. It also includes extending the curtilage and land use of the Public House into the adjoining Camping Field.

At this stage, no information is provided as to the Spa proposal and why it is necessary to incorporate the Camping Area Function room.

The applicant has provided a Landscape plan to show the integration of the Function building with the reinstatement of the drystone wall with a wider opening connection between the rear garden /yard of the Public house and the Camping Functions Barn to improve connection to the Barn Spa use.

The impact on the appearance of the curtilage of the Public House with the drystone wall reinstated is partly preserved. However, the drystone wall needs to be reinstated with only the narrower pedestrian entrance between the Public House curtilage and the neighbouring Camping field. This would preserve the original physical distinction between the two areas should the curtilage extension be considered acceptable.

A further issue that needs addressing as part of this application as the Spa use will include part of the Camping Field and in particular the Camping Functions Barn is the knock-on effect to the viability of the camping area and the need for a replacement Functions building. These elements need to be properly addressed in this application.

The existing Functions building is large and has a harmful impact on the setting of the listed Public House and associated buildings with the accommodation building to the east side of the Functions room. The application does not provide evidence for the need to further extend the size of the Functions room as part of this application to incorporate it into the Spa use. The extension is not supported as this would add additional harm to setting of the listed asset that is not justified.

Heritage Conclusion and Balance:

The NPPF confirms that when considering the impact of a proposed development on the significance of a designated heritage asset, great weight should be given to the asset’s conservation (and the more important the asset, the greater the weight should be). It makes clear that any harm to a designated heritage asset will require a clear and convincing justification.

The proposals would have benefited from a pre application with local planning authority as the issues it raises are now being negotiated during the application.

The Holford Arms has undergone significant authorised temporary construction related to supporting the Public House during and immediately after the Covid Pandemic, these temporary buildings have either been removed or will be removed by the end of this year.

There was also a period of some poor decisions some without Conservation advice and inappropriately combined with proposals for regularizing works retrospectively.

The amended proposals have positively adopted many of the previous Conservation Officer comments and objections.

However, while these alterations are beneficial the application still needs to address:

- The amendment of the proposal for the conversion of the curtilage listed barn so its responds to Historic England guidance Adapting Historic Farm Buildings. It also needs improved drawing and section details to show the preservation of the existing roof structure. As proposed the alterations internally to the Barn are harmful and without justification.

- The extension of curtilage of the Public House with reinstatement of the Drystone wall. The wall needs to be reinstated to its former appearance with full detail provided on its construction including the width of the opening to the wall. The reinstatement needs to be controlled by a time limited condition for it implementation should all other issues be resolved.

- The proposal to incorporate the function building into an extension of the historic curtilage of the Public House is assisted by the reinstatement of the drystone wall (as above) as this reinstates the physical/visual separation. However, there is no justification for further extension to the large Function room which is already harmful to the setting of the listed Holford Arms and curtilage barn. Removing the proposed extension could mitigate the additional harm along with the reinstated drystone boundary wall.

- This application for the Public House and Spa use include part of the Camping Field, and particularly the loss of the existing Camping Functions Barn to the camp site area. There is no information on how the effect on the viability of the camping area and the need for a replacement Functions building. These elements need to be properly addressed in this application.

Subject to these issues being addressed satisfactorily the conservation balance should be sufficient to support the application.