APPLICATION FOR PRIOR NOTIFICATION – PART 3

CASE OFFICER REPORT

| APPLICATION REFERENCE: | PL/2026/02953 |
| --- | --- |
| APPLICATION TYPE: | Prior approval Part 3 Class R: Agricultural buildings to a flexible commercial use |
| SITE ADDRESS: | Dutch Barn To The West of Knighton House, Ramsbury, Marlborough, Wiltshire |
| DEVELOPMENT DESCRIPTION: | Change of use from Agriculture to Flexible Commercial Use falling within Class E(g)(i), pursuant to Schedule 2, Part 3, Class R of the Town and Country Planning (General Permitted Development) (England) Order 2015. |
| RECOMMENDATION: | That Prior Approval is REQUIRED and GRANTED |

## SITE DESCRIPTION/CONTEXT

The application site comprises a Dutch barn located to the west of Knighton House, Ramsbury, within a rural agricultural setting. The building is associated with the wider agricultural unit and is served by an existing independent vehicular access. The site lies within the North Wessex Downs National Landscape.

The building is a steel-framed Dutch barn with a corrugated metal roof. Previous permissions have authorised the enclosure of the side and rear elevations and subsequent external alterations. A site inspection confirmed that these works have been partially implemented, with cladding installed to the side and rear elevations and openings formed within the structure. The building remains substantially open fronted and retains the appearance and character of a Dutch barn.

Knighton House, located to the east of the site, is a Grade II listed building. The application building itself is not listed.

PROPOSAL

This application seeks prior approval under Schedule 2, Part 3, Class R of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended) for the change of use of an existing agricultural building to a flexible commercial use falling within Class E(g)(i) of the Town and Country Planning (Use Classes) Order 1987 (as amended).

The application relates to a Dutch barn with a gross floor area of approximately 218.9 square metres together with associated land within its curtilage. The proposal relates solely to the change of use of the building and associated curtilage.

RELEVANT LEGISLATION, POLICY AND GUIDANCE

This application is assessed under Article 3 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), which grants planning permission for certain types of development described in Schedule 2 of the Order. The grant of permission is subject to the limitations, conditions, and provisions set out within the relevant Classes of permitted development.

Under Schedule 2, Part 3, Class R of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), development consisting of the change of use of a building and any land within its curtilage from a use as an agricultural building to a flexible commercial use may be permitted, subject to compliance with the limitations, conditions and prior approval requirements set out within the Order.

## RELEVANT PLANNING HISTORY

Proposal / Summary

PL/2021/11058 - Enclosing both sides and back of existing barn with metal corrugated sheet to match with existing – Approved with Conditions – 20/01/2022.

PL/2022/07757 - Prior Notification for a Change of use from Agriculture to Flexible Commercial Use falling within Classes E (g) (i), pursuant to Schedule 2, Part 3, Class R of the Town and Country Planning (General Permitted Development) (England) Order 2015 – Prior Approval Not Required – 30/11/2022.

PL/2023/02149 - External alterations to existing barn – Approved with Conditions – 11/05/2023

PL/2026/04508 - Material operations pursuant to, and consistent with, development granted planning permission under Application No. PL/2023/02149 – Undetermined at the time of writing.

## CONSULTATIONS

Wiltshire Council Highways Officer: No objections.

Wiltshire Council Public Protection Officer: No objections.

## ASSESSMENT

This application seeks prior approval under Schedule 2, Part 3, Class R of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), which permits the change of use of an agricultural building and any land within its curtilage to a flexible commercial use subject to compliance with the limitations, conditions and prior approval requirements set out within the Order.

Does the Proposal Qualify as Permitted Development?

Development under Class R is subject to a number of conditions and limitations. Development is not permitted by this class if any of the specified restrictions apply. These are set out at Appendix 1. Subject to compliance with these criteria and any limitations or conditions specified within the Order, the proposed development may be considered permitted development.

Having completed the detailed checklist at Appendix 1, it is concluded that the proposed development qualifies as permitted development under Schedule 2, Part 3, Class R of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), and the assessment may now proceed to consider whether prior approval is required by the LPA.

Is Prior Approval Required?

Where a proposal complies with all the provisions limitations set out under Class R of Schedule 2, Part 3 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), the developer must apply to the Local Planning Authority (LPA) for a determination as to whether prior approval is required for the specified impacts and, if so, whether it should be granted.

In respect of this proposal, the following considerations must be assessed as part of the prior approval process as set out by R.3 (1)(b)]:

(b) where the cumulative floor space of the building or buildings which have changed use under Class R within an established agricultural unit exceeds 150 square metres, apply to the local planning authority for a determination as to whether the prior approval of the authority will be required as to—

(i) transport and highways impacts of the development;

(ii) noise impacts of the development;

(iii) contamination risks on the site; and

(iv) flooding risks on the site.

The cumulative floor space of the building or buildings which have changed use under Class R exceeds 150m2 and as such, the above prior approval matters must be assessed.

In addition Paragraph W sets out a number of requirements for both the developer (in terms of the application submission) and the LPA (relating to consultations and advertisement of the application). Para. W(10) states that:

In the case of Class R development, Paragraph W requires the Local Planning Authority to consult specified bodies and to consider any representations received in relation to the prescribed prior approval matters. In accordance with these requirements, consultation has been undertaken with the Highway Authority and Public Protection Officers. The application has been assessed having regard to the consultation responses received and the matters prescribed under Class R, namely transport and highways impacts, noise impacts, contamination risks and flooding risks.

The local planning authority must, when determining an application—

- take into account any representations made to them as a result of any consultation under sub-paragraphs (5) or (6) and any notice given under sub-paragraph (8);

- have regard to the National Planning Policy Framework …, so far as relevant to the subject matter of the prior approval, as if the application were a planning application; and

- in relation to the contamination risks on the site—

(i) determine whether, as a result of the proposed change of use, taking into account any proposed mitigation, the site will be contaminated land as described in Part 2A of the Environmental Protection Act 1990, and in doing so have regard to the Contaminated Land Statutory Guidance issued by the Secretary of State for the Environment, Food and Rural Affairs in April 2012, and

(ii) if they determine that the site will be contaminated land, refuse to give prior approval.

Extent of Assessment Under Class R

Whilst paragraph W requires the Local Planning Authority to have regard to the National Planning Policy Framework, the authority's assessment remains limited to those matters specified within Class R and paragraph W of the GPDO. Matters falling outside the scope of the prior approval process cannot be taken into account when determining the application.

Accordingly, issues such as the principle of the development, compliance with the development plan, planning balance considerations, housing need (where relevant), the sustainability of the location and other matters falling outside the specified prior approval considerations are not relevant to the determination except where they directly relate to one of the prescribed prior approval matters.

## Transport and highways impacts of the development

Policy Context

Policies TR3-TR8 of the National Planning Policy Framework seek to ensure that development provides safe and suitable access for all users and does not result in unacceptable impacts on highway safety. National policy also confirms that development should only be refused on transport grounds where there would be an unacceptable impact on highway safety, or where the residual cumulative impacts on the road network would be severe.

In terms of locational sustainability, the Local Planning Authority (LPA) is not able to assess the general sustainability of the site's location in relation to access to services, employment, or facilities. This is not a consideration under the relevant prior approval provisions. Accordingly, the key matter for assessment is the impact of the proposal on highway safety, particularly in relation to the site access and the surrounding local highway network.

Assessment

The site is served by an existing vehicular access onto the B4192 which would continue to serve the building following the proposed change of use. Following initial concerns regarding visibility, parking provision and traffic generation, the applicant submitted additional highways information including a parking layout, visibility plan and transport assessment.

The submitted information demonstrates that the site can accommodate seven parking spaces and assesses the likely vehicle movements associated with the proposed Class E(g)(i) use. The application also includes details of the visibility available at the existing access.

Having reviewed the additional information, the Highway Officer has confirmed that the concerns previously raised have been satisfactorily addressed and raises no objection to the proposal. No conditions have been requested. Significant weight is therefore afforded to the professional advice of the Highway Authority. It is considered that the proposal would not result in an unacceptable impact on highway safety and the transport and highway impacts of the development are acceptable.

Conclusion

Prior approval in relation to traffic matters is required and is granted.

## Noise impacts

Policy Context

Policy P3 of the National Planning Policy Framework requires development to be appropriate for its location, taking account of the likely effects of pollution on health, living conditions and the natural environment. In relation to noise, the policy seeks to ensure that development provides healthy living conditions for future occupants, avoids unacceptable adverse effects on health and quality of life, and incorporates mitigation where necessary.

In the context of this proposal, the key consideration is whether the development would result in, or be exposed to, unacceptable levels of noise, and whether any such impacts can be appropriately mitigated to ensure a suitable living environment.

Assessment

The site occupies a relatively isolated rural location and is physically separated from neighbouring properties, including Knighton House. The proposed use falls within a category of development that can be carried out in a residential area without detriment to amenity.

Wiltshire Council Public Protection Officers were consulted on the application and raise no objection. Having regard to the nature, scale and location of the proposed use, it is not considered that the development would result in unacceptable noise impacts. Accordingly, the noise impacts of the proposed development are considered acceptable.

Conclusion

Prior approval in relation to noise matters is not required.

## Contamination risks

Policy Context

Policy P2 of the National Planning Policy Framework requires sites proposed for development to be suitable for their intended use, taking account of ground conditions, including any risks arising from land instability or contamination, and the extent to which such risks can be mitigated. Following remediation, land should not be capable of being determined as contaminated land under Part IIA of the Environmental Protection Act 1990. The policy also requires adequate site assessment information, prepared by a competent person, to be available to inform the assessment of these matters.

In the context of this proposal, the Local Planning Authority must be satisfied that the site is not affected by contamination or land stability issues that would make it unsuitable for its intended use, and that any necessary investigations or mitigation measures have been appropriately considered.

Assessment

There is no evidence before the Local Planning Authority to suggest that the site is affected by contamination or that the proposed use would give rise to contamination risks.

The site has historically been used for agricultural purposes and no contamination concerns have been identified through the application process. Furthermore, no objections have been raised by Wiltshire Council Public Protection Officers in respect of contamination risks. It is therefore considered that the proposed change of use would not result in the site becoming contaminated land as defined by Part 2A of the Environmental Protection Act 1990 and that the site is suitable for the proposed use.

Accordingly, the contamination risks associated with the proposed development are considered acceptable.

Conclusion

Prior approval in relation to contamination matters is not required.

## Flooding risks

Policy Context

Policies F4-F8 of the National Planning Policy Framework seek to ensure that flood risk is appropriately managed and that development is safe for its lifetime without increasing flood risk elsewhere. Where flood risk is a specified prior approval matter under the relevant GPDO class, the assessment should focus on the potential flood risk implications of the development and whether any mitigation measures are necessary.

Assessment

The application site is not located within an area identified as being at significant risk of flooding and no evidence has been submitted to suggest that the proposed change of use would be affected by, or increase, flood risk. The proposal relates to the change of use of an existing building and does not involve operational development which would be likely to materially affect surface water drainage characteristics.

Having regard to the location of the site and the nature of the proposal, it is considered that the development would not give rise to unacceptable flood risk concerns. Accordingly, the flooding risks associated with the proposed development are considered acceptable.

Conclusion

Prior approval in relation to flood risk matters is not required.

CONCLUSION

The proposed development has been assessed against the provisions, limitations, and conditions set out in Schedule 2, Part 3, Class R of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended). Based on the submitted plans, supporting information, and the findings of this report, it is concluded that the proposal complies with the criteria for permitted development, and therefore falls within the scope of Class R.

Following assessment, it is determined that prior approval is required in relation to transport and highways matters, but is not required for noise, contamination and flood risk matters.

Having considered the relevant matters that require prior approval, it is further concluded that:

Prior approval is granted, subject to the development being carried out in accordance with the submitted details.

RECOMMENDATION:

That prior approval is not required for noise, contamination and flood risk matters but is required in relation to transport and highways matters, and is GRANTED.

| A |  |  |
| --- | --- | --- |

| A |  |  |
| --- | --- | --- |
| APPENDIX 1 – PERMITTED DEVELOPMENT CHECKLIST CLASS R |  |  |
| Class R – agricultural buildings to a flexible commercial use |  |  |
|  |  | Yes / No |
| R. | Permitted Development Note: If the answer is "Yes", proceed to assessment under R.1. If the answer is "No", the proposal does not qualify as permitted development under this class. |  |
|  | Does the development consist of a change of use of a building and any land within its curtilage from a use as an agricultural building to a flexible use— (a) falling within one of the following provisions of the Use Classes Order— (i)Class B2 (general industrial) of Schedule 1; (ii)Class B8 (storage or distribution) of Schedule1; (iii)Class C1 (hotels) of Schedule 1; (iv)Class E (commercial, business or service) of Schedule 2; or (v)Class F.2(c) (outdoor sport or recreation) of Schedule 2; or (b) for the provision of agricultural training. | Yes |
| R.1 | Development is not permitted by Class R if - Note: If the answer to any of the questions below is "Yes", the proposed development would not constitute permitted development under this class |  |
| (a) | Was the building not used solely for an agricultural use as part of an established agricultural unit: (i) on 3rd July 2012? (ii) if not in use on that date, when it was last in use before that date? (iii) if brought into use after 3rd July 2012, for a continuous period of at least 10 years before the date development under Class R begins? | No |
| (b) | Has the cumulative floor space of buildings that have changed use under Class R within the established agricultural unit exceeded 1,000 square metres? | No |
| (c) | Is the site, or does it form part of, a military explosives storage area? | No |
| (d) | Is the site, or does it form part of, a safety hazard area? | No |
| (e) | Is the building a listed building or a scheduled monument? | No |