# Project: Proposed New Dwellings at Land West of North Lodge, Codford, Wiltshire BA12 0NJ – Light Spill Report – Revision 2

Council Comments

The site is in the core area of this International Dark Sky Reserve. The material considerations therefore include:

* The NPPF, especially paragraphs 189 (“Great weight should be given to conserving and enhancing landscape and scenic beauty in National Parks [...]”) and 198c (“[Planning policies and decisions should] limit the impact of light pollution from artificial light on local amenity, intrinsically dark landscapes and nature conservation.”)

* Section 84(4) of CRoW Act 2000 (“A local planning authority whose area consists of or includes the whole or any part of an area of outstanding natural beauty has power [...] to take all such action as appears to them expedient for the accomplishment of the purpose of conserving and enhancing the natural beauty of the area of outstanding natural beauty or so much of it as is included in their area.”  Part of the natural beauty of this National Landscape (NL) is its dark night skies.

* Section 85 of the CRoW Act 2000 (as amended by s.245 of LURA 2023), which obliges relevant authorities to “seek to further the purpose of conserving and enhancing the natural beauty of the area of outstanding natural beauty”. Part of the natural beauty of this NL is its dark night skies.

* Section 9 (especially DNS A, DNS1 and DNS2) of the Partnership (Management) Plan of this NL which constitutes the constituent LPA's policies for the NL and through which the LPAs are obliged to reduce light pollution year on year (ie not merely attempt to minimise increases).

* The location of the development is in the core zone of an IDSR so any external lighting, or internal lighting that shines outside, should comply with the criteria for Environmental Zone E0 as defined by the Institution of Lighting Professionals in its GN01/2021 on the Reduction of Obtrusive Light.

* This NL’s Position Statement on Light Pollution and Good Practice Guides on Good Lighting.

Ecology Comments

5. Ecological Enhancements

Post Construction

• Provision of two bat boxes built-in to the two new dwellings.

• Provision of two bird boxes attached to the two new dwellings.

• New lengths of species-rich native hedgerows will be planted across both proposed residential plots, totalling a length of approximately 135 meters with species including hawthorn Crataegus monogyna, beech Fagus sylvatica, holly Ilex aquifolium, hornbeam Carpinus betulus and spindle Euonymus europaeus.

• New native tree and shrub planting to be carried out in the retained off-site woodland, within the ownership boundary to the north-west. To include provision of common lime Tilia × europaea, hornbeam, field maple Acer campestre, hazel Corylus avellana, hawthorn, and spindle.

Lighting Design Comments (Oliver Atkins – Inform Consulting Engineers)

# Mitigation

- All external lighting will comprise LED downlights with 0° upward tilt, directing 100% of the light downwards to minimise sky glow, glare and light spill. The proposed luminaires have achieved Dark Sky Accreditation. This will minimise light spill from the development while maintaining appropriate lighting for the safety of residents.

- Warm white 2700K luminaires have been selected to reduce blue light content and minimise the potential impact on nocturnal wildlife. Bat boxes will also be provided to the new dwellings.

- All windows will be fitted with motorised internal opaque blinds, which will automatically close between dusk and dawn to prevent light spill from within the property. The blinds will be controlled by a photocell and timeclock arrangement and will fail to the closed position in the event of a loss of power.

- External lighting will generally remain off during the hours of darkness and will only operate when required for safe access and egress around the property.

- External lighting will be controlled by PIR movement detectors, with 1No. PIR provided to both the north and south elevations of each property. Each PIR will control the lighting serving its respective elevation independently. The PIR-controlled lighting will only be enabled between dusk and dawn, and where ambient light levels are insufficient for safe access and egress. When movement is detected, the relevant external lighting will operate for a pre-set period of approximately 3 minutes before automatically switching off. This period can be adjusted to suit any specific requirements of the Local Authority.

- Refer to the accompanying Light Spill Report and drawing IN827(632)001 for further information.

Lux Levels

The Relux report demonstrates that all light spills beyond the boundaries of new hedgerows and boundaries will be <0.5lx. 

Due to bat box installation, Vertical lighting calculations have been produced to indicate the light spill at first floor level to the bat box positions. This is proven to be 0 Lux.

Oliver Atkins

Associate Director

Equinox South

Great Park Road

Bradley Stoke

BS32 4QL

07502 206106

0117 901 2534

oliver.atkins@informce.co.uk

www.informce.co.uk