APPLICATION FOR LISTED BUILDING CONSENT

OFFICER DELGATED REPORT

| APPLICATION REFERENCE: | PL/2026/03751 |
| --- | --- |
| APPLICATION TYPE: | Listed building consent (Alt/Ext) |
| SITE ADDRESS: | Lacock Abbey, High Street, Lacock, SN15 2LG |
| DESCRIPTION OF WORKS: | Addition of support to roof structure above dining room ceiling to reduce movement which is causing ceiling cracking |
| RECOMMENDATION: |  |

### SITE DESCRIPTION/CONTEXT

Lacock Abbey is a mostly two storey Country house formed in the mid-16th century through remodelling and extension of cloister court of 13th to 15th Augustinian nunnery founded 1229 that was sold to Sir W. Sharington at the Dissolution 1540.

Further alterations were undertaken around 1700 and 1740 for Sir John Talbot and John Ivory Talbot and entrance front rebuilt as Gothick Great Hall 1754-5 by Sanderson Miller for J.I. Talbot. The South front in a Tudor style 1828-30 for W.H. Fox-Talbot and circa 1900 a restoration of medieval parts by H. Brakspear for C.H. Talbot.

The building is constructed in ashlar and rubble stone with mostly stone slate roofs with numerous ornate 16th twisted stacks.

The major rooms of the house are located on the first floor over the medieval basement, which comprises three sides of a lierne vaulted 15th century cloister with two 14th  century bays at south-west angle and spaces off.

Some medieval roofs survive, to the upper part of house, 15th century with arch braced collar trusses and 3 tiers of windbracing concealed to the north side refectory, and to south side dormitory, 14th century with 3 tiers of cusped windbracing.

The Abbey is located in its own large estate grounds at the west edge of Lacock village and within the Conservation Area.

### PROPOSAL

Additional structural support to roof structure above dining room ceiling removal and repair of ceiling cracking.

### RELEVANT PLANNING HISTORY

Proposal / Summary

ENQ/2021/04585 - Pre-Application enquiry Decision - Pre-App Response Date: 20/12/2021 – To Install a Lightning Protection System to the External Surfaces (Roof and Walls) at Lacock Abbey

PL/2022/00616 - Listed Building Discharge of Condition Decision - Approve Date: 13/6/2022 – Discharge of Conditions 2 and 3 of 21/01057/LBC (Taking down and rebuilding the bottom masonry section of the dormer window. Replacing section of plaster to the top of the rendered section. Splice repairs to the timber mullions and replacement of the bott

N/92/00701/LBC - Listed building consent Decision - Approved Date: 26/7/1992 – REFURBISHMENT OF VACANT CORNERFLAT TO INCLUDE A NEW ROOM WITHIN THE WATER HOUSE/ ALTERATIONS TO INTERNAL PARTITIONS AND GENERAL REPAIRS REFURBISH FLAT/ALTERATIONS

N/11/02612/PRAP - Pre-Application enquiry Decision - Object Date: 21/9/2011 – Surfacing of an Existing Grassed Car Parking Area.

N/04/00768/TCA - Work to Trees in a Cons Area Decision - Approved Date: 10/5/2004 – TREE SURGERY

N/98/00629/LBC - Listed building consent Decision - Unknown Date: 30/4/1998 – ALTERATIONS TO ROOF/REMOVAL OFCORRIGATED IRON SHEETING & CLADDING FROM GARDENERS STORE & REPLACE WITH SECOND HAND WELSH SLATE REPLACE ROOF

N/98/01924/LBC - Listed building consent Decision - Unknown Date: 11/12/1998 – INSTALLATION OF REPLACEMENT GAS FIRED BOILERS AND ASSOCIATED WORKS REPLACEMENT GAS BOILERS

N/07/01928/LBC - Listed building consent Decision - Approve with Conditions Date: 17/9/2007 – Repairs to Existing Earth Floor Coverings Within Cloister Rooms and Bakery

N/98/01959/LBC - Listed building consent Decision - Unknown Date: 3/11/1998 – PROVISION OF A PAIR OF GATES WITHIN ARCHED GATEWAY FROM DRIVE TO MAIN ABBEY ENTRANCE GATES

N/06/02054/TCA - Work to Trees in a Cons Area Decision - Approved Date: 20/10/2006 – Crown Reduction of Swamp Cypress Tree

N/08/02373/TCA - Work to Trees in a Cons Area Decision - Approved Date: 28/11/2008 – Fell 2 Robinia, 1 Siberian Crab Apple, 1 Cedar, 1 Holly and 2 Planes

N/00/01182/LBC - Listed building consent Decision - Approve with Conditions Date: 21/7/2000 – FIX CAST IRON GUTTERING

N/99/02969/LBC - Listed building consent Decision - Unknown Date: 25/2/2000 – ADDITION OF FRAME AND INNER DOORS IN FORMER OIL STORE; UNBLOCK OPENING BETWEEN NUNS KITCHEN AND SITING ROOM

N/03/02855/TCA - Work to Trees in a Cons Area Decision - Approved Date: 9/12/2003 – TREE FELLING AND SURGERY WORK

14/05541/FUL - Full planning permission Decision - Approve with Conditions Date: 3/9/2014 – Relocation of Vehicle Parking within Old Woodyard to Replace Existing Parking within Courtyard

N/04/00056/TCA - Work to Trees in a Cons Area Decision - Approved Date: 19/2/2004 – TREE SURGERY WORK

14/00008/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 28/1/2014 – Fell 1 Cedar

N/89/00106/TCA - Work to Trees in a Cons Area Decision - Approved Date: 1/3/1989 – TREE SURGERY WORK WITHIN LACOCK CONSERVATION AREA

13/04248/PREAPP - Pre-Application enquiry Decision - Pre-App Response Date: 4/10/2013 – Resubmission of Pre-app 12/03488/PRAP Redevelopment of Abbey Courtyard to Exclude Extension to Visitor Toilets and Conversion of Old Stables into Catering Facilities. Use of Old Woodyard as Parking Area for Vehicles Currently Parking in the Abbey Courtya

N/99/00498/LBC - Listed building consent Decision - Unknown Date: 27/4/1999 – INTERNAL AND EXTERNAL ALTERATIONS

N/97/01843/TCA - Work to Trees in a Cons Area Decision - Unknown Date: 6/10/1997 – TREE FELLING TREE FELLING

N/95/00388/TCA - Work to Trees in a Cons Area Decision - Approved Date: 24/4/1995 – TREE SURGERY TREE SURGERY

N/11/01568/PRAP - Pre-Application enquiry – Conservation of Wall painting

N/92/01823/LBC - Listed building consent Decision - Approved Date: 7/12/1992 – PROPOSED INSTALLATION OF PRIVATE RESIDENTIAL BATHROOM INSTALLATION OF BATHROOM

N/92/01988/LBC - Listed building consent Decision - Approved Date: 15/1/1993 – INTERNAL ALTERATIONS TO DOORS/PARTITIONS/CEILINGS AND INSULATION AS PART OF FIRE PRECAUTIONS WORK INTERNAL ALTERATIONS

N/12/00184/TCA - Work to Trees in a Cons Area Decision - Approved Date: 5/3/2012 – Works to Various Trees as Per Agents Schedule

N/94/01579/LBC - Listed building consent Decision - Approved Date: 31/10/1994 – ALTERATIONS TO FIRE DOORS AND TWO NEW ACCESS HATCHES ALTS TO FIRE DOORS & HATCHES

N/10/00365/TCA - Work to Trees in a Cons Area Decision - Approved Date: 24/3/2010 – Tree Surgery to 4 Willow Trees

N/01/02648/TCA - Work to Trees in a Cons Area Decision - Approved Date: 1/2/2002 – FELL ONE SILVER MAPLE

N/00/01376/TCA - Work to Trees in a Cons Area Decision - Approved Date: 24/7/2000 – TREE SURGERY COMPRISING DEAD WOODING, MAKING SAFE AND CROWN REDUCTION OF 10 NO MATURE TREES

N/11/01632/LBC - Listed building consent Decision - Refused Date: 13/7/2011 – Surfacing of an Existing Grassed Car Parking Area.

N/11/00076/TCA - Work to Trees in a Cons Area Decision - Approved Date: 24/2/2011 – Fell Horse Chestnut, 2 Yew Trees, 1 Ash & Holly, Crown Reduction to Beech Tree and Works to 3 Bay Trees and One Holly.

N/11/01655/FUL - Full planning permission Decision - Refused Date: 13/7/2011 – Surfacing of an Existing Grassed Parking Area

N/03/03051/LBC - Listed building consent Decision - Approve with Conditions Date: 30/1/2004 – RETAINING WALLS AND GRILLES TO ALLOW PERMANENT DISPLAY OF MONASTIC DRAIN

N/10/03093/PENQ - Pre-Application enquiry Decision - Pre-App Response Date: 20/8/2010 – Erect Ventilated Screen in front of Wall Painting of St Augustine

N/96/00123/FUL - Full planning permission Decision - Unknown Date: 19/4/1996 – INSTALLATION OF NEW WASTE TREATMENT PLANT WASTE TREATMENT PLANT

N/92/01452/LBC - Listed building consent Decision - Approved Date: 25/9/1992 – FIXING OF 7 CARVED STONE BEASTS ONTO THE PARAPET OF SHARINGTONS TOWER FIXING OF 7 STONE BEASTS

N/07/01224/TCA - Work to Trees in a Cons Area Decision - Approved Date: 4/7/2007 – Fell one yew tree

N/09/00881/TCA - Work to Trees in a Cons Area Decision - Approved Date: 2/7/2009 – Fell Beech & Cherry Trees

14/05161/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 24/6/2014 – Fell 1 Mulberry Tree

N/93/02202/TCA - Work to Trees in a Cons Area Decision - Approved Date: 27/1/1994 – TREE FELLING TREE FELLING

N/04/01905/TCA - Work to Trees in a Cons Area Decision - Approved Date: 9/8/2004 – REMOVE 2 PLUM TREES

N/06/01644/LBC - Listed building consent Decision - Approve with Conditions Date: 13/9/2006 – Install Latchway to Improve Safety of Access to Cloister Roof

N/07/00146/TCA - Work to Trees in a Cons Area Decision - Approved Date: 12/3/2007 – Tree Surgery Work and Felling

N/06/00582/TCA - Work to Trees in a Cons Area Decision - Approved Date: 9/5/2006 – Tree Surgery Work to Trees within the Grounds of Lacock Abbey

N/12/00721/PRAP - Pre-Application enquiry Decision - Pre-App Response Date: 10/5/2012 – Erection of free-standing screen to protect St Augustine wall painting in The Cloisters. Screen to be sited close to the wall and to include environmental monitoring and control.

N/05/01981/TCA - Work to Trees in a Cons Area Decision - Approved Date: 20/9/2005 – Tree Surgery and Felling Work to Various Sites

N/95/00416/LBC - Listed building consent Decision - Approved Date: 4/5/1995 – REMOVAL OF PARTITION WALL AND INSTALLATION OF BOILER AND HEATING SYSTEM TO GROUND FLOORFLAT AND FIRST FLOOR GUEST ROOMS ALTERATIONS/HEATING SYSTEM

N/97/01846/LBC - Listed building consent Decision - Unknown Date: 6/1/1998 – ALTERATIONS TO STORE ROOM IN STABLEBLOCK TO FORM OFFICE FOR NATIONAL TRUST CUSTODIAN AND REST ROOM FOR ROOM STEWARDS ALTERATIONS TO STABLEBLOCK

14/00514/PREAPP - Pre-Application enquiry Decision - Pre-App Response Date: 25/9/2014 – Redevelopment of Abbey Courtyard to include Extension to Visitors Toilets and Conversion of Old Stables into Catering Facilities. Use of Old Wood Yard as Parking Area

14/06065/DDD - Dead Die and Dangerous Decision - DD Decision Letter Date: 27/6/2014 – Fell 1 Robinia

14/05830/LBC - Listed building consent Decision - Withdrawn Date: 3/9/2014 – Relocation of Vehicle Parking within Old Woodyard to Replace Existing Parking within Courtyard

14/11717/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 13/1/2015 – Fell 8 Holly, 1 Horse Chestnut, 1 Twin Stem Sycamore, 2 Beech, 6 Sycamores, 10% Crown Reduction to 1 Beech and 1 Horse Chestnut, 20% Crown Reduction to 1 Robina, Reduction to 2 Limbs of Acer by up to 6 Metre, and Up to 5 Metre Crown Reduction to 1 Judas

15/08997/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 13/10/2015 – Fell 2 Beech Trees and 1 Elder.

16/10014/FUL - Full planning permission Decision - Approve with Conditions Date: 4/1/2017 – Alteration of existing toilets on western side of the Abbey Courtyard and conversion of coach house to the south of the existing toilets to form larger toilet faciilties.

16/10317/LBC - Listed building consent Decision - Approve with Conditions Date: 4/1/2017 – Alteration of existing toilets on western side of the Abbey Courtyard and conversion of coach house to the south of the existing toilets to form larger toilet facilities.

16/00579/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 16/2/2016 – Fell 3 Horse Chestnut Trees, 1 Ash Tree, 1 Sycamore Tree, 4 Hawthorn Trees, 1 Yew Tree and 1 Beech Tree.

17/05974/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 18/7/2017 – Fell 1 Twin-Stem Goat Willow, 2 Sycamore, 1 Ash and Group of Thorn

16/11558/FUL - Full planning permission Decision - Approve with Conditions Date: 2/3/2017 – Alterations to the stables in the ground floor of the Stables Courtyard at Lacock Abbey to create a tea room and to make a new refuse area to the rear of the existing visitor toilets (following enlargement of existing facilities).

16/11956/LBC - Listed building consent Decision - Approve with Conditions Date: 2/3/2017 – Alterations to the stables in the ground floor of the Stables Courtyard at Lacock Abbey to create a tea room and to make a new refuse area to the rear of the existing visitor toilets (following enlargement of existing facilities).

16/10186/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 16/11/2016 – See Work Schedule

17/07219/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 5/9/2017 – Crown Lift 3 Beech Trees to 5 Metres from Ground Level, Fell main Stem of Holly Tree, and Redcue Branches of Beech Tree Overhanging Wall by up to 6 Metres,

17/07231/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 5/9/2017 – Fell 1 Willow Tree

17/12332/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 16/1/2018 – Fell 4 Elder, a Group of 20 Blackthourn Suckers, 2 Prunus and 1 Sycamore

20/06223/CLPLB - Certificate of Lawfulness of Proposed Works to a listed building Decision - Approve Date: 24/9/2021 – To make the existing fire exit compliant with fire safety

19/11961/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 10/1/2020 – Crown Lift Beech Tree to 4 Metres from Ground Level

17/05484/DDD - Dead Die and Dangerous Decision - DD Decision Letter Date: 7/6/2017 – Fell 1 Dangerous Robinia

19/09665/DDD - Dead Die and Dangerous Decision - DD Decision Letter Date: 10/10/2019 – Fell 1 Dead Poplar Tree - leaning heavily, fell due to high risk of failure - could potentially send debris onto the pavement/road or cause secondary tree failure onto the pavement/road.

20/09157/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 27/1/2021 – Fell 1 Willow (T1), Fell 1 Elder (T12), Reduce Height of North Stem of Maple by 4 Metres and Extended Laterals by 2 Metres (T13),Fell 1 HOlly Tree (T14), Fell 1 Hazel (T15), Fell Group of Dead Elm (T16)

20/01342/LBC - Listed building consent Decision - Withdrawn Date: 18/6/2020 – Apply Stabilising Coat to Earthen Floors to Cloisters.

19/03633/DOC - Approval of details reserved by a condition Decision - Withdrawn Date: 17/10/2019 – Discharge of condition 3 on 17/04729/FUL

19/10437/LBC - Listed building consent Decision - Approve with Conditions Date: 22/4/2020 – Structural Works to Turret, Including Replacement of Historic Timber Beams with new oak Beams.

16/06719/PREAPP - Pre-Application enquiry Decision - Pre-App Response Date: 2/9/2016 – Enlargement / Alteration to Exiting WC's and Provision of Tea Room to Stables within Lacock Abbey Courtyard

19/09149/FUL - Full planning permission Decision - Approve with Conditions Date: 14/1/2020 – Construct composting clamps from steel posts and timber railway sleepers, including a lightly gravelled work area in front of the compost clamps and extend an existing track to connect to work area.

19/09255/LBC - Listed building consent Decision - Approve with Conditions Date: 17/12/2019 – Construct a 30 minute fire screen floor to ceiling. Including door, inside the ground floor store in the courtyard building under the cafeteria for fire safety purposes.

20/00803/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 22/4/2020 – Fell 1 Chestnut tree (T1)

20/00809/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 21/5/2020 – 1 Metre Crown Reduction to 3 Apple Trees (T1, T4 and T8) and 1 Cherry Tree (T2) Fell 3 Damson Trees (T3, T5 and T6) and 1 Ash (T9), Remove Small Branch from Laburnum Tree (T7)

20/00811/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 21/5/2020 – Crown Lift 5 Yew Trees, 1 Beech Tree, 1 Holly Tree and 1 Lime Tree (T1 - T8)

21/01057/LBC - Listed building consent Decision - Approve with Conditions Date: 12/8/2021 – Taking down and rebuilding the bottom masonry section of the dormer window. Replacing section of plaster to the top of the rendered section. Splice repairs to the timber mullions and replacement of the bottom timber rail. Replacement of a decorative timbe

19/11972/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 16/1/2020 – Fell 4 Apple Trees

18/00140/TCA - Work to Trees in a Cons Area Decision - NOOBJECT Date: 9/2/2018 – Fell 9 Silver Acer Trees and Coppice 4 portugal Laurens to up to 900mm Height

PL/2023/09842 - Approval of details reserved by a condition Decision - Partial Discharge of Condition(s) Date: 27/2/2024 – Discharge of Condition No 3 on PL/2023/05998 - Installation of conventional lightning protection system employing a Faraday Cage involving network of air terminations and down conductors over roofs and elevations.

PL/2022/07408 - Notification of proposed works to trees in a conservation area Decision - No Objection Date: 3/11/2022 – The Chestnut lost major limb in the early part of the summer this has left the tree unbalanced so the plan is to reduce the crown

PL/2022/07907 - Notification of proposed works to trees in a conservation area Decision - No Objection Date: 22/11/2022 – Judas tree on South Front of Lacock Abbey. Multiple branches over hanging border and lawns making tree unstable. Trunk of tree is vast and undermining the foundations and integrity of the 13th Century Abbey walls and foundations. We are seeking to f

PL/2022/08079 - Notification of proposed works to trees in a conservation area Decision - No Objection Date: 21/11/2022 – We have an overgrown and unsightly group of 12 x young Thuja Conifers forming an unattractive hedge along the end of the short driveway into the National Trust Estate yard. The tree trunks are all no wider in diameter than 8 inches at 4 ft high. We are s

PL/2022/08101 - Notification of proposed works to trees in a conservation area Decision - No Objection Date: 22/11/2022 – Tall Ash tree with a significant lean, root plate has lifted gradually over the past years that the tree has been monitored. The tree has signs of Ash Die Back with purple discoloration to the leaves. There are also signs monitored of ganaderma fungus. T

PL/2022/08556 - Dead Die and Dangerous Decision - No Objection Date: 3/11/2022 – Horse Chestnut tree - reduce crown away from Hither Way

ENQ/2022/01902 - PD Enquiries Decision - Pre-App Response Date: 5/12/2022 – Proposed Installation of Non Permanent Internal Access Ramps/ Viewing Platforms

PL/2023/02254 - Notification of proposed works to trees in a conservation area Decision - No Objection Date: 3/5/2023 – Fell Walnut Tree

PL/2023/02765 - Notification of proposed works to trees in a conservation area Decision - No Objection Date: 2/5/2023 – Walnut, Holme Oak, group of over mature Portuguese laurel shrubs - Fell.

PL/2023/04381 - Notification of proposed works to trees in a conservation area Decision - No Objection Date: 4/7/2023 – Cherry tree - Fell.

PL/2023/04415 - Notification of proposed works to trees in a conservation area Decision - No Objection Date: 10/7/2023 – Box tree - fell

ENQ/2023/00981 - Pre-Application enquiry Decision - Pre-App Response Date: 14/11/2023 – Carry out exploratory works to boiler flue and remove blockage, open up fireplaces internally to encourage drying out, improvement of the cowl system to prevent birds nesting, consult a heating engineer on best course of action for extraction of boiler ga

PL/2023/05998 - Listed building consent (Alt/Ext) Decision - Approve with Conditions Date: 12/9/2023 – Installation of conventional lightning protection system employing a Faraday Cage involving network of air terminations and down conductors over roofs and elevations

PL/2023/07181 - Notification of proposed works to trees in a conservation area Decision - No Objection Date: 27/9/2023 – Work to the Taxodium (Photo Taxodium 1), Beech (Photo Beech 1), Horse Chestnut (Photo Horse chestnut 1), Cedar (Photo Cedar 1/2) and crown lift of other trees around the pond (Photo pond 1/2) in Lacock Abbey. Map attached with various trees locations (Pho

ENQ/2024/00819 - PD Enquiries Decision - PP Required Date: 26/6/2024 – Summer of play hub with marquee tipi and 2 bell tents

PL/2024/11549 - Notification of proposed works to trees in a conservation area Decision - No Objection Date: 21/1/2025 – Juglans Nigra (T1) - Sensitively reduce limbs with historic wounds on up to 2 meters. Reduce weight to reduce risk of limb failure at fault points. This is an historic tree planted in the 1800's by William Henry Fox Talbot (WHFT) that need work done to pr

PL/2025/00860 - Notification of proposed works to trees in a conservation area Decision - No Objection Date: 26/2/2025 – Apple (T1a), Hawthorn T1b, T1c, Malus T1d, London Plane T8, Laburnum T6, Magnolia T7 - Remove; Prunus (T1e) - Central leader stem removed and branch reduction; Lime (T2), (T3a), (T3b), (T4), (T5a), (T5b), (T11), T12) - Crown lift lower to 4 metres; Apple

PL/2025/04417 - Notification of proposed works to trees in a conservation area Decision - No Objection Date: 19/6/2025 – Scots Pine - Fell.

PL/2025/05070 - Full planning permission Decision - Approve with Conditions Date: 30/9/2025 – Temporary planning permission to use the area as a summer of play hub during July and August each year for the next 3 years (up to September 2028). To include the siting of a marquee, tipi, 2 bell tents, stage and festival flags.

PL/2025/09068 - Listed building consent (Alt/Ext) Decision - Approve Date: 12/3/2026 – Works to carefully dismantle and re-build defective chimney

PL/2025/09087 - Listed building consent (Alt/Ext) Decision - Approve Date: 12/3/2026 – Removal of masonry chimney capping and replacement with stainless steel flue liner and terminal

PL/2025/09735 - Approval of details reserved by a condition Decision - Approve Date: – Discharge of conditions 1 (work commencement deadline) and 3b (Archaeological works) of PL/2023/05998 - Installation of conventional lightning protection system employing a Faraday Cage involving network of air terminations and down conductors over roofs

PL/2026/02061 - Dead Die and Dangerous Decision - No Objection Date: 30/3/2026 – Cherry tree - Fell

20/09716/LBC - Listed building consent Decision - Approve with Conditions Date: 18/1/2021 – Retention of works to replace most of the 20th century floor joists and sole plates within the living room.

N/94/00657/FUL - Full planning permission Decision - Approved Date: 22/4/1994 – ERECTION OF TIMBER GREENHOUSE TIMBER GREENHOUSE

N/98/00784/LBC - Listed building consent Decision - Withdrawn Date: 29/12/1998 – CONTRUCTION OF FOOTPATH WITHIN THE CLOISTERS USING SECONDHAND PENNANT FLAGSTONES FOOTPATH WITHIN THE CLOISTERS

PL/2024/02660 - Listed building consent (Alt/Ext) Decision - Approve with Conditions Date: 7/10/2024 – The installation of a viewing platform in the Tithe Barn and installation of a ramp in the Infirmary Passage at The Cloisters, Lacock Abbey

PL/2026/03751 - Listed building consent (Alt/Ext) – Addition of support to roof structure above dining room ceiling to reduce movement which is causing ceiling cracking

RELEVANT LEGISLATION, POLICIES AND GUIDANCE

Legislation

From the point of view of the historic environment the main statutory tests are set out within the Planning (Listed Building and Conservation Areas) Act 1990.  Section 16 requires that special regard be given to the desirability of preserving listed buildings, their settings or any features of special architectural or historic interest which they possess.

Section 72(1) of the Planning (Listed Building and Conservation Areas) Act 1990 also requires the Council to pay special attention to the desirability of preserving or enhancing the character or appearance of designated Conservation Areas.

National Policy and Guidance

The National Planning Policy Framework (NPPF) August 2026 sets out government policy, including in respect of the historic environment.

Chapter 20 - Conserving and enhancing the historic environment, recognises heritage assets as an irreplaceable resource. Heritage assets should be conserved in a manner appropriate to their significance, with harm avoided unless there is a clear and convincing justification. Decision-makers are required to apply a balanced approach, giving substantial weight to the conservation of heritage assets and weighing any harm to significance against the public benefits of the proposal.

Chapter 5 of the NPPF sets out policies for Meeting the challenge of climate change and gives substantial weight to the benefits of improving the energy efficiency of existing buildings and/or drawing energy from district heat networks, renewable and low carbon sources (including through the installation of heat pumps and solar panels where these do not already benefit from permitted development rights).

National Planning Practice Guidance provides guidance on interpreting the NPPF.

Historic England’s Conservation Principles, Policies and Guidance (2008) provides policy and guidance for the “sustainable management of the historic environment”. Additional guidance on the application of policies set out in the NPPF is contained within their publications:

- Historic Environment Good Practice Planning Advice Note 2 – Managing Significance in Decision-Taking in the Historic Environment (2015)

- Historic Environment Good Practice Planning Advice Note 3 – The Setting of Heritage Assets 2nd edition (2017)

- Historic England Advice Note 2 – Making changes to Heritage Assets (2025)

- Historic England Advice Note 18 – Adapting Historic Buildings for Energy and Carbon Efficiency (2026)

- Traditional Windows: their care, repair and upgrading (2017)

Wiltshire Core Strategy:

Whilst not directly relevant to listed building consent applications, the below policies give an indication of the Council’s approach to development affecting heritage assets:

- Core Policy 57 ‘Ensuring high quality design and place shaping’: A high standard of design is required in all new developments, including extensions, alterations, and changes of use of existing buildings. Development is expected to create a strong sense of place through drawing on the local context and being complementary to the locality.

- Core Policy 58 ‘Ensuring the conservation of the historic environment’ requires that “designated heritage assets and their settings will be conserved, and where appropriate enhanced, in a manner appropriate to their significance.”

ISSUES

Impact upon the preservation of the listed building or its setting or any features of special architectural and/or historic interest.

CONSULTATION RESPONSES

Lacock Parish Council – No comments received.

Historic England Advice

The proposals concern the strengthening of the roof structure of the grade I listed Lacock Abbey to prevent further cracking to the dining room ceiling.

We have consulted with our structural engineer, and they have confirmed that they have no concerns regarding the proposed options and agree that the works are required to protect the ceiling below.

To aid your consideration of the application I have summarised their comments:

• It is proposed to introduce two pairs of tension rods on each truss to support the lower tie beam from the upper tie beam. This would distribute stresses more evenly and reduce the deflection in the lower tie beam.

Two options are given in the application.

One using U-shaped brackets which fit underneath the lower truss beam or face-fixed plates on the sides of the beam.

The preferred option will depend on whether there is sufficient space to install a bracket, or whether face-fixing is the only viable solution.

• Two drawings (3.03 and 3.04) show options for proposed works to a secondary beam spanning between the roof trusses.

Option 1 re-supports the secondary beam onto the truss using a joist hangar. The existing strapping is to be retained if found to be in good condition or replaced with similar strapping if necessary.

Option 2 also re-supports the beam onto the truss using a joist hanger, with the addition of two new joists installed on either side of the failed joist to support the floor.

Both options require small sections of the ceiling to be cut out to install the joist hangars.

• Both fixing options shown are reasonable and minimise the disturbance to the historic fabric, and the preferred option will depend on the space available for the fixings.

Recommendation Historic England recommends that your Council may approve this application, which would not harm the significance of Lacock Abbey, unless it finds that other considerations indicate refusal. If there are any material changes to the proposals, please contact us. If you propose to determine the application in its current form, please inform us of the date of the committee and send us a copy of your report at the earliest opportunity. Please also advise us of the decision in due course

REPRESENTATIONS

In accordance with the requirements of Section 5(1) of the Planning (Listed Buildings and Conservation Areas) Regulations 1990 (as amended) the application for listed building consent was publicised by way of:

- A site notice displayed near the site;

- An advertisement placed in the local press.

These methods were used to inform interested parties and to allow a statutory consultation period of 21 days for representations to be submitted.

No representations were received during the consultation period. In accordance with the provisions of the Planning (Listed Buildings and Conservation Areas) Act 1990, only matters relevant to the consideration of the building’s special architectural or historic interest have been taken into account in the assessment of this Listed Building Consent application.

Assessment of significance:

According to the National Planning Policy Framework (NPPF), significance is defined as:

“The value of a heritage asset to this and future generations because of its heritage interest. The interest may be archaeological, architectural, artistic or historic. Significance derives not only from a heritage asset’s physical presence, but also from its setting.”

Historic England further categorises significance into four key values:

- Evidential – the potential of the asset to yield evidence about past human activity.

- Historic – the ways in which the asset illustrates aspects of history.

- Aesthetic – the sensory and design qualities of the asset.

- Communal – the meanings and associations attached to the asset by people.

In this case the significance of the building lies largely with its architectural and historic interest including architectural style, form, layout and architectural expression/detailing and use of local design approaches and materials which have significant interest

ASSESSMENT

In accordance with Section 16(2) of the Planning (Listed Buildings and Conservation Areas) Act 1990, special regard has been given to the significance of the listed building, including its architectural and historic interest, and the desirability of preserving both the building and its setting. Relevant policies within the National Planning Policy Framework (NPPF), particularly Section 16: Conserving and Enhancing the Historic Environment, have also been taken into account. These policies require that any harm to the significance of a designated heritage asset must be clearly justified and weighed against the public benefits of the proposal.

In order to be acceptable, the application needs to be assessed against the following criteria:

Information considered/submitted

Paragraph 207 requires that applicants describe the significance of any heritage assets affected, including any contribution made by their setting. This should include sufficient information to provide a clear understanding of the significance of the heritage asset and its setting and the potential impact of any proposals on that significance.

The application includes a Heritage Design and Access Statement, Dining Room Structural Report, Location Plan, Details Drawings of Secondary Beam strapping, Dining Room Ceiling Existing and Proposed, and Secondary Beam Alternative Scheme.

Assessment of significance:

According to the National Planning Policy Framework (NPPF), significance is defined as:

“The value of a heritage asset to this and future generations because of its heritage interest. The interest may be archaeological, architectural, artistic or historic. Significance derives not only from a heritage asset’s physical presence, but also from its setting.”

In this case the significance of the building comes from its architectural and historic interest,  that is described in the Historic England List description is and its prominent position in respect to the Lacock Conservation Area’s character and appearance.

Impact of proposals on the significance and setting of the building/structure:

Understanding the impact of proposed works on a listed building requires a careful and proportionate assessment of how those changes affect the asset’s heritage significance and its setting. This includes both the physical alterations to the building itself and any changes to its spatial or visual relationship with surrounding structures or spaces.

The significance of a heritage asset is not static; it is shaped by its architectural, historic, and communal values, and by how these are experienced in context. Therefore, any intervention must be considered in light of the asset’s identified significance, with attention given to both potential harm and benefits. Benefits may include repairs that support the long-term conservation of the building, improved energy performance, or enhanced public access and understanding.

This assessment is grounded in the statutory duty under Section 16 of the Planning (Listed Buildings and Conservation Areas) Act 1990, which requires special regard to be given to the desirability of preserving the building and its features of special interest. It is also informed by national policy, particularly policies HE4-9 of the National Planning Policy Framework (NPPF), and relevant local heritage policies and guidance.

The application relates to the Dinng Room that was created from a two-storey interior. The walls in this area of the building show evidence including a filled in window visible that the current ceiling height cuts through and previous ceiling joist sockets in the wall plate above, show the previous modifications where the top storey floor structure was removed to allow the ceiling to be raised to create the dining room in the mid-18th century, so it is expected the ceiling and roof structure date from this time.

Evidence of cracking in the plaster ceiling led to a Structural Engineer’s review of the ceiling. Their investigation found previous (1980’s) metal strapping repairs to the split secondary joist and resin repairs to the ceiling that suggested failure of the plaster nibs. This failure is not apparent throughout the ceiling, so it was concluded that the ceiling cracking that likely relates to the deflection of the joists.

The Engineers have offered ceiling repairs that consist of three main elements.

Firstly, the additional strengthening of the main trusses to reduce the deflection towards the centre of the ceiling. The works involve the addition of a wire tensioning system with the suggested type 3 bracket. This approach requires minimal intervention and is largely reversible as necessary.

Secondly, adding support to the split secondary joist to the northwest of the room. These repairs will require a section of the existing ceiling to be removed to allow a hanger to be installed.

The area of plaster to be removed is along the length of the secondary beam expected to be only as large as needed to install the hanger, rather than removing the whole area below the joist. This area of failed cracked ceiling and that removed to undertake the works around the cracking will require repair in lime plaster once the brackets are in place. The ceiling repairs to the ceiling will be carried out with matching timber lathes and lime plaster. It is not expected that any of these works and repairs will be required close to the decorative cornice.

This approach will preserve as much of the historic fabric as possible. The addition of coach screws from below will then be carried out, spaced evenly along the beam.

If the timber is unable to hold a particular screw, strapping will be installed as an alternative from above, without the need for further plaster removal. This assessment will be made on a per-fixing basis.

The Engineers have indicated that while it is expected that this repair is substantial enough to support the beam, if further cracking reappears, it may need a pair of additional beams to be inserted fitted by hanger to the primary joist and external wall.

The details are provided and these beams will then support the ceiling joist with angle bracket, removing the load from the failed secondary beam. No further ceiling opening will be required.

Thirdly, additional support by metal strapping is proposed to the joint between the primary and secondary ceiling beams. This is intended for the joints above where there is an area of failed ceiling plaster but would also be considered on any joints where the timber blocks are loose and  will allow existing timber support blocks to be retained.

The application therefore is a belt and braces approach where hopefully only the belt is required the latter repairs indicated for new beams and further strapping are potential repairs.

The works that are proposed in the first instance are the wire tensioning system to the beams and adding support to the split secondary joist to the northwest of the room that require a section of the existing ceiling to be removed to allow a hanger to be installed.

The proposal allows for the ceiling repairs to be carried out with matching timber lathes and lime plaster. It is not expected that any of these works and repairs will be required close to the decorative cornice.

Historic England advice from their Structural Engineer comments ‘Both fixing options shown are reasonable and minimise the disturbance to the historic fabric, and the preferred option will depend on the space available for the fixings.’

The proposals involve minor intervention into fabric but this is repairable or reversible and none of the works will be visible within the dining room or include decorative elements of the ceiling moulding. In this case the proposals will also result in considerable heritage benefits by virtue of the proposed investment structural repair which will help to secure the future stability of the building and allow the continued use of the Dining room for National Trust visitors.

CONCLUSION

The NPPF confirms that when considering the impact of a proposed development on the significance of a designated heritage asset, great weight should be given to the asset’s conservation (and the more important the asset, the greater the weight should be). It makes clear that any harm to a designated heritage asset requires clear and convincing justification.

The proposal involves minor intervention into fabric, but this is repairable or reversible and none of the works will be visible within the dining room or include decorative elements of the ceiling moulding. In this case the proposals will also result in considerable heritage benefits by virtue of the proposed investment in structural repair which will help to secure the future stability of the building and allow the continued use of the Dining room for National Trust visitors.

RECOMMENDATION:

That Listed Building Consent be GRANTED, subject to the standard 3yr consent and plans list conditions:

WA5 	LISTED BUILDING CONSENT -COMMENCEMENT 3 YEARS

The works for which Listed Building Consent is hereby granted shall be begun before the expiration of three years from the date of this consent.

REASON:  To comply with the provisions of Section 18 of the Planning (Listed Buildings and Conservation Areas) Act 1990 as amended by the Planning and Compulsory Purchase Act 2004.

Plans List conditions.